Study Report

Predicted Environmental Concentrations in Surface Water 
of Cyanamide and PERLKA after fertilization with PERLKA 
based on FOCUS STEP3

Simulations maize, potatoes, pome and stone fruit, and vegetables

Sponsor

AlzChem Trostberg GmbH

Dr.-Albert-Frank-Str. 32

83308 Trostberg

Germany

Institute

Fraunhofer-Institute for Molecular
	Biology and Applied Ecology (IME)

Auf dem Aberg 1

57392 Schmallenberg
	Germany

Director Applied Ecology
	Prof. Dr. Christoph Schäfers

Author
Dr. Michael Klein

May 7, 2018

This page was intentionally left blank for statements of the sponsor or submitter.

GLP-compliance

This study “Predicted Environmental Concentrations in Surface Water for cyanamide and PERLKA based on FOCUS STEP3 Simulations in maize, potatoes, pome and stone fruit, and vegetables” was conducted according to the procedures described herein. This report is a true an accurate record of the results obtained. There were no circumstances that may have adversely impacted the quality or integrity of the study.

The study does not describe an experimental study, so the GLP-regulation is not applicable. However, the study wasperformed in accordance to the “Codex of Good Modelling Practices” (Görlitz 1993 und Travis 1995).

______________________________________	__7 May 2018______

Dr. Michael Klein		Date

Exposure Modelling

Fraunhofer Institute

Auf dem Aberg 1

57392 Schmallenberg

Tel +49 2972 302 317

Fax +49 2972 302 319

michael.klein@ime.fraunhofer.de

Contents	page

GLP-compliance	3

1.	Simulation models	5

2.	Input parameters	5

2.1	Agricultural use pattern	5

2.2	Substance properties of Calcium Cyanamide (PERLKA) and Cyanamide	7

3.	Results	9

4.	References	13

5.	Appendix 1: Fitting the release and degradation of cyanamide	14

6.	Appendix 2: SWASH Report Files	15

6.1	Application in Vegetables (400 kg/ha)	15

6.2	Application in Potatoes (250 kg/ha)	17

6.3	Application in Pome/stone fruit (200 kg/ha)	19

6.4	Application in Maize (200 kg/ha)	21

# Simulation models

The program SWASH 5.3 (FOCUS 2001) was used to create the necessary input data for MACRO 5.5.4, PRZM 4.3.1 and TOXSWA 4.4 which were used for the simulations.  All models are described in FOCUS (2001). The standard buffer was 1 m.

# Input parameters

## Agricultural use pattern

PERLKA is applied in various crops with different application pattern as summarised in the following table.

Table 1: Application pattern of PERLKA in various crops considered for the simulations

| Crop | App. method | Incorp. Depth (cm) | App. Rate (kg/ha) | Start of the application window |
| --- | --- | --- | --- | --- |
| Vegetables leafy | Incorporated | at 15 cm | 400 | 14 days before emergence (= planting) |
| Maize | Incorporated | at 10 cm | 200 | 14 days before emergence (= planting) |
| Potatoes | Incorporated | at 15 cm | 250 | 14 days before emergence (= planting) |
| Pome/Stone fruit | Granular app. | 0 cm | 200 | 14 days before leaf emergence |

Based on the information given in Table 1 following application dates were automatically calculated for the different crops at the different scnearios.

Table 2: Application pattern in winter cereals considered for the simulations

| Crop | Scenario | application date calculated by FOCUS PAT |
| --- | --- | --- |
| Maize | D3 (drainage) | 20 April |
| Maize | D4 (drainage) | 26 April |
| Maize | D5 (drainage) | 26 April |
| Maize | D6 (drainage) | 9 April |
| Maize | R1 (runoff) | 26 April |
| Maize | R2 (runoff) | 22 April |
| Maize | R3 (runoff) | 22 April |
| Maize | R4 (runoff) | 7 April |
| Potatoes | D3 (drainage) | 04 May |
| Potatoes | D4 (drainage) | 17 May |
| Potatoes | D6 1st season (drainage) | 02 April |
| Potatoes | D6 2nd season (drainage) | 25 July |
| Potatoes | R1 (runoff) | 26 April |
| Potatoes | R2 (runoff) | 01 March |
| Potatoes | R3 (runoff) | 28 March |
| Pome/stone fruit | D3 (drainage) | 04 April |
| Pome/stone fruit | D4 (drainage) | 18 April |
| Pome/stone fruit | D5 (drainage) | 08 April |
| Pome/stone fruit | R1 (runoff) | 26 April |
| Pome/stone fruit | R2 (runoff) | 01 March |
| Pome/stone fruit | R3 (runoff) | 28 March |
| Pome/stone fruit | R4 (runoff) | 05 March |
| vegetables | D3 1st season (drainage) | 10 April |
| vegetables | D3 2nd season (drainage) | 25 July |
| vegetables | D4 (drainage) | 16 May |
| vegetables | D6 (drainage) | 04 August |
| vegetables | R1 1st season (runoff) | 26 April |
| vegetables | R1 2nd season (runoff) | 28 July |
| vegetables | R2 1st season (runoff) | 6 March |
| vegetables | R2 2nd season (runoff) | 08 August |
| vegetables | R3 1st season (runoff) | 19 February |
| vegetables | R3 2nd season (runoff) | 02 June |
| vegetables | R4 1st season (runoff) | 01 Mar |
| vegetables | R4 2nd season (runoff) | 01 June |

## Substance properties of Calcium Cyanamide (PERLKA) and Cyanamide

PERLKA

PERLKA consists to about 44% of calcium cyanamide. In order to simulate the slow release in soil of cyanamide from PERLKA granules adequately cyanamide was defined as metabolite. According to experimental data within 1 day up to 50% of the cyanamide is released out of the granules at 12 °C and 10% soil moisture (see appendix 1). Consequently, a half life of 1 day under standard conditions was assumed for the formation of cyanamide. The same release (half life of 1 day) was considered for the water phase. This value was used as a minimum number for the simulation model FOCUS TOXSWA.

Cyanamide

According to experimental data cyanamide is further transformed to urea (DegT50 of cyanamide 2.89 days at 12 °C) at 5% and 10% soil moisture.

Furthermore, according to EFSA (2010) the half life of cyanamide in the water-sediment system is 3.5 days (geometric mean of two studies). According to the FOCUS recommendations, this value of 3.5 days must be considered for the transformation of cyanamide in water whereas a default value of 1000 days was considered for the sediment phase. The experimental disappearance time cannot be used for the modelling since it includes not only degradation but also transportation which is considered in the model separately.

All DegT50 values in water, sediment-system and in soil considered in the simulations are presented in Table 3.

Table 3: DegT50-values (d) of PERLKA and cyanamide

| Parameter | PERLKA | Cyanamide |
| --- | --- | --- |
| Water | 1 (at 12 °C) | 2.89 (at 12 °C) |
| Sediment | 1 (at 12 °C) | 1000 (at 20 °C) |
| Soil | 1 (at 12 °C) | 1.4 (at 20 °C) |

PERLKA granules cannot be dissolved in water without being transformed to cyanamide. In order to simulate the fate of PERLKA realistically the sorption constant in soil KOC was set to an artificial, high number (172400 L/kg). That should guarantee that the granules remain at the applied location in soil and are only transformed to cyanamide without leaching to deeper soil layers. For PERLKA residues in surface water the same value was considered as for cyanamide.

For cyanamide an average (geomean) sorption constant of 4 L/kg was considered which was based on experimental sorption studies. It was considered more reliable than results based on the HPLC methodology.

Cyanamide has a Henry’s law constant of 2.68 10-5 J/mol. However this value cannot be entered directly into the simulation model but will be internally calculated based on water solubility, vapour pressure and molecular mass.

Plant uptake was not considered since the granules are usually applied before emergence of the crop.

All other input parameters used for the simulations are summarised in Table 4.

Table 4: Other input parameters used for the simulations of PERLKA and Cyanamide

| Parameter | PERLKA | Cyanamide | remark |
| --- | --- | --- | --- |
| Sorption constant KOC in soil (L/kg) | 172400 | 4 | EFSA (2010) |
| Sorption constant KOC in water body (L/kg) | 0 | 4 | EFSA (2010) |
| Freundlich exponent (-) | 1 | 1 | EFSA (2010) |
| vapour pressure (20°C, Pa) | 0 | 0.51 Pa |  |
| Molar mass (g/mol) | 80.11 | 42.04* |  |
| Henry’s law constant (J/mol) | - | 2.68 10-5 | EFSA (2010) |
| Water solubility (20°C, mg/L) | 800000 | 800000 | EFSA (2010) |
| molar enthalpy of vaporisation | 95000 | 95000 | default |
| molar enthalpy of dissolution | 27000 | 27000 | default |
| diffusion coefficient in water | 4.3 10-5 | 4.3 10-5 | default |
| diffusion coefficient in air | 0.43 | 0.43 | default |
| Plant uptake factor | 0 | 0 | default |

* hydrogen cyanamide

# Results

The global maximum concentrations for all scenarios and crops are summarised in the following Table 5. In total 58 FOCUS crop-location combinations were simulated.

The following maximum concentrations were calculated for cyanamide in the simulations:

Maize: 200 kg/ha, Underground fertilisation at 10 cm: < 0.1 µg/L

Potatoes: 250 kg/ha, Underground fertilisation at 15 cm: < 0.1 µg/L

Pome/stone fruits: 200 kg/ha, granular application: 2.639 µg/L

Vegetables: 400 kg/ha Underground fertilisation at 15 cm: <0.1 µg/L

Table 5: Global maximum concentrations or PERLKA and Cyanamide at FOCUS Step 3

| Crop | Scenario | PERLKA | Cyanamide |  |
| --- | --- | --- | --- | --- |
|  |  | PECsw (µg/L) | PECsw (µg/L) | PECsed (µg/kg) |
| Maize | D3_Ditch | 0.000 | 0.001 | 0.001 |
|  | D4_Pond | 0.000 | 0.004 | 0.002 |
|  | D4_Stream | 0.000 | 0.012 | 0.006 |
|  | D5_Pond | 0.000 | 0.000 | 0.000 |
|  | D5_Stream | 0.000 | 0.000 | 0.000 |
|  | D6_Ditch | 0.000 | 0.049 | 0.021 |
|  | R1_Pond | 0.000 | 0.000 | 0.000 |
|  | R1_Stream | 0.000 | 0.000 | 0.000 |
|  | R2_Stream | 0.000 | 0.000 | 0.000 |
|  | R3_Stream | 0.000 | 0.000 | 0.000 |
|  | R4_Stream | 0.000 | 0.000 | 0.000 |
| Potatoes | D3_Ditch | 0.000 | 0.000 | 0.000 |
|  | D4_Pond | 0.000 | 0.000 | 0.000 |
|  | D4_Stream | 0.000 | 0.002 | 0.001 |
|  | D6_Ditch | 0.000 | 0.065 | 0.003 |
|  | D6_Ditch | 0.000 | 0.012 | 0.001 |
|  | R1_Pond | 0.000 | 0.000 | 0.000 |
|  | R1_Stream | 0.000 | 0.000 | 0.000 |
|  | R2_Stream | 0.000 | 0.000 | 0.000 |
|  | R3_Stream | 0.000 | 0.000 | 0.000 |
| Pome/stone fruit | D3_Ditch | 0.000 | 0.000 | 0.000 |
|  | D4_Pond | 0.000 | 0.001 | 0.001 |
|  | D4_Stream | 0.000 | 0.004 | 0.002 |
|  | D5_Pond | 0.000 | 0.000 | 0.000 |
|  | D5_Stream | 0.000 | 0.000 | 0.000 |
|  | R1_Pond | 0.000 | 0.001 | 0.000 |
|  | R1_Stream | 0.000 | 0.293 | 0.005 |
|  | R2_Stream | 0.000 | 2.052 | 0.039 |
|  | R3_Stream | 0.000 | 2.639 | 0.091 |
|  | R4_Stream | 0.000 | 0.000 | 0.000 |
| Leafy vegetables (400 kg/ha underground fertilisation at 15 cm) | D3_Ditch | 0.000 | 0.000 | 0.000 |
|  | D3_Ditch | 0.000 | 0.003 | 0.002 |
|  | D4_Pond | 0.000 | 0.001 | 0.000 |
|  | D4_Stream | 0.000 | 0.002 | 0.001 |
|  | D6_Ditch | 0.000 | 0.022 | 0.002 |
|  | R1_Pond | 0.000 | 0.000 | 0.000 |
|  | R1_Pond | 0.000 | 0.000 | 0.000 |
|  | R1_Stream | 0.000 | 0.000 | 0.000 |
|  | R1_Stream | 0.000 | 0.000 | 0.000 |
|  | R2_Stream | 0.000 | 0.000 | 0.000 |
|  | R2_Stream | 0.000 | 0.000 | 0.000 |
|  | R3_Stream | 0.000 | 0.000 | 0.000 |
|  | R3_Stream | 0.000 | 0.000 | 0.000 |
|  | R4_Stream | 0.000 | 0.000 | 0.000 |
|  | R4_Stream | 0.000 | 0.000 | 0.000 |

Conclusions

The following maximum concentrations were calculated for cyanamide at step 3 simulations:

Maize: 200 kg/ha, Underground fertilisation at 10 cm: < 0.1 µg/L

Potatoes: 250 kg/ha, Underground fertilisation at 15 cm: < 0.1 µg/L

Pome/stone fruits: 200 kg/ha, granular application: 2.639 µg/L

Vegetables: 400 kg/ha Underground fertilisation at 15 cm: <0.1 µg/L

# References

- FOCUS (2000): “FOCUS groundwater scenarios in the EU review of active substances”. Report of the FOCUS Groundwater Scenarios Workgroup, EC Doc. Ref. SANCO/321/2000 rev. 2; and “Generic guidance for FOCUS groundwater scenarios”. FOCUS Groundwater Scenario Workgroup, May 2001, Version

- FOCUS (2001). “FOCUS Surface Water Scenarios in the EU Evaluation Process under 91/414/EEC”. Report of the FOCUS Working Group on Surface Water Scenarios, EC

- Görlitz. G. (1993): Verfahrensregeln zur korrekten Durchführung und Auswertung von Modellrechnungen zur Simulation des Umweltverhaltens von Pflanzenschutzmitteln.

- Travis. K.Z. (1995): Recommendations for the correct use of models and reporting of modelling results.- in: ‘Leaching Models and EU registration’. Final report of the FOCUS Group. Doc. 4952/VI/95.

- FOCUS (2007). “Landscape And Mitigation Factors In Aquatic Risk Assessment. Volume 1. Extended Summary and Recommendations”. Report of the FOCUS Working Group on Landscape and MitigationFactors in Ecological Risk Assessment, EC Document Reference SANCO/10422/2005 v2.0. 169 pp.

# Appendix 1: Fitting the release and degradation of cyanamide

# Appendix 2: SWASH Report Files

## Application in Vegetables (400 kg/ha)

* SWASH report file

* made by FOCUS-SWASH UI v. 5 (internal version 5.1.0, 02 April 2015)

*

* File Name   : E:\SwashProjects\CN2_VEG3\CN2_VEG3_report.txt

* Description : 400 kg 15 cm uniform

* Substance   : CN2Gr

*

* Creation    : 03-May-2018, 15:32

*

* Remarks : SWASH report helps you to set up the needed runs to calculate the PECsw and PECsed, occuring in the EU

*           for the selected substance, used on the selected crop. The scenario code informs you which models you need to

*           run for this scenario.

*           D1-D6: drainage entries calculated by the MACRO model, fate in surface water calculated by the TOXSWA model

*

*           R1-R4: runoff and erosion entries calculated by the PRZM model, fate in surface water calculated by the TOXSWA model

*

*

*           For STREAMS the Mean Deposition and Mass Loading, as calculated by the FOCUS Drift Calculator, have been multiplied by a

*           factor 1.2 to account for pesticide mass incoming from the upstream catchment as decided by the FOCUS Surface Water

*           Scenarios Working Group.

*

**********************************************************************************************************************************

*  CREATED RUNS

**********************************************************************************************************************************

*                                                     |--------------- APPLICATION -----------------|----- on Water Surface -----|

* -ID------Crop(1st/2nd)-------Scenario-WaterbodyType-|-Method--------First/Last/Interval--#---Rate-|-Mean Deposition-Mass Loading

(d)          (kg/ha) (% of Appl. Rate)    (mg/m2)

*

* 1048  Vegetables, leafy(1st)   D3_Ditch              soil incorp.   11-Apr/11-May/1      1  400.0000     0.000           0.000

*

* 1049  Vegetables, leafy(2nd)   D3_Ditch              soil incorp.   22-Jul/21-Aug/1      1  400.0000     0.000           0.000

*

* 1050  Vegetables, leafy(1st)   D4_Pond               soil incorp.   26-Apr/26-May/1      1  400.0000     0.000           0.000

*

* 1051  Vegetables, leafy(1st)   D4_Stream             soil incorp.   26-Apr/26-May/1      1  400.0000     0.000           0.000

*

* 1052  Vegetables, leafy(1st)   D6_Ditch              soil incorp.   1-Aug /31-Aug/1      1  400.0000     0.000           0.000

*

* 1053  Vegetables, leafy(1st)   R1_Pond               soil incorp.   6-Apr /6-May /1      1  400.0000     0.000           0.000

*

* 1054  Vegetables, leafy(2nd)   R1_Pond               soil incorp.   17-Jul/16-Aug/1      1  400.0000     0.000           0.000

*

* 1055  Vegetables, leafy(1st)   R1_Stream             soil incorp.   6-Apr /6-May /1      1  400.0000     0.000           0.000

*

* 1056  Vegetables, leafy(2nd)   R1_Stream             soil incorp.   17-Jul/16-Aug/1      1  400.0000     0.000           0.000

*

* 1057  Vegetables, leafy(1st)   R2_Stream             soil incorp.   14-Feb/16-Mar/1      1  400.0000     0.000           0.000

*

* 1058  Vegetables, leafy(2nd)   R2_Stream             soil incorp.   17-Jul/16-Aug/1      1  400.0000     0.000           0.000

*

* 1059  Vegetables, leafy(1st)   R3_Stream             soil incorp.   15-Feb/17-Mar/1      1  400.0000     0.000           0.000

*

* 1060  Vegetables, leafy(2nd)   R3_Stream             soil incorp.   1-Jun /1-Jul /1      1  400.0000     0.000           0.000

*

* 1061  Vegetables, leafy(1st)   R4_Stream             soil incorp.   15-Feb/17-Mar/1      1  400.0000     0.000           0.000

*

* 1062  Vegetables, leafy(2nd)   R4_Stream             soil incorp.   1-Jun /1-Jul /1      1  400.0000     0.000           0.000

*

************************************************** Surface WAter Scenarios Help **************************************************

## Application in Potatoes (250 kg/ha)

* SWASH report file

* made by FOCUS-SWASH UI v. 5 (internal version 5.1.0, 02 April 2015)

*

* File Name   : E:\SwashProjects\CN2_PO1\CN2_PO1_report.txt

* Description : 250 kg at 15 cm

* Substance   : CN2Gr

*

* Creation    : 25-Apr-2018, 10:44

*

* Remarks : SWASH report helps you to set up the needed runs to calculate the PECsw and PECsed, occuring in the EU

*           for the selected substance, used on the selected crop. The scenario code informs you which models you need to

*           run for this scenario.

*           D1-D6: drainage entries calculated by the MACRO model, fate in surface water calculated by the TOXSWA model

*

*           R1-R4: runoff and erosion entries calculated by the PRZM model, fate in surface water calculated by the TOXSWA model

*

*

*           For STREAMS the Mean Deposition and Mass Loading, as calculated by the FOCUS Drift Calculator, have been multiplied by a

*           factor 1.2 to account for pesticide mass incoming from the upstream catchment as decided by the FOCUS Surface Water

*           Scenarios Working Group.

*

*

**********************************************************************************************************************************

*  CREATED RUNS

**********************************************************************************************************************************

*

*                                                     |--------------- APPLICATION -----------------|----- on Water Surface -----|

* -ID------Crop(1st/2nd)-------Scenario-WaterbodyType-|-Method--------First/Last/Interval--#---Rate-|-Mean Deposition-Mass Loading

(d)          (kg/ha) (% of Appl. Rate)    (mg/m2)

*

* 960   Potatoes(1st)            D3_Ditch              soil incorp.   26-Apr/26-May/1      1  250.0000     0.000           0.000

*

* 961   Potatoes(1st)            D4_Pond               soil incorp.   8-May /7-Jun /1      1  250.0000     0.000           0.000

*

* 962   Potatoes(1st)            D4_Stream             soil incorp.   8-May /7-Jun /1      1  250.0000     0.000           0.000

*

* 963   Potatoes(1st)            D6_Ditch              soil incorp.   27-Mar/26-Apr/1      1  250.0000     0.000           0.000

*

* 964   Potatoes(2nd)            D6_Ditch              soil incorp.   22-Jul/21-Aug/1      1  250.0000     0.000           0.000

*

* 965   Potatoes(1st)            R1_Pond               soil incorp.   21-Apr/21-May/1      1  250.0000     0.000           0.000

*

* 966   Potatoes(1st)            R1_Stream             soil incorp.   21-Apr/21-May/1      1  250.0000     0.000           0.000

*

* 967   Potatoes(1st)            R2_Stream             soil incorp.   1-Mar /31-Mar/1      1  250.0000     0.000           0.000

*

* 968   Potatoes(1st)            R3_Stream             soil incorp.   27-Mar/26-Apr/1      1  250.0000     0.000           0.000

*

************************************************** Surface WAter Scenarios Help **************************************************

## Application in Pome/stone fruit (200 kg/ha)

* SWASH report file

* made by FOCUS-SWASH UI v. 5 (internal version 5.1.0, 02 April 2015)

*

* File Name   : E:\SwashProjects\CN2_POM1\CN2_POM1_report.txt

* Description : 200 kg at surface

* Substance   : CN2Gr

*

* Creation    : 25-Apr-2018, 10:53

*

* Remarks : SWASH report helps you to set up the needed runs to calculate the PECsw and PECsed, occuring in the EU

*           for the selected substance, used on the selected crop. The scenario code informs you which models you need to

*           run for this scenario.

*           D1-D6: drainage entries calculated by the MACRO model, fate in surface water calculated by the TOXSWA model

*

*           R1-R4: runoff and erosion entries calculated by the PRZM model, fate in surface water calculated by the TOXSWA model

*

*

*           For STREAMS the Mean Deposition and Mass Loading, as calculated by the FOCUS Drift Calculator, have been multiplied by a

*           factor 1.2 to account for pesticide mass incoming from the upstream catchment as decided by the FOCUS Surface Water

*           Scenarios Working Group.

*

*

**********************************************************************************************************************************

*  CREATED RUNS

**********************************************************************************************************************************

*

*                                                     |--------------- APPLICATION -----------------|----- on Water Surface -----|

* -ID------Crop(1st/2nd)-------Scenario-WaterbodyType-|-Method--------First/Last/Interval--#---Rate-|-Mean Deposition-Mass Loading

(d)          (kg/ha) (% of Appl. Rate)    (mg/m2)

*

* 978   Pome/stone fruit, ea(1st) D3_Ditch              granular appl. 1-Apr /1-May /1      1  200.0000     0.000           0.000

*

* 979   Pome/stone fruit, ea(1st) D4_Pond               granular appl. 6-Apr /6-May /1      1  200.0000     0.000           0.000

*

* 980   Pome/stone fruit, ea(1st) D4_Stream             granular appl. 6-Apr /6-May /1      1  200.0000     0.000           0.000

*

* 981   Pome/stone fruit, ea(1st) D5_Pond               granular appl. 18-Mar/17-Apr/1      1  200.0000     0.000           0.000

*

* 982   Pome/stone fruit, ea(1st) D5_Stream             granular appl. 18-Mar/17-Apr/1      1  200.0000     0.000           0.000

*

* 983   Pome/stone fruit, ea(1st) R1_Pond               granular appl. 1-Apr /1-May /1      1  200.0000     0.000           0.000

*

* 984   Pome/stone fruit, ea(1st) R1_Stream             granular appl. 1-Apr /1-May /1      1  200.0000     0.000           0.000

*

* 985   Pome/stone fruit, ea(1st) R2_Stream             granular appl. 1-Mar /31-Mar/1      1  200.0000     0.000           0.000

*

* 986   Pome/stone fruit, ea(1st) R3_Stream             granular appl. 18-Mar/17-Apr/1      1  200.0000     0.000           0.000

*

* 987   Pome/stone fruit, ea(1st) R4_Stream             granular appl. 1-Mar /31-Mar/1      1  200.0000     0.000           0.000

*

************************************************** Surface WAter Scenarios Help **************************************************

## Application in Maize (200 kg/ha)

* SWASH report file

* made by FOCUS-SWASH UI v. 5 (internal version 5.1.0, 02 April 2015)

*

* File Name   : E:\SwashProjects\CN2_MZ1\CN2_MZ1_report.txt

* Description : 200 kg at 10 cm

* Substance   : CN2Gr

*

* Creation    : 03-May-2018, 15:28

*

* Remarks : SWASH report helps you to set up the needed runs to calculate the PECsw and PECsed, occuring in the EU

*           for the selected substance, used on the selected crop. The scenario code informs you which models you need to

*           run for this scenario.

*           D1-D6: drainage entries calculated by the MACRO model, fate in surface water calculated by the TOXSWA model

*

*           R1-R4: runoff and erosion entries calculated by the PRZM model, fate in surface water calculated by the TOXSWA model

*

*

*           For STREAMS the Mean Deposition and Mass Loading, as calculated by the FOCUS Drift Calculator, have been multiplied by a

*           factor 1.2 to account for pesticide mass incoming from the upstream catchment as decided by the FOCUS Surface Water

*           Scenarios Working Group.

*

*

**********************************************************************************************************************************

*  CREATED RUNS

**********************************************************************************************************************************

*

*                                                     |--------------- APPLICATION -----------------|----- on Water Surface -----|

* -ID------Crop(1st/2nd)-------Scenario-WaterbodyType-|-Method--------First/Last/Interval--#---Rate-|-Mean Deposition-Mass Loading

(d)          (kg/ha) (% of Appl. Rate)    (mg/m2)

*

* 1037  Maize(1st)               D3_Ditch              soil incorp.   21-Apr/21-May/1      1  200.0000     0.000           0.000

*

* 1038  Maize(1st)               D4_Pond               soil incorp.   26-Apr/26-May/1      1  200.0000     0.000           0.000

*

* 1039  Maize(1st)               D4_Stream             soil incorp.   26-Apr/26-May/1      1  200.0000     0.000           0.000

*

* 1040  Maize(1st)               D5_Pond               soil incorp.   26-Apr/26-May/1      1  200.0000     0.000           0.000

*

* 1041  Maize(1st)               D5_Stream             soil incorp.   26-Apr/26-May/1      1  200.0000     0.000           0.000

*

* 1042  Maize(1st)               D6_Ditch              soil incorp.   6-Apr /6-May /1      1  200.0000     0.000           0.000

*

* 1043  Maize(1st)               R1_Pond               soil incorp.   19-Apr/19-May/1      1  200.0000     0.000           0.000

*

* 1044  Maize(1st)               R1_Stream             soil incorp.   19-Apr/19-May/1      1  200.0000     0.000           0.000

*

* 1045  Maize(1st)               R2_Stream             soil incorp.   17-Apr/17-May/1      1  200.0000     0.000           0.000

*

* 1046  Maize(1st)               R3_Stream             soil incorp.   17-Apr/17-May/1      1  200.0000     0.000           0.000

*

* 1047  Maize(1st)               R4_Stream             soil incorp.   27-Mar/26-Apr/1      1  200.0000     0.000           0.000

*

************************************************** Surface WAter Scenarios Help **************************************************