Study Report

Predicted Environmental Concentrations in Surface Water 
for cyanamide (product PERLKA) 
based on FOCUS STEP3

Simulations in oilseed rape, potatoes, pome and stone fruit, and vegetables

Sponsor

AlzChem Trostberg GmbH

Dr.-Albert-Frank-Str. 32

83308 Trostberg

Germany

Institute

Fraunhofer-Institute for Molecular
	Biology and Applied Ecology (IME)

Auf dem Aberg 1

57392 Schmallenberg
	Germany

Director Applied Ecology
	Prof. Dr. Christoph Schäfers

Author
Dr. Michael Klein

April  27, 2018

This page was intentionally left blank for statements of the sponsor or submitter.

GLP-compliance

This study “Predicted Environmental Concentrations in Surface Water for cyanamide (product PERLKA) based on FOCUS STEP3 Simulations in oilseed rape, potatoes, pome and stone fruit, and vegetables” was conducted according to the procedures described herein. This report is a true an accurate record of the results obtained. There were no circumstances that may have adversely impacted the quality or integrity of the study.

The study does not describe an experimental study, so the GLP-regulation is not applicable. However, the study wasperformed in accordance to the “Codex of Good Modelling Practices” (Görlitz 1993 und Travis 1995).

______________________________________	__27 April 2018______

Dr. Michael Klein		Date

Exposure Modelling

Fraunhofer Institute

Auf dem Aberg 1

57392 Schmallenberg

Tel +49 2972 302 317

Fax +49 2972 302 319

michael.klein@ime.fraunhofer.de

Contents	page

GLP-compliance	3

1.	Simulation models	5

2.	Input parameters	5

2.1	Agricultural use pattern	5

2.2	Substance properties of Cynamide (PERLKA)	8

2.3	Refinement (Step 4)	9

3.	Results	10

4.	Conclusions	17

5.	References	17

6.	Appendix: Summary Output Files	18

6.1	Application in Hops	18

6.2	Application in Maizes	19

6.3	Application in Oilseed rape	21

6.4	Application in Potatoes	23

6.5	Application in Pome/stone fruit	25

6.6	Application in Sugar beet	27

6.7	Application in vegetables (leafy)	29

6.8	Application in vines	31

# Simulation models

The program SWASH 5.3 (FOCUS 2001) was used to create the necessary input data for MACRO 5.5.4, PRZM 4.3.1 and TOXSWA 4.4 which were used for the simulations.  All models are described in FOCUS (2001). The standard buffer was 1 m.

# Input parameters

## Agricultural use pattern

PERLKA is applied in various crops with different application pattern as summarised in the following table.

Table 1: Application pattern PERLKA in various crops considered for the simulations

| Crop | App. method | Incorp. Depth (cm) | App. Rate (kg/ha) | Start of the application window |
| --- | --- | --- | --- | --- |
| Vegetables leafy | incorporated# | uniformly over 7.5 | 500 | 14 days before emergence |
| Vegetables leary | incorporated# | at 10 cm | 500 | 14 days before emergence (= planting) |
| Potatoes | Incorporated# | at 15 cm | 250 | 14 days before emergence (= planting) |
| Oilseed rape (spring) | Granular appl. | 0 | 300 | 30 days after emergence (BBCH 29/30) |
| Pome/Stone fruit | Granular app. | 0 | 200 | 14 days before leaf emergence |

Based on the information given in Table 1 following application dates were automatically calculated for the different crops at the different scnearios.

Table 2: Application pattern in winter cereals considered for the simulations

| Crop | Scenario | application date calculated by FOCUS PAT |
| --- | --- | --- |
| Oilseed rape | D1 (drainage) | 24 Jun |
| Oilseed rape | D3 (drainage) | 22 May |
| Oilseed rape | D4 (drainage) | 01 June |
| Oilseed rape | D5 (drainage) | 22 April |
| Oilseed rape | R1 (runoff) | 10 May |
| Potatoes | D3 (drainage) | 04 May |
| Potatoes | D4 (drainage) | 17 May |
| Potatoes | D6 1st season (drainage) | 02 April |
| Potatoes | D6 2nd season (drainage) | 25 July |
| Potatoes | R1 (runoff) | 26 April |
| Potatoes | R2 (runoff) | 01 March |
| Potatoes | R3 (runoff) | 28 March |
| Pome/stone fruit | D3 (drainage) | 04 April |
| Pome/stone fruit | D4 (drainage) | 18 April |
| Pome/stone fruit | D5 (drainage) | 08 April |
| Pome/stone fruit | R1 (runoff) | 26 April |
| Pome/stone fruit | R2 (runoff) | 01 March |
| Pome/stone fruit | R3 (runoff) | 28 March |
| Pome/stone fruit | R4 (runoff) | 05 March |
| vegetables | D3 1st season (drainage) | 10 April |
| vegetables | D3 2nd season (drainage) | 25 July |
| vegetables | D4 (drainage) | 16 May |
| vegetables | D6 (drainage) | 04 August |
| vegetables | R1 1st season (runoff) | 26 April |
| vegetables | R1 2nd season (runoff) | 28 July |
| vegetables | R2 1st season (runoff) | 6 March |
| vegetables | R2 2nd season (runoff) | 08 August |
| vegetables | R3 1st season (runoff) | 19 February |
| vegetables | R3 2nd season (runoff) | 02 June |
| vegetables | R4 1st season (runoff) | 01 Mar |
| vegetables | R4 2nd season (runoff) | 01 June |

## Substance properties of Calcium Cyanamide (PERLKA) and Cyanamide

PERLKA

PERLKA consists to about 45% of calcium cyanamide. In order to simulate the slow release in soil of cyanamide from PERLKA granules adequately cyanamide was defined as metabolite. According to experimental data at 12 °C within 1 day up to 50% of the cyanamide is released out of the granules. Consequtently, a half life of 1 day under standard conditions was assumed for the formation of cyanamide. The same release was considered (half life of 1 day) was considered for the water phase. This value was used as a minimum number for the simulation model FOCUS TOXSWA.

Cyanamide

According to experimental data cyanamide is further transformed to urea (DegT50 of cyanamide 2.89 days at 12 °C).

Furthermore, according to EFSA (2010) the half life of cyanamide in the water-sediment system is 3.5 days (geometric mean of two studies) . According to the FOCUS recommendations, this value of 3.5 days must be considered for the transformation of cyanamide in water whereas a default value of 1000 days was considered for the sediment phase. The experimental disappearance time cannot be used for the modelling since it includes not only degradation but also transportation which is considered in the model separately.

All DegT50 values in water, sediment-system and in soil considered in the simulations are presented in Table 3.

Table 3: DegT50-values (d) of PERLKA and cyanamide

| Parameter | PERLKA | Cyanamide |
| --- | --- | --- |
| Water | 1 (at 12 °C) | 2.89 (at 12 °C) |
| Sediment | 1 (at 12 °C) | 1000 (at 20 °C) |
| Soil | 1 (at 12 °C) | 1.4 (at 20 °C) |

PERLKA granules cannot be dissolved in water without being transformed to cyanamide . In order to simulate the fate of PERLKA realistically the sorption constant in soil KOC was set to a artificial, high number (172400 L/kg). That should guarantee that the granules remain at the applied location in soil and are only transformed to cyanamide without leaching to deeper soil layers. For PERLKA residues in surface water the same value was considered as for cynamaide.

For cyanamide an average (geomean) sorption constant of 4 L/kg was considered which was based on experimental sorption studies. It was considered more reliable than results based on the HPLC methodology.

Cyanamide has a Henry’s law constant of 2.68 10-5 J/mol. However this value cannot be entered directly into the simulation model but will be internally calculated based on water solubility, vapour pressure and molecular mass.

Plant uptake was not considered since the granules are usually applied before emergence of the crop.

All other input parameters used for the simulations are summarised in Table 4.

Table 4: Other input parameters used for the simulations of PERLKA and Cynamide

| Parameter | PERLKA | Cyanamide | remark |
| --- | --- | --- | --- |
| Sorption constant KOC in soil (L/kg) | 172400 | 4 | EFSA (2010) |
| Sorption constant KOC in water body (L/kg) | 0 | 4 | EFSA (2010) |
| Freundlich exponent (-) | 1 | 1 | EFSA (2010) |
| vapour pressure (20°C, Pa) | 0 | 0.51 Pa |  |
| Molar mass (g/mol) | 80.11 | 42.04* |  |
| Henry’s law constant (J/mol) | - | 2.68 10-5 | EFSA (2010) |
| Water solubility (20°C, mg/L) | 800000 | 800000 | EFSA (2010) |
| molar enthalpy of vaporisation | 95000 | 95000 | default |
| molar enthalpy of dissolution | 27000 | 27000 | default |
| diffusion coefficient in water | 4.3 10-5 | 4.3 10-5 | default |
| diffusion coefficient in air | 0.43 | 0.43 | default |
| Plant uptake factor | 0 | 0 | default |

* hydrogen cyanamide

# Results

The global maximum concentrations for all scenarios and crops are summarised in the following Table 5 and Table 8 for step 3 (standard) and step 4 (including mitigation: no transport of PERLKA in to the surface water as it is not applied in slopy fields), respectively.

In total 69 FOCUS crop-location combinations were simulated.

At simulation level FOCUS step 3 the maximum concentrations for cyanamide in surface water were found to be 1735 µg/L (vines at D6, Thebes, Greece), 1603 µg/L (maize at R4, Roujan, France) and 1503 µg/L (pome/stone fruit at R3, Bologna, Italy). All other FOCUS crop-location combinations remained below 1000 µg/L. In total 21 of the 69 combinations resulted in concentrations between 100 µg/L 1000 µ/L and 18 of 69 combinations led to concentrations between 1 µg/L and 100 µg/L. For the remaining combinations (21 of 69) maximum concentrations in surface were were below 1 µg/L.

At simulation level FOCUS step 4 an additional mitigation option was simulated: no PERLKA reaches surface water. However, the respective cyanamide concentrations did not change very much, because already at simulation level FOCUS step 3 only low amounts of PERLKA reached surface water:

The maximum concentrations for cyanamide were still simulated to be 1735 µg/L (vines at D6, Thebes, Greece), 1597 µg/L (maize at R4, Roujan, France) and 1501 µg/L (pome/stone fruit at R3, Bologna, Italy). All other FOCUS crop-location combinations remained below 1000 µg/L. In total 21 of the 69 combinations resulted in concentrations between 100 µg/L 1000 µ/L and 18 of 69 combinations led to concentrations between 1 µg/L and 100 µg/L. For the remaining combinations (n=21) maximum concentrations in surface were were below 1 µg/L.

Overall, the FOCUS step 4 simulations only changed the results slightly as the mitigation did not include any buffer zones.

Table 5: Global maximum concentrations or PERLKA and Cynamide at FOCUS Step 3

| Crop | Scenario | PERLKA | Cyanamide |  |
| --- | --- | --- | --- | --- |
|  | Scenario | PECsw (µg/L) | PECsw (µg/L) | PECsed (µg/kg) |
| Oilseed rape | D1_Ditch | 0.000 | 487.200 | 160.100 |
|  | D1_Stream | 0.000 | 342.000 | 93.070 |
|  | D3_Ditch | 0.000 | 0.866 | 0.579 |
|  | D4_Pond | 0.000 | 15.980 | 5.859 |
|  | D4_Stream | 0.000 | 33.190 | 5.865 |
|  | D5_Pond | 0.000 | 0.031 | 0.010 |
|  | D5_Stream | 0.000 | 0.040 | 0.008 |
|  | R1_Pond | 0.019 | 0.172 | 0.138 |
|  | R1_Stream | 0.068 | 5.096 | 2.137 |
| Potatoes | D3_Ditch | 0.000 | 0.007 | 0.004 |
|  | D4_Pond | 0.000 | 0.006 | 0.004 |
|  | D4_Stream | 0.000 | 0.030 | 0.013 |
|  | D6_Ditch | 0.000 | 12.270 | 0.839 |
|  | D6_Ditch | 0.000 | 3.093 | 0.237 |
|  | R1_Pond | 0.184 | 9.878 | 4.011 |
|  | R1_Stream | 2.340 | 272.500 | 47.750 |
|  | R2_Stream | 18.310 | 242.600 | 599.200 |
|  | R3_Stream | 9.291 | 1503.600 | 78.630 |
| Pome/stone fruit | D3_Ditch | 0.000 | 0.000 | 0.000 |
|  | D4_Pond | 0.000 | 0.003 | 0.001 |
|  | D4_Stream | 0.000 | 0.005 | 0.002 |
|  | D5_Pond | 0.000 | 0.000 | 0.000 |
|  | D5_Stream | 0.000 | 0.000 | 0.000 |
|  | R1_Pond | 0.032 | 1.634 | 0.478 |
|  | R1_Stream | 2.209 | 167.500 | 13.170 |
|  | R2_Stream | 0.707 | 63.500 | 11.490 |
|  | R3_Stream | 5.456 | 534.200 | 49.530 |
|  | R4_Stream | 0.950 | 88.910 | 8.868 |
| Leafy vegetables | D3_Ditch | 0.000 | 0.001 | 0.001 |
|  | D3_Ditch | 0.000 | 0.489 | 0.315 |
|  | D4_Pond | 0.000 | 0.009 | 0.006 |
|  | D4_Stream | 0.000 | 0.028 | 0.014 |
|  | D6_Ditch | 0.000 | 10.170 | 0.842 |
|  | R1_Pond | 0.252 | 16.190 | 5.990 |
|  | R1_Pond | 0.008 | 0.664 | 0.120 |
|  | R1_Stream | 3.341 | 307.700 | 65.860 |
|  | R1_Stream | 0.081 | 10.700 | 1.020 |
|  | R2_Stream | 1.894 | 359.000 | 23.340 |
|  | R2_Stream | 0.003 | 0.246 | 0.130 |
|  | R3_Stream | 8.721 | 369.800 | 21.300 |
|  | R3_Stream | 14.870 | 346.800 | 259.300 |
|  | R4_Stream | 13.750 | 732.900 | 150.400 |
|  | R4_Stream | 19.370 | 774.100 | 801.900 |
| Leafy vegetables | D3_Ditch | 0.000 | 0.001 | 0.001 |
|  | D3_Ditch | 0.000 | 0.489 | 0.315 |
|  | D4_Pond | 0.000 | 0.009 | 0.006 |
|  | D4_Stream | 0.000 | 0.028 | 0.014 |
|  | D6_Ditch | 0.000 | 10.170 | 0.842 |
|  | R1_Pond | 0.252 | 16.190 | 5.990 |
|  | R1_Pond | 0.008 | 0.664 | 0.120 |
|  | R1_Stream | 3.341 | 307.700 | 65.860 |
|  | R1_Stream | 0.081 | 10.700 | 1.020 |
|  | R2_Stream | 1.894 | 359.000 | 23.340 |
|  | R2_Stream | 0.003 | 0.246 | 0.130 |
|  | R3_Stream | 8.721 | 369.800 | 21.300 |
|  | R3_Stream | 14.870 | 346.800 | 259.300 |
|  | R4_Stream | 13.750 | 732.900 | 150.400 |
|  | R4_Stream | 19.370 | 774.100 | 801.900 |

Table 6: Global maximum concentrations or PERLKA considering different buffer zones (FOCUS Step 4)

| Crop | Scenario | No buffer | 5 m buffer | 10 m buffer | 20 m buffer |
| --- | --- | --- | --- | --- | --- |
| Hop | R1_Pond | 0.030 | 0.018 | 0.008 | 0.004 |
|  | R1_Stream | 2.104 | 1.296 | 0.852 | 0.430 |
| Maize | R1_Pond | 0.516 | 0.310 | 0.122 | 0.053 |
|  | R1_Stream | 6.008 | 3.900 | 2.490 | 1.288 |
|  | R2_Stream | 2.366 | 1.525 | 0.954 | 0.490 |
|  | R3_Stream | 2.118 | 1.376 | 0.911 | 0.471 |
|  | R4_Stream | 21.610 | 14.070 | 9.631 | 5.025 |
| Oil seed rape | R1_Pond | 0.019 | 0.011 | 0.004 | 0.002 |
|  | R1_Stream | 0.068 | 0.041 | 0.024 | 0.013 |
| Potato | R1_Pond | 0.184 | 0.111 | 0.036 | 0.020 |
|  | R1_Stream | 2.340 | 1.504 | 0.964 | 0.508 |
|  | R2_Stream | 18.310 | 11.010 | 0.943 | 0.931 |
|  | R3_Stream | 9.291 | 5.918 | 3.504 | 1.845 |
| Pome/stone fruit | R1_Pond | 0.032 | 0.019 | 0.009 | 0.004 |
|  | R1_Stream | 2.209 | 1.393 | 0.914 | 0.465 |
|  | R2_Stream | 0.707 | 0.454 | 0.276 | 0.141 |
|  | R3_Stream | 5.456 | 3.536 | 2.368 | 1.227 |
|  | R4_Stream | 0.950 | 0.583 | 0.388 | 0.196 |
| Sugar beet | R1_Pond | 0.196 | 0.118 | 0.051 | 0.023 |
|  | R1_Stream | 2.499 | 1.626 | 1.060 | 0.550 |
|  | R3_Stream | 10.610 | 6.687 | 3.639 | 1.883 |
| Leafy vegetables | R1_Pond | 0.252 | 0.152 | 0.065 | 0.030 |
|  | R1_Pond | 0.008 | 0.005 | 0.002 | 0.001 |
|  | R1_Stream | 3.341 | 2.174 | 1.417 | 0.735 |
|  | R1_Stream | 0.081 | 0.053 | 0.036 | 0.019 |
|  | R2_Stream | 1.894 | 1.217 | 0.796 | 0.409 |
|  | R2_Stream | 0.003 | 0.002 | 0.000 | 0.000 |
|  | R3_Stream | 8.721 | 5.624 | 3.834 | 1.989 |
|  | R3_Stream | 14.870 | 8.990 | 2.477 | 0.908 |
|  | R4_Stream | 13.750 | 8.942 | 6.127 | 3.194 |
|  | R4_Stream | 19.370 | 11.730 | 3.364 | 1.434 |
| Vine | R1_Pond | 0.035 | 0.021 | 0.010 | 0.004 |
|  | R1_Stream | 2.850 | 1.771 | 1.162 | 0.588 |
|  | R2_Stream | 0.665 | 0.427 | 0.260 | 0.133 |
|  | R3_Stream | 4.844 | 3.136 | 2.100 | 1.087 |
|  | R4_Stream | 15.360 | 9.779 | 6.602 | 3.396 |

Table 7: Global maximum concentrations or Cyanamide considering different buffer zones (FOCUS Step 4)

| Crop | Scenario | No buffer | 5 m buffer | 10 m buffer | 20 m buffer |
| --- | --- | --- | --- | --- | --- |
| Hop | R1_Pond | 1.517 | 0.911 | 0.606 | 0.303 |
|  | R1_Stream | 173.000 | 106.800 | 72.210 | 36.630 |
| Maize | R1_Pond | 25.880 | 15.610 | 10.380 | 5.199 |
|  | R1_Stream | 687.200 | 428.600 | 291.400 | 148.700 |
|  | R2_Stream | 471.900 | 304.900 | 211.500 | 110.100 |
|  | R3_Stream | 156.200 | 101.300 | 70.510 | 36.870 |
|  | R4_Stream | 1603.700 | 1046.100 | 728.900 | 382.000 |
| Oil seed rape | R1_Pond | 0.172 | 0.104 | 0.069 | 0.035 |
|  | R1_Stream | 5.096 | 3.288 | 2.284 | 1.192 |
| Potato | R1_Pond | 9.878 | 5.959 | 3.957 | 1.986 |
|  | R1_Stream | 272.500 | 170.200 | 115.800 | 59.150 |
|  | R2_Stream | 242.600 | 158.400 | 110.500 | 57.920 |
|  | R3_Stream | 1503.600 | 982.600 | 685.700 | 359.600 |
| Pome/stone fruit | R1_Pond | 1.634 | 0.980 | 0.653 | 0.326 |
|  | R1_Stream | 167.500 | 102.900 | 69.410 | 35.130 |
|  | R2_Stream | 63.500 | 40.970 | 28.400 | 14.770 |
|  | R3_Stream | 534.200 | 346.900 | 241.100 | 125.900 |
|  | R4_Stream | 88.910 | 54.630 | 36.850 | 18.660 |
| Sugar beet | R1_Pond | 11.040 | 6.661 | 4.432 | 2.221 |
|  | R1_Stream | 205.600 | 133.900 | 93.240 | 48.790 |
|  | R3_Stream | 970.900 | 632.800 | 440.700 | 230.600 |
| Leafy vegetables | R1_Pond | 16.190 | 9.769 | 6.505 | 3.260 |
|  | R1_Pond | 0.664 | 0.400 | 0.267 | 0.134 |
|  | R1_Stream | 307.700 | 200.400 | 139.500 | 73.000 |
|  | R1_Stream | 10.700 | 6.983 | 4.867 | 2.551 |
|  | R2_Stream | 359.000 | 230.800 | 159.600 | 82.850 |
|  | R2_Stream | 369.800 | 0.159 | 0.110 | 0.057 |
|  | R3_Stream | 346.800 | 238.700 | 165.300 | 86.050 |
|  | R3_Stream | 732.900 | 226.700 | 158.200 | 82.980 |
|  | R4_Stream | 774.100 | 477.200 | 332.300 | 173.800 |
|  | R4_Stream | 1735.200 | 505.700 | 352.900 | 185.100 |
| Vine | R1_Pond | 2.047 | 1.229 | 0.818 | 0.409 |
|  | R1_Stream | 252.100 | 156.900 | 106.600 | 54.320 |
|  | R2_Stream | 74.630 | 45.410 | 30.480 | 15.360 |
|  | R3_Stream | 478.400 | 310.300 | 215.600 | 112.500 |
|  | R4_Stream | 644.500 | 410.600 | 282.400 | 145.800 |

Table 8: Global maximum concentrations or PERLKA and Cynamide at FOCUS Step 4 when assuming no erosion (no buffer zones)

| Crop | Scenario | PERLKA | Cyanamide |  |
| --- | --- | --- | --- | --- |
|  | Scenario | PECsw (µg/L) | PECsw (µg/L) | PECsed (µg/kg) |
| Hop | R1_Pond | 0.000 | 1.508 | 0.3174 |
|  | R1_Stream | 0.000 | 172.4 | 10.32 |
| Maize | D3_Ditch | 0.000 | 0.010 | 0.007 |
|  | D4_Pond | 0.000 | 0.015 | 0.009 |
|  | D4_Stream | 0.000 | 0.047 | 0.024 |
|  | D5_Pond | 0.000 | 0.000 | 0.000 |
|  | D5_Stream | 0.000 | 0.000 | 0.000 |
|  | D6_Ditch | 0.000 | 1.749 | 0.759 |
|  | R1_Pond | 0.000 | 25.58 | 6.475 |
|  | R1_Stream | 0.000 | 685.5 | 39.21 |
|  | R2_Stream | 0.000 | 471.3 | 31.04 |
|  | R3_Stream | 0.000 | 155.6 | 10.52 |
|  | R4_Stream | 0.000 | 1597.3 | 122.5 |
| Oilseed rape | D1_Ditch | 0.000 | 487.200 | 160.100 |
|  | D1_Stream | 0.000 | 342.000 | 93.070 |
|  | D3_Ditch | 0.000 | 0.866 | 0.579 |
|  | D4_Pond | 0.000 | 15.980 | 5.859 |
|  | D4_Stream | 0.000 | 33.190 | 5.865 |
|  | D5_Pond | 0.000 | 0.031 | 0.010 |
|  | D5_Stream | 0.000 | 0.040 | 0.008 |
|  | R1_Pond | 0.000 | 0.1719 | 0.0589 |
|  | R1_Stream | 0.000 | 5.081 | 0.294 |
| Potatoes | D3_Ditch | 0.000 | 0.007 | 0.004 |
|  | D4_Pond | 0.000 | 0.006 | 0.004 |
|  | D4_Stream | 0.000 | 0.030 | 0.013 |
|  | D6_Ditch | 0.000 | 12.270 | 0.839 |
|  | D6_Ditch | 0.000 | 3.093 | 0.237 |
|  | R1_Pond | 0.000 | 9.771 | 2.492 |
|  | R1_Stream | 0.000 | 271.8 | 15.58 |
|  | R2_Stream | 0.000 | 242.3 | 17.32 |
|  | R3_Stream | 0.000 | 1501.2 | 70.01 |
| Pome/stone fruit | D3_Ditch | 0.000 | 0.000 | 0.000 |
|  | D4_Pond | 0.000 | 0.003 | 0.001 |
|  | D4_Stream | 0.000 | 0.005 | 0.002 |
|  | D5_Pond | 0.000 | 0.000 | 0.000 |
|  | D5_Stream | 0.000 | 0.000 | 0.000 |
|  | R1_Pond | 0.000 | 1.627 | 0.3143 |
|  | R1_Stream | 0.000 | 166.9 | 11.16 |
|  | R2_Stream | 0.000 | 63.33 | 5.446 |
|  | R3_Stream | 0.000 | 532.5 | 45.12 |
|  | R4_Stream | 0.000 | 88.59 | 6.407 |
| Sugar beet | D3_Ditch | 0.000 | 0.007 | 0.005 |
|  | D4_Pond | 0.000 | 0.007 | 0.004 |
|  | D4_Stream | 0.000 | 0.021 | 0.010 |
|  | R1_Pond | 0.196 | 10.92 | 2.715 |
|  | R1_Stream | 2.499 | 205 | 13.21 |
|  | R3_Stream | 10.610 | 968.5 | 45.19 |
| Leafy vegetables | D3_Ditch | 0.000 | 0.001 | 0.001 |
|  | D3_Ditch | 0.000 | 0.489 | 0.315 |
|  | D4_Pond | 0.000 | 0.009 | 0.006 |
|  | D4_Stream | 0.000 | 0.028 | 0.014 |
|  | D6_Ditch | 0.000 | 10.170 | 0.842 |
|  | R1_Pond | 0.000 | 16.190 | 5.990 |
|  | R1_Pond | 0.000 | 16.04 | 3.947 |
|  | R1_Stream | 0.000 | 0.6638 | 0.1037 |
|  | R1_Stream | 0.000 | 306.8 | 19.52 |
|  | R2_Stream | 0.000 | 10.68 | 0.8185 |
|  | R2_Stream | 0.000 | 358.5 | 20.99 |
|  | R3_Stream | 0.000 | 0.2454 | 0.01663 |
|  | R3_Stream | 0.000 | 367 | 20.36 |
|  | R4_Stream | 0.000 | 345.9 | 29.83 |
|  | R4_Stream | 0.000 | 772.3 | 55.86 |
| Vines | D6_Ditch | 0.000 | 1735.200 | 164.200 |
|  | R1_Pond | 0.000 | 2.034 | 0.4505 |
|  | R1_Stream | 0.000 | 251.3 | 15.18 |
|  | R2_Stream | 0.000 | 74.47 | 5.369 |
|  | R3_Stream | 0.000 | 476.9 | 40.24 |
|  | R4_Stream | 0.000 | 639.8 | 46.57 |

# Conclusions

At simulation level FOCUS step 3 the maximum concentrations for cyanamide were found to be 1735 µg/L (vines at D6, Thebes, Greece), 1603 µg/L (maize at R4, Roujan, France) and 1503 µg/L (pome/stone fruit at R3, Bologna, Italy). All other FOCUS crop-location combinations remained below 1000 µg/L. In total 21 of the 69 combinations resulted in concentrations between 100 µg/L 1000 µ/L and 18 of 69 combinations led to concentrations between 1 µg/L and 100 µg/L. For the remaining combinations (21 of 69) maximum concentrations in surface were were below 1 µg/L.

At simulation level FOCUS step 4 an additional mitigation option was simulated: no PERLKA reaches surface wate. However, the respective cyanamide concentrations did not change very much, because already at simulation level FOCUS step 3 only low amounts of PERLKA reached surface water:

The maximum concentrations for cyanamide were still simulated to be 1735 µg/L (vines at D6, Thebes, Greece), 1597 µg/L (maize at R4, Roujan, France) and 1501 µg/L (pome/stone fruit at R3, Bologna, Italy). All other FOCUS crop-location combinations remained below 1000 µg/L. In total 21 of the 69 combinations resulted in concentrations between 100 µg/L 1000 µ/L and 18 of 69 combinations led to concentrations between 1 µg/L and 100 µg/L. For the remaining combinations (n=21) maximum concentrations in surface were were below 1 µg/L.

Overall, the FOCUS step 4 simulations only changed the results slightly as the mitigation did not include any buffer zones.

# References

- FOCUS (2000): “FOCUS groundwater scenarios in the EU review of active substances”. Report of the FOCUS Groundwater Scenarios Workgroup, EC Doc. Ref. SANCO/321/2000 rev. 2; and “Generic guidance for FOCUS groundwater scenarios”. FOCUS Groundwater Scenario Workgroup, May 2001, Version

- FOCUS (2001). “FOCUS Surface Water Scenarios in the EU Evaluation Process under 91/414/EEC”. Report of the FOCUS Working Group on Surface Water Scenarios, EC

- Görlitz. G. (1993): Verfahrensregeln zur korrekten Durchführung und Auswertung von Modellrechnungen zur Simulation des Umweltverhaltens von Pflanzenschutzmitteln.

- Travis. K.Z. (1995): Recommendations for the correct use of models and reporting of modelling results.- in: ‘Leaching Models and EU registration’. Final report of the FOCUS Group. Doc. 4952/VI/95.

- FOCUS (2007). “Landscape And Mitigation Factors In Aquatic Risk Assessment. Volume 1. Extended Summary and Recommendations”. Report of the FOCUS Working Group on Landscape and MitigationFactors in Ecological Risk Assessment, EC Document Reference SANCO/10422/2005 v2.0. 169 pp.

# Appendix: SWASH Report Files

## Application in Hops

* SWASH report file

* made by FOCUS-SWASH UI v. 5 (internal version 5.1.0, 02 April 2015)

*

* File Name   : E:\SwashProjects\CN2_HO\CN2_HO_report.txt

* Description : PERLKA Hop

* Substance   : CN2Gr

*

* Creation    : 07-Mar-2018, 11:55

* Remarks : SWASH report helps you to set up the needed runs to calculate the PECsw and PECsed, occuring in the EU

*           for the selected substance, used on the selected crop. The scenario code informs you which models you need to

*           run for this scenario.

*           D1-D6: drainage entries calculated by the MACRO model, fate in surface water calculated by the TOXSWA model

*

*           R1-R4: runoff and erosion entries calculated by the PRZM model, fate in surface water calculated by the TOXSWA model

*

*

*           For STREAMS the Mean Deposition and Mass Loading, as calculated by the FOCUS Drift Calculator, have been multiplied by a

*           factor 1.2 to account for pesticide mass incoming from the upstream catchment as decided by the FOCUS Surface Water

*           Scenarios Working Group.

**********************************************************************************************************************************

*  CREATED RUNS

**********************************************************************************************************************************

*

*                                                     |--------------- APPLICATION -----------------|----- on Water Surface -----|

* -ID------Crop(1st/2nd)-------Scenario-WaterbodyType-|-Method--------First/Last/Interval--#---Rate-|-Mean Deposition-Mass Loading

(d)          (kg/ha) (% of Appl. Rate)    (mg/m2)

*

* 896   Hops(1st)                R1_Pond               granular appl. 1-Apr /1-May /1      1  300.0000     0.000           0.000

*

* 897   Hops(1st)                R1_Stream             granular appl. 1-Apr /1-May /1      1  300.0000     0.000           0.000

*

************************************************** Surface WAter Scenarios Help **************************************************

## Application in Maizes

* SWASH report file

* made by FOCUS-SWASH UI v. 5 (internal version 5.1.0, 02 April 2015)

*

* File Name   : E:\SwashProjects\CN2_MZ\CN2_MZ_report.txt

* Description : PERLKA maize

* Substance   : CN2Gr

*

* Creation    : 05-Mar-2018, 14:23

*

* Remarks : SWASH report helps you to set up the needed runs to calculate the PECsw and PECsed, occuring in the EU

*           for the selected substance, used on the selected crop. The scenario code informs you which models you need to

*           run for this scenario.

*           D1-D6: drainage entries calculated by the MACRO model, fate in surface water calculated by the TOXSWA model

*

*           R1-R4: runoff and erosion entries calculated by the PRZM model, fate in surface water calculated by the TOXSWA model

*

*

*           For STREAMS the Mean Deposition and Mass Loading, as calculated by the FOCUS Drift Calculator, have been multiplied by a

*           factor 1.2 to account for pesticide mass incoming from the upstream catchment as decided by the FOCUS Surface Water

*           Scenarios Working Group.

*

*

**********************************************************************************************************************************

*  CREATED RUNS

**********************************************************************************************************************************

*

*                                                     |--------------- APPLICATION -----------------|----- on Water Surface -----|

* -ID------Crop(1st/2nd)-------Scenario-WaterbodyType-|-Method--------First/Last/Interval--#---Rate-|-Mean Deposition-Mass Loading

(d)          (kg/ha) (% of Appl. Rate)    (mg/m2)

*

* 854   Maize(1st)               D3_Ditch              granular appl. 21-Apr/21-May/1      1  300.0000     0.000           0.000

*

* 855   Maize(1st)               D4_Pond               granular appl. 26-Apr/26-May/1      1  300.0000     0.000           0.000

*

* 856   Maize(1st)               D4_Stream             granular appl. 26-Apr/26-May/1      1  300.0000     0.000           0.000

*

* 857   Maize(1st)               D5_Pond               granular appl. 26-Apr/26-May/1      1  300.0000     0.000           0.000

*

* 858   Maize(1st)               D5_Stream             granular appl. 26-Apr/26-May/1      1  300.0000     0.000           0.000

*

* 859   Maize(1st)               D6_Ditch              granular appl. 6-Apr /6-May /1      1  300.0000     0.000           0.000

*

* 860   Maize(1st)               R1_Pond               granular appl. 19-Apr/19-May/1      1  300.0000     0.000           0.000

*

* 861   Maize(1st)               R1_Stream             granular appl. 19-Apr/19-May/1      1  300.0000     0.000           0.000

*

* 862   Maize(1st)               R2_Stream             granular appl. 17-Apr/17-May/1      1  300.0000     0.000           0.000

*

* 863   Maize(1st)               R3_Stream             granular appl. 17-Apr/17-May/1      1  300.0000     0.000           0.000

*

* 864   Maize(1st)               R4_Stream             granular appl. 27-Mar/26-Apr/1      1  300.0000     0.000           0.000

*

************************************************** Surface WAter Scenarios Help **************************************************

## Application in Oilseed rape

* SWASH report file

* made by FOCUS-SWASH UI v. 5 (internal version 5.1.0, 02 April 2015)

*

* File Name   : E:\SwashProjects\CN2_OSR\CN2_OSR_report.txt

* Description : PERLKA Oil seed rape

* Substance   : CN2Gr

*

* Creation    : 08-Mar-2018, 14:19

*

* Remarks : SWASH report helps you to set up the needed runs to calculate the PECsw and PECsed, occuring in the EU

*           for the selected substance, used on the selected crop. The scenario code informs you which models you need to

*           run for this scenario.

*           D1-D6: drainage entries calculated by the MACRO model, fate in surface water calculated by the TOXSWA model

*

*           R1-R4: runoff and erosion entries calculated by the PRZM model, fate in surface water calculated by the TOXSWA model

*

*

*           For STREAMS the Mean Deposition and Mass Loading, as calculated by the FOCUS Drift Calculator, have been multiplied by a

*           factor 1.2 to account for pesticide mass incoming from the upstream catchment as decided by the FOCUS Surface Water

*           Scenarios Working Group.

*

*

**********************************************************************************************************************************

*  CREATED RUNS

**********************************************************************************************************************************

*

*                                                     |--------------- APPLICATION -----------------|----- on Water Surface -----|

* -ID------Crop(1st/2nd)-------Scenario-WaterbodyType-|-Method--------First/Last/Interval--#---Rate-|-Mean Deposition-Mass Loading

(d)          (kg/ha) (% of Appl. Rate)    (mg/m2)

*

* 871   Oil seed rape, sprin(1st) D1_Ditch              granular appl. 9-Aug /8-Sep /1      1  300.0000     0.000           0.000

*

* 872   Oil seed rape, sprin(1st) D1_Stream             granular appl. 9-Aug /8-Sep /1      1  300.0000     0.000           0.000

*

* 873   Oil seed rape, sprin(1st) D3_Ditch              granular appl. 26-Jul/25-Aug/1      1  300.0000     0.000           0.000

*

* 874   Oil seed rape, sprin(1st) D4_Pond               granular appl. 1-Aug /31-Aug/1      1  300.0000     0.000           0.000

*

* 875   Oil seed rape, sprin(1st) D4_Stream             granular appl. 1-Aug /31-Aug/1      1  300.0000     0.000           0.000

*

* 876   Oil seed rape, sprin(1st) D5_Pond               granular appl. 30-Jun/30-Jul/1      1  300.0000     0.000           0.000

*

* 877   Oil seed rape, sprin(1st) D5_Stream             granular appl. 30-Jun/30-Jul/1      1  300.0000     0.000           0.000

*

* 878   Oil seed rape, sprin(1st) R1_Pond               granular appl. 16-Jul/15-Aug/1      1  300.0000     0.000           0.000

*

* 879   Oil seed rape, sprin(1st) R1_Stream             granular appl. 16-Jul/15-Aug/1      1  300.0000     0.000           0.000

*

************************************************** Surface WAter Scenarios Help **************************************************

## Application in Potatoes

* SWASH report file

* made by FOCUS-SWASH UI v. 5 (internal version 5.1.0, 02 April 2015)

*

* File Name   : E:\SwashProjects\CN2_PO\CN2_PO_report.txt

* Description : PERLKA potato

* Substance   : CN2Gr

*

* Creation    : 02-Mar-2018, 14:58

*

* Remarks : SWASH report helps you to set up the needed runs to calculate the PECsw and PECsed, occuring in the EU

*           for the selected substance, used on the selected crop. The scenario code informs you which models you need to

*           run for this scenario.

*           D1-D6: drainage entries calculated by the MACRO model, fate in surface water calculated by the TOXSWA model

*

*           R1-R4: runoff and erosion entries calculated by the PRZM model, fate in surface water calculated by the TOXSWA model

*

*

*           For STREAMS the Mean Deposition and Mass Loading, as calculated by the FOCUS Drift Calculator, have been multiplied by a

*           factor 1.2 to account for pesticide mass incoming from the upstream catchment as decided by the FOCUS Surface Water

*           Scenarios Working Group.

*

*

**********************************************************************************************************************************

*  CREATED RUNS

**********************************************************************************************************************************

*

*                                                     |--------------- APPLICATION -----------------|----- on Water Surface -----|

* -ID------Crop(1st/2nd)-------Scenario-WaterbodyType-|-Method--------First/Last/Interval--#---Rate-|-Mean Deposition-Mass Loading

(d)          (kg/ha) (% of Appl. Rate)    (mg/m2)

*

* 830   Potatoes(1st)            D3_Ditch              soil incorp.   26-Apr/26-May/1      1  500.0000     0.000           0.000

*

* 831   Potatoes(1st)            D4_Pond               soil incorp.   8-May /7-Jun /1      1  500.0000     0.000           0.000

*

* 832   Potatoes(1st)            D4_Stream             soil incorp.   8-May /7-Jun /1      1  500.0000     0.000           0.000

*

* 833   Potatoes(1st)            D6_Ditch              soil incorp.   27-Mar/26-Apr/1      1  500.0000     0.000           0.000

*

* 834   Potatoes(2nd)            D6_Ditch              soil incorp.   22-Jul/21-Aug/1      1  500.0000     0.000           0.000

*

* 835   Potatoes(1st)            R1_Pond               soil incorp.   21-Apr/21-May/1      1  500.0000     0.000           0.000

*

* 836   Potatoes(1st)            R1_Stream             soil incorp.   21-Apr/21-May/1      1  500.0000     0.000           0.000

*

* 837   Potatoes(1st)            R2_Stream             soil incorp.   1-Mar /31-Mar/1      1  500.0000     0.000           0.000

*

* 838   Potatoes(1st)            R3_Stream             soil incorp.   27-Mar/26-Apr/1      1  500.0000     0.000           0.000

*

************************************************** Surface WAter Scenarios Help **************************************************

## Application in Pome/stone fruit

* SWASH report file

* made by FOCUS-SWASH UI v. 5 (internal version 5.1.0, 02 April 2015)

*

* File Name   : E:\SwashProjects\CN2_POM\CN2_POM_report.txt

* Description : PERLKA Pome fruit

* Substance   : CN2Gr

*

* Creation    : 06-Mar-2018, 15:46

*

* Remarks : SWASH report helps you to set up the needed runs to calculate the PECsw and PECsed, occuring in the EU

*           for the selected substance, used on the selected crop. The scenario code informs you which models you need to

*           run for this scenario.

*           D1-D6: drainage entries calculated by the MACRO model, fate in surface water calculated by the TOXSWA model

*

*           R1-R4: runoff and erosion entries calculated by the PRZM model, fate in surface water calculated by the TOXSWA model

*

*

*           For STREAMS the Mean Deposition and Mass Loading, as calculated by the FOCUS Drift Calculator, have been multiplied by a

*           factor 1.2 to account for pesticide mass incoming from the upstream catchment as decided by the FOCUS Surface Water

*           Scenarios Working Group.

*

*

**********************************************************************************************************************************

*  CREATED RUNS

**********************************************************************************************************************************

*

*                                                     |--------------- APPLICATION -----------------|----- on Water Surface -----|

* -ID------Crop(1st/2nd)-------Scenario-WaterbodyType-|-Method--------First/Last/Interval--#---Rate-|-Mean Deposition-Mass Loading

(d)          (kg/ha) (% of Appl. Rate)    (mg/m2)

*

* 880   Pome/stone fruit, ea(1st) D3_Ditch              granular appl. 1-Apr /1-May /1      1  300.0000     0.000           0.000

*

* 881   Pome/stone fruit, ea(1st) D4_Pond               granular appl. 6-Apr /6-May /1      1  300.0000     0.000           0.000

*

* 882   Pome/stone fruit, ea(1st) D4_Stream             granular appl. 6-Apr /6-May /1      1  300.0000     0.000           0.000

*

* 883   Pome/stone fruit, ea(1st) D5_Pond               granular appl. 18-Mar/17-Apr/1      1  300.0000     0.000           0.000

*

* 884   Pome/stone fruit, ea(1st) D5_Stream             granular appl. 18-Mar/17-Apr/1      1  300.0000     0.000           0.000

*

* 885   Pome/stone fruit, ea(1st) R1_Pond               granular appl. 1-Apr /1-May /1      1  300.0000     0.000           0.000

*

* 886   Pome/stone fruit, ea(1st) R1_Stream             granular appl. 1-Apr /1-May /1      1  300.0000     0.000           0.000

*

* 887   Pome/stone fruit, ea(1st) R2_Stream             granular appl. 1-Mar /31-Mar/1      1  300.0000     0.000           0.000

*

* 888   Pome/stone fruit, ea(1st) R3_Stream             granular appl. 18-Mar/17-Apr/1      1  300.0000     0.000           0.000

*

* 889   Pome/stone fruit, ea(1st) R4_Stream             granular appl. 1-Mar /31-Mar/1      1  300.0000     0.000           0.000

*

************************************************** Surface WAter Scenarios Help **************************************************

## Application in Sugar beet

* SWASH report file

* made by FOCUS-SWASH UI v. 5 (internal version 5.1.0, 02 April 2015)

*

* File Name   : E:\SwashProjects\CN2_SB\CN2_SB_report.txt

* Description : PERLKA sugar beet

* Substance   : CN2Gr

*

* Creation    : 05-Mar-2018, 14:30

*

* Remarks : SWASH report helps you to set up the needed runs to calculate the PECsw and PECsed, occuring in the EU

*           for the selected substance, used on the selected crop. The scenario code informs you which models you need to

*           run for this scenario.

*           D1-D6: drainage entries calculated by the MACRO model, fate in surface water calculated by the TOXSWA model

*

*           R1-R4: runoff and erosion entries calculated by the PRZM model, fate in surface water calculated by the TOXSWA model

*

*

*           For STREAMS the Mean Deposition and Mass Loading, as calculated by the FOCUS Drift Calculator, have been multiplied by a

*           factor 1.2 to account for pesticide mass incoming from the upstream catchment as decided by the FOCUS Surface Water

*           Scenarios Working Group.

*

*

**********************************************************************************************************************************

*  CREATED RUNS

**********************************************************************************************************************************

*

*                                                     |--------------- APPLICATION -----------------|----- on Water Surface -----|

* -ID------Crop(1st/2nd)-------Scenario-WaterbodyType-|-Method--------First/Last/Interval--#---Rate-|-Mean Deposition-Mass Loading

(d)          (kg/ha) (% of Appl. Rate)    (mg/m2)

*

* 865   Sugar beets(1st)         D3_Ditch              soil incorp.   11-Apr/11-May/1      1  500.0000     0.000           0.000

*

* 866   Sugar beets(1st)         D4_Pond               soil incorp.   20-Apr/20-May/1      1  500.0000     0.000           0.000

*

* 867   Sugar beets(1st)         D4_Stream             soil incorp.   20-Apr/20-May/1      1  500.0000     0.000           0.000

*

* 868   Sugar beets(1st)         R1_Pond               soil incorp.   2-Apr /2-May /1      1  500.0000     0.000           0.000

*

* 869   Sugar beets(1st)         R1_Stream             soil incorp.   2-Apr /2-May /1      1  500.0000     0.000           0.000

*

* 870   Sugar beets(1st)         R3_Stream             soil incorp.   6-Mar /5-Apr /1      1  500.0000     0.000           0.000

*

************************************************** Surface WAter Scenarios Help **************************************************

## Application in vegetables (leafy)

* SWASH report file

* made by FOCUS-SWASH UI v. 5 (internal version 5.1.0, 02 April 2015)

*

* File Name   : E:\SwashProjects\CN2_Veg\CN2_Veg_report.txt

* Description : PERLKA vegetable leafy

* Substance   : CN2Gr

*

* Creation    : 02-Mar-2018, 14:47

*

* Remarks : SWASH report helps you to set up the needed runs to calculate the PECsw and PECsed, occuring in the EU

*           for the selected substance, used on the selected crop. The scenario code informs you which models you need to

*           run for this scenario.

*           D1-D6: drainage entries calculated by the MACRO model, fate in surface water calculated by the TOXSWA model

*

*           R1-R4: runoff and erosion entries calculated by the PRZM model, fate in surface water calculated by the TOXSWA model

*

*

*           For STREAMS the Mean Deposition and Mass Loading, as calculated by the FOCUS Drift Calculator, have been multiplied by a

*           factor 1.2 to account for pesticide mass incoming from the upstream catchment as decided by the FOCUS Surface Water

*           Scenarios Working Group.

*

*

**********************************************************************************************************************************

*  CREATED RUNS

**********************************************************************************************************************************

* -ID------Crop(1st/2nd)-------Scenario-WaterbodyType-|-Method--------First/Last/Interval--#---Rate-|-Mean Deposition-Mass Loading

(d)          (kg/ha) (% of Appl. Rate)    (mg/m2)

*

* 839   Vegetables, leafy(1st)   D3_Ditch              soil incorp.   11-Apr/11-May/1      1  500.0000     0.000           0.000

*

* 840   Vegetables, leafy(2nd)   D3_Ditch              soil incorp.   22-Jul/21-Aug/1      1  500.0000     0.000           0.000

*

* 841   Vegetables, leafy(1st)   D4_Pond               soil incorp.   26-Apr/26-May/1      1  500.0000     0.000           0.000

*

* 842   Vegetables, leafy(1st)   D4_Stream             soil incorp.   26-Apr/26-May/1      1  500.0000     0.000           0.000

*

* 843   Vegetables, leafy(1st)   D6_Ditch              soil incorp.   1-Aug /31-Aug/1      1  500.0000     0.000           0.000

*

* 844   Vegetables, leafy(1st)   R1_Pond               soil incorp.   6-Apr /6-May /1      1  500.0000     0.000           0.000

*

* 845   Vegetables, leafy(2nd)   R1_Pond               soil incorp.   17-Jul/16-Aug/1      1  500.0000     0.000           0.000

*

* 846   Vegetables, leafy(1st)   R1_Stream             soil incorp.   6-Apr /6-May /1      1  500.0000     0.000           0.000

*

* 847   Vegetables, leafy(2nd)   R1_Stream             soil incorp.   17-Jul/16-Aug/1      1  500.0000     0.000           0.000

*

* 848   Vegetables, leafy(1st)   R2_Stream             soil incorp.   14-Feb/16-Mar/1      1  500.0000     0.000           0.000

*

* 849   Vegetables, leafy(2nd)   R2_Stream             soil incorp.   17-Jul/16-Aug/1      1  500.0000     0.000           0.000

*

* 850   Vegetables, leafy(1st)   R3_Stream             soil incorp.   15-Feb/17-Mar/1      1  500.0000     0.000           0.000

*

* 851   Vegetables, leafy(2nd)   R3_Stream             soil incorp.   1-Jun /1-Jul /1      1  500.0000     0.000           0.000

*

* 852   Vegetables, leafy(1st)   R4_Stream             soil incorp.   15-Feb/17-Mar/1      1  500.0000     0.000           0.000

*

* 853   Vegetables, leafy(2nd)   R4_Stream             soil incorp.   1-Jun /1-Jul /1      1  500.0000     0.000           0.000

*

************************************************** Surface WAter Scenarios Help **************************************************

## Application in vines

* SWASH report file

* made by FOCUS-SWASH UI v. 5 (internal version 5.1.0, 02 April 2015)

*

* File Name   : E:\SwashProjects\CN2_VI\CN2_VI_report.txt

* Description : PERLKA Vine

* Substance   : CN2Gr

*

* Creation    : 07-Mar-2018, 07:49

*

* Remarks : SWASH report helps you to set up the needed runs to calculate the PECsw and PECsed, occuring in the EU

*           for the selected substance, used on the selected crop. The scenario code informs you which models you need to

*           run for this scenario.

*           D1-D6: drainage entries calculated by the MACRO model, fate in surface water calculated by the TOXSWA model

*

*           R1-R4: runoff and erosion entries calculated by the PRZM model, fate in surface water calculated by the TOXSWA model

*

*

*           For STREAMS the Mean Deposition and Mass Loading, as calculated by the FOCUS Drift Calculator, have been multiplied by a

*           factor 1.2 to account for pesticide mass incoming from the upstream catchment as decided by the FOCUS Surface Water

*           Scenarios Working Group.

*

*

**********************************************************************************************************************************

*  CREATED RUNS

**********************************************************************************************************************************

*

*                                                     |--------------- APPLICATION -----------------|----- on Water Surface -----|

* -ID------Crop(1st/2nd)-------Scenario-WaterbodyType-|-Method--------First/Last/Interval--#---Rate-|-Mean Deposition-Mass Loading

(d)          (kg/ha) (% of Appl. Rate)    (mg/m2)

*

* 890   Vines, early applns(1st) D6_Ditch              granular appl. 18-Jan/17-Feb/1      1  300.0000     0.000           0.000

*

* 891   Vines, early applns(1st) R1_Pond               granular appl. 1-Apr /1-May /1      1  300.0000     0.000           0.000

*

* 892   Vines, early applns(1st) R1_Stream             granular appl. 1-Apr /1-May /1      1  300.0000     0.000           0.000

*

* 893   Vines, early applns(1st) R2_Stream             granular appl. 1-Mar /31-Mar/1      1  300.0000     0.000           0.000

*

* 894   Vines, early applns(1st) R3_Stream             granular appl. 18-Mar/17-Apr/1      1  300.0000     0.000           0.000

*

* 895   Vines, early applns(1st) R4_Stream             granular appl. 24-Feb/26-Mar/1      1  300.0000     0.000           0.000

*

************************************************** Surface WAter Scenarios Help **************************************************