Study Report

Predicted Environmental Concentrations in Surface Water 
of PERLKA®, Cyanamid, Urea and Dicyandiamide after fertilization with PERLKA® based on FOCUS STEP3

Simulations in potatoes

Sponsor

AlzChem Trostberg GmbH

Dr.-Albert-Frank-Str. 32

83308 Trostberg

Germany

Institute

Fraunhofer-Institute for Molecular
	Biology and Applied Ecology (IME)

Auf dem Aberg 1

57392 Schmallenberg
	Germany

Director Applied Ecology
	Prof. Dr. Christoph Schäfers

Author
Dr. Michael Klein

Dr. Judith Klein

November 12, 2019

This page was intentionally left blank for statements of the sponsor or submitter.

Statement of compliance

This study “Predicted Environmental Concentrations in Surface Water of PERLKA®, Cyanamid, Urea and Dicyandiamide after fertilization with PERLKA® based on FOCUS STEP3, Simulations for potatoes” was conducted according to the procedures described herein. This report is a true and accurate record of the results obtained. There were no circumstances that may have adversely impacted the quality or integrity of the study.

The GLP-regulation is not applicable. However, the study was performed in accordance to the “Codex of Good Modelling Practices” (Görlitz 1993 und Travis 1995).

______________________________________	__12 November 2019______

Dr. Judith Klein		Date

Exposure Modelling

Fraunhofer Institute

Auf dem Aberg 1

57392 Schmallenberg

Germany

Tel +49 2972 302 256

Fax +49 2972 302 319

michael.klein@ime.fraunhofer.de

Contents	page

Statement of compliance	3

1.	Simulation models	5

2.	Input parameters	5

2.1	Agricultural use pattern	5

2.2	Substance properties	7

3.	Results	12

4.	Conclusions	14

5.	References	15

6.	Appendix: SWASH Report Files	17

6.1	Potatoes, 300kg, soil incoperated at 15 cm	17

# Simulation models

For the STEP3 calculations the computer tool SWASH was used which mainly creates the necessary input data for MACRO 5.5.4, PRZM 4.3.1 and TOXSWA 4.4 which were used for the simulations.  All models are described in FOCUS (2001). The standard buffer zone was 1 m.

They represent start-of-the-art PEC-calculations for all type active compounds (pesticides, biocides and veterinary compounds). They are also the most recent versions. The history of versions is summarised at the FOCUS homepage (https://esdac.jrc.ec.europa.eu/projects/focus-dg-sante).

Unfortunately, MACRO (the drainage model) is only able to simulate the behaviour of single metabolite in soil after the simulation run for the parent compound has been completed. Thus it is not possible to consider the full pathway: PERLKA® → cyanamid→urea respectively PERLKA® → cyanamid → dicyanadiamide.

PRZM can describe three transformation schemes and up to two metabolites: 1) a parent with one metabolite, 2) a parent with two sequential metabolites and 3) a parent with two metabolites.

In 2015 FOCUS_TOXSWA 4.4.3 was released, enabling the simulation of metabolites formed in water and formed in sediment (Beltman et al., 2014). This model is able to consider the full cascade of all 4 substances within a signle run.

In order to consider the all substances – at least for the run-off scenarios - the calculation in SWASH was done in a step wise approach:

Run 1) PERLKA® → cyanamid→urea

Run 2) PERLKA® → cyanamid → dicyanadiamide.

For MACRO (drainage scenarios) no simulations for urea and dicyanadiamide were performed.

# Input parameters

## Agricultural use pattern

As scenario 300 kg/ha of PERLKA® is applied in potatoes “iccorporated at 15 cm”, which means a uniform incorporation into the upper 15 cm of the soil.

Based on the information given in Error: Reference source not found the following application dates were automatically calculated for potatoes using the default settings.

Table 1: Application pattern in winter cereals considered for the simulations

| App. ID | Scenario | application date calculated by FOCUS PAT |
| --- | --- | --- |
| 1 | R1_Pond | 26. Apr 84 |
| 1 | R1_Stream | 26. Apr 84 |
| 1 | R2_Stream | 01. Mar 77 |
| 1 | R3_Stream | 28. Mar 80 |

## Substance properties

PERLKA®

The maximum concentration of calcium cyanamide in PERLKA® is about 45%. In order to adequately simulate the gradual hydrolysis of cyanamide from PERLKA® granules to soil, cyanamide was defined as a metabolite. According to experimental data the half-life of PERLKA® (CaCN2) in soil was found to be between 0.60 days and 2.51 days (Klein 2019). The experimental values were normalised to 20 °C using a Q10 factor of 2.2 as recommended by FOCUS (2000). The experimental half-lives were also normalised to pF 2 using an exponent of 0.7 as the model requires degradation at optimised moisture conditions. Also the moisture correction was done according to FOCUS (2000). The geometric mean of all normalised half-lives was found to be 0.721 days. This value was considered for the modelling transformation in soil (see Klein (2019)).

A half life of 1 day was considered for the water phase. This value was used as a minimum number for the simulation model FOCUS TOXSWA.

Cyanamide

According to experimental data cyanamide is further transformed to urea (Vilsmeier et Amberger 1978). Also for cyanamide the experimental half-lives were normalised to 20 °C using a Q10-Factor of 2.2 as given by FOCUS (2000). However, for cyanamide no soil moisture normalisation was done since according to the experimental results the degradation of cyanamide does not increase with soil moisture (Klein 2019). Consequently, the soil moisture correction in the models FOCUS MACRO and PRZM are not suitable and the moisture correction was switched off. For the modelling a half-life of 0.78 days was used. The value represents the geometric mean of all experimental data after normalisation to 20 °C but without soil moisture normalisation [see Klein 2019].

Furthermore, according to EFSA (2010) the half life of cyanamide in the water-sediment system is 3.5 days (geometric mean of two studies). According to the FOCUS recommendations, this value of 3.5 days must be considered for the transformation of cyanamide in water whereas a default value of 1000 days was considered for the sediment phase. The experimental disappearance time cannot be used for the modelling since it includes not only degradation but also transportation which is considered in the model separately.

Urea

According to experimental data cyanamide is further transformed to urea (Vilsmeier et Amberger 1978). The formation fraction from cyanamide to urea was set to 95.7 %.

For the half-life of urea 3.9 days at 20°C was taken. This is the geometric mean given in EFSA (2010). The computer automatically transfers the half lives at standard temperatures into the actual conditions of the scenarios. The sorption constant in soil KOC was set to 7.2 L/kg which was calculated from Hongprayoon (1991). The taken KOC value for urea corresponds to the mean of KOC values ranging from 5.3 to 9.1.

Plant uptake was not considered since the granules are usually applied before emergence of the crop.

Dicyandiamide

In addition, cyanamide is also transformed to dicyandiamide. The formation fraction from cyanamide to dicyandiamide was set to 4.25 %. For dicyandiamide a half-life of 11.1 days at 20°C was considered. The sorption constant in soil KOC was set to 5.25 L/kg (registration dossier).

Plant uptake was not considered since the granules are usually applied before emergence of the crop.

All DegT50 values in water, sediment-system and in soil considered in the simulations are presented in Table 2.

Table 2: DegT50-values (d) of PERLKA®, cyanamide, urea, dicyandiamide

| Parameter | Calcium cyanamide (PERLKA®) | Cyanamide | Urea | Dicyandiamide |
| --- | --- | --- | --- | --- |
| Water | 1 (at 20 °C) | 3.5 (at 20 °C) | 4.8 (at 20°C) | 1000 (at 20 °C) |
| Sediment | 1 (at 20 °C) | 1000 (at 20 °C) | 1000 (at 20 °C) | 1000 (at 20 °C) |
| Soil | 0.721 (at 20 °C) | 0.78 (at 20 °C) | 3.9 (at 20°C) | 11.1 (at 20 °C) |

The DegT5050 values in soil represent geometric mean half lives of several studies which are summarised in Klein (2019).

PERLKA® granules cannot be dissolved in water without being transformed to cyanamide. In order to simulate the fate of PERLKA® realistically the sorption constant in soil KOC was set to an artificial, high number (172,400 L/kg). That should guarantee that the granules remain at the applied location in soil and are only transformed to cyanamide without leaching to deeper soil layers. For PERLKA® residues in surface water the same value was considered as for cyanamide.

For cyanamide an average (geomean) sorption constant of 4 L/kg was considered which was based on experimental sorption studies (EFSA 2010). It was considered more reliable than results based on the HPLC methodology.

Cyanamide has a Henry’s law constant of 2.68×10-5 J/mol. However, this value cannot be entered directly into the simulation model but will be internally calculated based on water solubility, vapour pressure and molecular mass.

Plant uptake via roots was not considered since the granules are usually applied before emergence of the crop.

All other input parameters used for the simulations are summarised in Table 3.

Table 3: Other input parameters used for the simulations of PERLKA, cyanamide, urea, dicyandiamide

| Parameter | PERLKA® | Cyanamide | Urea | Dicyandiamide | Remark |
| --- | --- | --- | --- | --- | --- |
| Parent/Transformation product | Parent | Transformation product of calcium cyanamide | Transformation product of cyanamide | Transformation product of cyanamide |  |
| Sorption constant KOC in soil (L/kg) | 172400 | 4 | 7.2 | 5.25 | EFSA (2010) |
| Sorption constant KOC in water body (L/kg) | 0 | 4 | 7.2 | 5.25 | EFSA (2010) |
| Freundlich exponent (-) | 1 | 1 | 1 | 1 | EFSA (2010) |
| Vapour pressure ( Pa) | 0 at 20°C | 0.5 at 20°C | 0.0016 at 25°C | <0.004 at 100°C (assumed to be 0 at 20°C) | EFSA (2010) |
| Molar mass (g/mol) | 80.11 | 42.04 | 60.06 | 42.04 |  |
| Water solubility (20°C, mg/L) | 800000 | 800000 | 624000 | 32000 | EFSA (2010) |
| Molar enthalpy of vaporisation | 95000 | 95000 | 95000 | 95000 | default |
| Molar enthalpy of dissolution | 27000 | 27000 | 27000 | 27000 | default |
| Diffusion coefficient in water | 4.3 10-5 | 4.3 10-5 | 4.3 10-5 | 4.3 10-5 | default |
| Diffusion coefficient in air | 0.43 | 0.43 | 0.43 | 0.43 | default |
| Plant uptake factor | 0 | 0 | 0 | 0 | default |

Metabolites in SWASH

For the STEP3 calculations the computer tool SWASH was used which mainly creates the necessary input data for MACRO 5.5.4, PRZM 4.3.1 and TOXSWA 4.4 which were used for the simulations.  All models are described in FOCUS (2001). The standard buffer zone was 1 m.

They represent start-of-the-art PEC-calculations for all type active compounds (pesticides, biocides and veterinary compounds). They are also the most recent versions. The history of versions is summarised at the FOCUS homepage (https://esdac.jrc.ec.europa.eu/projects/focus-dg-sante).

Unfortunately, MACRO (the drainage model) is only able to simulate the behaviour of single metabolite in soil after the simulation run for the parent compound has been completed. Thus it is not possible to consider the full pathway: PERLKA® → cyanamid→urea respectively PERLKA® → cyanamid → dicyanadiamide.

PRZM can describe three transformation schemes and up to two metabolites: 1) a parent with one metabolite, 2) a parent with two sequential metabolites and 3) a parent with two metabolites.

In 2015 FOCUS_TOXSWA 4.4.3 was released, enabling the simulation of metabolites formed in water and formed in sediment (Beltman et al., 2014). This model is able to consider the full cascade of all 4 substances within a signle run.

In order to consider the all substances – at least for the run-off scenarios - the calculation in SWASH was done in a step wise approach:

Run 1) PERLKA® → cyanamid→urea

Run 2) PERLKA® → cyanamid → dicyanadiamide.

For MACRO (drainage scenarios) no simulations for urea and dicyanadiamide were performed.

# Results

In the calculation it is considered that PERLKA® is formed to cyanamide, which in turn is formed to urea and dicyanadiamid. This “metabolite scheme” is only possible for PRZM and TOXWA. Consequently, only run-off scenarios (R-scenarios) are reported.

The maximum concentrations for PERLKA®, cyanamide, urea and dicyandiamide are summarised in the following Table 5. In all considered “R” scenarios, the maximum concentrations in surface water are simulated for urea.

Table 4: Maximum concentrations (PECmax) of PERLKA®, Cyanamide, Urea and Dicyandiamide at FOCUS Step 3

| App. Id. | Scenaro/Crop | PECmax (µg/L) |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | PERLKA® | Cyanamide | Urea | Dicyandiamide |
| 1 | Potatoes 300kg, incorp.in soil at 15 cm | 0.557 | 449.9 | 1165.6 | 38.71 |

Table 5: Global maximum concentrations or PERLKA®, cyanamide, urea and dicyandiamide at FOCUS Step 3

| App. Id | Crop |  | PERLKA® | Cyanamide |  | Urea |  | Dicyandiamide |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | PECsw (µg/L) | PECsw (µg/L) | PECsed (µg/kg) | PECsw (µg/L) | PECsed (µg/kg) | PECsw (µg/L) | PECsed (µg/kg) |
| 1 | Potatoes 300kg, incorp.in soil at 15 cm | R1_Pond | 0.00008 | 0.05153 | 0.008903 | 0.6815 | 0.154 | 0.02635 | 0.01302 |
|  |  | R1_Stream | 0.0114 | 9.433 | 0.5415 | 124.2 | 7.723 | 4.767 | 0.2826 |
|  |  | R2_Stream | 0.02022 | 2.212 | 0.3149 | 91.38 | 7.11 | 3.65 | 0.2704 |
|  |  | R3_Stream | 0.557 | 449.9 | 21.23 | 1165.6 | 59.52 | 38.71 | 1.893 |

# Conclusions

In the calculation it is considered that PERLKA® is formed to cyanamide, which in turn is formed to urea and dicyanadiamid. This “metabolite scheme” is only possible for PRZM and TOXWA. Consequently, only run-off scenarios (R-scenarios) are reported.

In all considered “R” scenarios, the maximum concentrations in surface water are simulated for urea.

# References

- AGES (2019): “National exposure assessment for the authorization of plant protection products (PPP) in Austria: Calculation of predicted environmental concentrations (PEC) in soil, groundwater, surface water, sediment and air.” Information for notifier/applicant and other interested parties Document version 02 (2017/04/19).

- EFSA (2010): “Conclusion on the peer review of the pesticide risk assessment of the active substance cyanamide”. EFSA Journal 2010;8(11):1873.

- EFSA (2014): “EFSA Guidance Document for evaluating laboratory and field dissipation studies to obtain DegT50 values of active substances of plant protection products and transformation products of these active substances in soil”, EFSA Journal 2014;12(5):3662

- FOCUS (2000): “FOCUS groundwater scenarios in the EU review of active substances”. Report of the FOCUS Groundwater Scenarios Workgroup, EC Doc. Ref. SANCO/321/2000 rev. 2; and “Generic guidance for FOCUS groundwater scenarios”. FOCUS Groundwater Scenario Workgroup, May 2001, Version

- FOCUS (2001). “FOCUS Surface Water Scenarios in the EU Evaluation Process under 91/414/EEC”. Report of the FOCUS Working Group on Surface Water Scenarios, EC

- FOCUS (2007). “Landscape And Mitigation Factors In Aquatic Risk Assessment. Volume 1. Extended Summary and Recommendations”. Report of the FOCUS Working Group on Landscape and MitigationFactors in Ecological Risk Assessment, EC Document Reference SANCO/10422/2005 v2.0. 169 pp.

- Görlitz. G. (1993): „Verfahrensregeln zur korrekten Durchführung und Auswertung von Modellrechnungen zur Simulation des Umweltverhaltens von Pflanzenschutzmitteln.“

- Klein J and Klein M. (2018): “Predicted Environmental Concentrations in Groundwater of Cyanamide and Calcium cyanamide after fertilization with PERLKA® using FOCUSPEARL Simulations in cabbage and potatoes.”

- Klein J. and Klein M. (2019): “ Moisture and Temperature correction of experimentally determined DT50 values for calcium cyanamide and cyanamide according to FOCUS”

- Klein, M. (2019): Moisture and Temperature correction of experimentally determined DT50 values for calcium cyanamide and cyanamide according to FOCUS. Report Fraunhofer IME, Schmallenberg.

- Travis. K.Z. (1995): “Recommendations for the correct use of models and reporting of modelling results.”: ‘Leaching Models and EU registration’. Final report of the FOCUS Group. Doc. 4952/VI/95.

- Vilsmeier, K.; Amberger, A. (1978): Modellversuche zum Umsatz von gemahlenen Kalkstickstoff und Perlkalkstickstoff in Abhängigkeit von Bodenfeuchtigkeit und Applikationsform. In: Z. Acker- und Pflanzenbau 147, S. 68–77

# Appendix: SWASH Report Files

## Potatoes, 300kg, soil incoperated at 15 cm

* SWASH report file

* made by FOCUS-SWASH UI v. 5 (internal version 5.1.0, 02 April 2015)

*

* File Name   : E:\SwashProjects\PERLKA\CN2_PO\CN2_PO_report.txt

* Description : PERLKA, 300 kg/ha, potatoes, 15 cm soil incorp.

* Substance   : CN2Gr

*

* Creation    : 08-Nov-2019, 07:07

*

* Remarks : SWASH report helps you to set up the needed runs to calculate the PECsw and PECsed, occuring in the EU

*           for the selected substance, used on the selected crop. The scenario code informs you which models you need to

*           run for this scenario.

*           D1-D6: drainage entries calculated by the MACRO model, fate in surface water calculated by the TOXSWA model

*

*           R1-R4: runoff and erosion entries calculated by the PRZM model, fate in surface water calculated by the TOXSWA model

*

*

*           For STREAMS the Mean Deposition and Mass Loading, as calculated by the FOCUS Drift Calculator, have been multiplied by a

*           factor 1.2 to account for pesticide mass incoming from the upstream catchment as decided by the FOCUS Surface Water

*           Scenarios Working Group.

*

*

**********************************************************************************************************************************

*  CREATED RUNS

**********************************************************************************************************************************

*

*                                                     |--------------- APPLICATION -----------------|----- on Water Surface -----|

* -ID------Crop(1st/2nd)-------Scenario-WaterbodyType-|-Method--------First/Last/Interval--#---Rate-|-Mean Deposition-Mass Loading

(d)          (kg/ha) (% of Appl. Rate)    (mg/m2)

*

* 159   Potatoes(1st)            D3_Ditch              soil incorp.   26-Apr/26-May/1      1  300.0000     0.000           0.000

*

* 160   Potatoes(1st)            D4_Pond               soil incorp.   8-May /7-Jun /1      1  300.0000     0.000           0.000

*

* 161   Potatoes(1st)            D4_Stream             soil incorp.   8-May /7-Jun /1      1  300.0000     0.000           0.000

*

* 162   Potatoes(1st)            D6_Ditch              soil incorp.   27-Mar/26-Apr/1      1  300.0000     0.000           0.000

*

* 163   Potatoes(2nd)            D6_Ditch              soil incorp.   22-Jul/21-Aug/1      1  300.0000     0.000           0.000

*

* 164   Potatoes(1st)            R1_Pond               soil incorp.   21-Apr/21-May/1      1  300.0000     0.000           0.000

*

* 165   Potatoes(1st)            R1_Stream             soil incorp.   21-Apr/21-May/1      1  300.0000     0.000           0.000

*

* 166   Potatoes(1st)            R2_Stream             soil incorp.   1-Mar /31-Mar/1      1  300.0000     0.000           0.000

*

* 167   Potatoes(1st)            R3_Stream             soil incorp.   27-Mar/26-Apr/1      1  300.0000     0.000           0.000

*

************************************************** Surface WAter Scenarios Help **************************************************

Surface WAter Scenarios Help **************************************************