# APPENDIX G

TEST PROTOCOL AND RESULTS OF STEPS 1, 2 & 3 COMPARISONS

# Part 1

PROTOCOL

Evaluation of FOCUS Surface Water

Step 1, 2 and 3 Scenarios
 using Representative Test Substances

PLEASE NOTE

The data presented in the following Appendix represent model runs conducted with the Step 3 models in order to test the Step 1, 2 and 3 scenarios and also to provide example output for a series of "real" compounds in order that the pass/fail rate of these compounds could be assessed through the Step 1, 2 and 3 process and compared with current methodology. The modelling was conducted between November 2001 and June 2002 using development versions of the PRZM, MACRO and TOXSWA modelling tools. As all of these tools have subsequently been modified in response to the beta testing programme, and the SWASH tool has become fully commissioned, it is no longer possible for modellers to exactly reproduce the results found in these sections and they should be regarded as examples only. Therefore, for modellers looking for a test data set to reproduce as part of training/familiarisation, it is recommended that the test dataset released with the modelling tools on the JRC website at ISPRA be used.

INTRODUCTION

The FOCUS Surface Water Scenarios Work Group is developing a stepwise procedure for the calculation of exposure concentrations in surface water and sediment for use in ecological risk assessments under the framework of EU directive 91/414 (see Figure 1).

At the start of the exposure assessment process the user calculates the “worst case loading” situation using STEP1-2, a Visual Basic Calculator.  The calculated result may be compared to the relevant toxicity concentrations, the lethal or effect concentration, L(E)C50, or the No-effect concentration, NOEC, of the aquatic organisms investigated. If the use is considered safe no further work is required on the specific topic. If the result indicates the use being not safe, the next step is entered, i.e. Step 2.

Step 2 assumes a loading based on sequential application patterns taking into account the degradation of the substance in between successive applications. Again the PEC’s are calculated and may be compared to the same and/or different toxicity levels for aquatic organisms. Safe use again implies no further work while Toxicity Exposure Ratios below the specified trigger values indicates that a Step 3 calculation using deterministic models is necessary.

In the Step 3 the realistic worst case scenarios developed by the working group covering major agricultural areas in Europe are used together with the chosen model(s).  For ‘run-off’ scenarios PRZM and TOXSWA are selected whereas for ‘drainage scenarios’ MACRO is used in place of PRZM.

Before implementation of this approach and the associated scenarios within a regulatory framework a systematic evaluation of the tools and a comparison of the PEC values at each step are needed.

# OBJECTIVE

A number of inter-related objectives are defined:

## Objective 1: Definition of Generic Run-off and Drainage Losses at Steps 1 and 2.

To define the fraction of applied chemical or residue remaining in the soil that is lost via run-off or drainage to an adjacent water body at step 1 and 2, based on the results of step 3 calculations.  To make intra-scenario comparisons at step 3, i.e. establish how the runoff and drainage losses, as well as the PEC’s are influenced by compound properties.

## Objective 2: Comparison of PEC values and TER’s at Steps 1,2 and 3.

To make a quantitative comparison of PEC values with relevant ecotoxicological endpoints at each step using a number of test compounds in order to demonstrate the stepwise approach and to compare with existing risk assessment principles.  To make inter-scenario comparisons at step 3 (relative vulnerability).

The process of achieving the three objectives will be described in subsequent sections together with a format for the presentation of results and evaluations.

# TEST SUBSTANCES

Three groups of test compounds are to be used depending upon the objective and associated evaluations.

## Test compounds for derivation of step 1 and 2 run-off and drainage losses and intra-scenario comparisons at step 3

This test will be conducted with a series of Compound parameters to evaluate the impact of environmental fate properties on the magnitude of run-off and drainage losses and subsequent PEC values in surface water and sediment.  These are not real compounds but cover the typical range of key parameters influencing losses via runoff and drainage and fate in surface water.  They are summarized in Table 1.

# Test compounds for comparison of PEC values and TER’s at Steps 1,2 and 3

This test will be conducted with a series of real compounds compiled from a set of EPPO ‘Compound compounds’ created for a risk assessment workshop, from recently completed EU reviews leading to the inclusion of the compounds on Annex I and from ECPA-member companies.  A total of seven compounds are included.  The properties of these compounds are included in Table 2.

# TEST SIMULATIONS

## Derivation of step 1 and 2 run-off and drainage losses and intra-scenario comparisons at step 3

The nine test compounds will be evaluated in the step 1 and 2 calculator using both the original Excel version and the most recent Visual Basic version as a validation of the new program.  The compounds will also be run with the step 3 scenarios using three different application times.  These are summarized in Table 3.  The PAT (Pesticide Application Timing) calculator built into the MACRO in FOCUS and PRZM in FOCUS shells selects the actual application date.  It is recommended to use an application of + 8 days around the start date in Table 3.

## Comparison of PEC values and TER’s at Steps 1,2 and 3

The seven test compounds will be tested with the step 1 and 2 calculator plus the relevant step 3 scenarios as part of a typical risk assessment process.  Application scenarios are given in Table 4.  For single applications use a + 8 day application window around the given date in the PAT calculator.  For multiple applications follow the rules given in the table.  As a representative of the owner is evaluating each compound then the proposed input parameters in Table 2 should be reviewed and modified as appropriate.  Please make changes and notify FOCUS group as soon as possible.  The evaluator should also summarize ecotoxicology endpoints.

# PRESENTATION OF RESULTS

In order to harmonize the results the model outputs will be summarized in standard tables. These tables will be supplied in an Excel spreadsheet allowing data to be compiled and filtered for presentation purposes.

# DISTRIBUTION OF WORK EFFORT AND TIMELINE

In order to expedite completion of the task and to achieve the greatest level of testing of the tools and models with maximum feedback, a number of Surface Water Workgroup members and ECPA companies have indicated support for conducting this test protocol.  However other organizations are also invited to take part.

Table G.1-1 	Test compounds for derivation of step 1 and 2 run-off and drainage losses and intra-scenario comparisons at step 3

|  | Example Compound: |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | A | B | C | D | E | F | G | H | I |
| Molar mass (g/mol) | 300 for all compounds |  |  |  |  |  |  |  |  |
| Vapour pressure (Pa @ 20C) | 1.0 x 10-7 for all compounds |  |  |  |  |  |  |  |  |
| Water solubility (mg/L @ 20C) | 1.0 for all compounds |  |  |  |  |  |  |  |  |
| Log Kow | 0.2 | 2.1 | 4.1 | 0.2 | 2.1 | 4.1 | 0.2 | 2.1 | 4.1 |
| Application rate (kg/ha) | 0.1 | 0.1 | 0.1 | 0.1 | 0.1 | 0.1 | 0.1 | 0.1 | 0.1 |
| Soil half-life (days) | 3 | 3 | 3 | 30 | 30 | 30 | 300 | 300 | 300 |
| Koc (cm3 g-1) | 10 | 100 | 1000 | 10 | 100 | 1000 | 10 | 100 | 1000 |
| Freundlich 1/n | 1 |  |  |  |  |  |  |  |  |
| Surface water half-life (days) | 1 | 1 | 1 | 10 | 10 | 10 | 100 | 100 | 100 |
| Sediment half-life (days) | 3 | 3 | 3 | 30 | 30 | 30 | 300 | 300 | 300 |
| Total system half-life (days) | 1 | 1 | 2 | 10 | 12 | 22 | 102 | 126 | 219 |

Table G.1-2 	Test compounds for comparison of PEC values and TER’s at Steps 1,2 and 3

|  | Test Compound |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 1 (I) | 2 (H) | 3 (H) | 4 (I) | 5 (F) | 6 (H) | 6 * (metab) | 7 (F) |
| Molar mass (g/mol) | 190.3 | 215.7 | 221.0 | 505.2 | 376.0 | 255.0 | 197.0 | 286.1 |
| Vapour pressure (Pa @ 20C) | 0.017 | 3.85 x 10-5 | <1 x 10-5 | 1.24 x 10-8@ 25°C | 6.4 x 10-9 | 3.78 x 10-9 Pa | Assumed low (<1E-7 mPa) | 1.3 x 10-4 |
| Water solubility (mg/L @ 20C) | 6000 @ 25C | 30 | 620 @ 25°C | 0.0002 @25°C | 1.15 | 91 @ pH 7 | Assumed same as parent | 2.6 @ pH 7 |
| Log Kow | 1.6 | 2.5 | 2.8 | 4.6 | 3.2 | 2.0 | N/A | 3.0 |
| Soil half-life (days) | 6 | 43 | 4 | 26 | 250 | 28 | 58 a | 50 |
| Koc | 15 | 91 | 1 | 1024000 | 860 | 66 | 580 | 500 |
| Freundlich 1/n | 1.0 | 0.88 | 1.0 | 0.93 | 1.0 | 1.0 | 1.0 | 1.0 |
| Surface water half-life (days) | 6 | 26 | 1.5 | 0.7 | 6 | 24 | 33 | 2.5 |
| Sediment half-life (days) | 6 | 26 | 1.5 | 76 | 118 | 24 | 33 b | 28 |
| Fish acute LC50 (mg/L) | 0.115 | 11 | 18 | 0.00026 | 1.9 | 14.3 | 39 | >18 |
| Aquatic Invertebrate EC50 (mg/L) | 0.41 | 87 | <100 | 0.00025 | >5 | >100 | >49 | 4 |
| Algae EC50 (mg/L) | 1.4 | 0.043 | 9.8 | >9.1 | 0.014 | 49.8 | >45 | >1.02 |
| Lemna EC50 (mg/L) | -- | 0.020 | 12.3 | -- | 1.4 | 12.3 | -- | -- |
| Fish chronic NOEC (mg/L) | -- | 0.25 | 0.2 | 0.000032 | 0.3 | 0.2 | -- | 0.05 |
| Aquatic invertebrate chronic NOEC (mg/L) | 0.11 | 0.040 | 0.1 | 0.0000041 | 0.648 | 0.1 | -- | 1.95 |
| Method of application | Pre-plant soil inc | pre-em ground app | post-em ground app | orchard air-blast | Air-blast in vines | Post-em ground app | N/A | Air-blast in vines |
| Crop | Potatoes | maize | winter wheat | Apples | Vines | Cereals | N/A | Vines |
| Application rate (kg/ha) | 3 | 1 | 1 | 0.0125 | 0.075 | 0.4 (NZ) 0.2 (SZ) | N/A | 0.75 |
| Number of applications | 1 | 1 | 1 | 3 | 5 | 1 | N/A | 4 |
| Timing | minus 1day before planting | First possible app 1 day after sowing | First possible app day after 1 March | First possible app day after 15 April. min 14 day interval between remain-ing apps. | First possible app day after 1 April. Min 10 days between remain-ing apps. | First possible app day after 1 March | First possible app day after 1 April. Min 14 days between remain-ing apps |  |

Soil inc = soil incorporation,  pre-em = pre-emergence, ground app = ground application,  NZ = Northern zone, SZ = Southern zone,  App = applications.  
a  Maximum occurrence in soil = 11%,  b Maximum occurrence in sediment = 35%.

* the fraction of formation of the metabolite of substance H is DENIS

Table G.1-3  	Application dates to winter wheat (& maize) for first group of test compounds (Compounds A to I)

| Scenario | Autumn (pre-emergence) | Spring (post-emergence) | Summer (post-emergence) |
| --- | --- | --- | --- |
| D1 | 23 September (266) | 6 May (126) | 23 June (174) |
| D2 | 23 October (296) | 4 April (94) | 30 June (181) |
| D3 | 19 November (323) | 16 April (106) | 24 July (205) |
| D4 | 20 September (263) | 18 March (77) | 21 June (172) |
| D5 | 19 October (292) | 14 March (73) | 31 May (151) |
| D6 | 28 November (332) | 16 February (47) | 30 March (89) |
| R1 | 10 November (314) | 1 April (91) | 10 June (161) |
| R2a | 28 April (118) | 30 May (150) | 15 August (227) |
| R3 | 28 November (332) | 16 March (75) | 10 May (130) |
| R4 | 4 November (308) | 3 March (62) | 27 April (117) |

a Maize.  Winter wheat not grown at R2

Table G.1-4  	Matrix of step 3 calculations needed for test compounds 1 to 7 (Table 2).  Table gives crop and application dates.

| Scenario | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| D1 |  |  | Winter wheat 25 March |  |  | Winter wheat 25 March 0.4 kg/ha |  |
| D2 |  |  | Winter wheat 4 April |  |  | Winter wheat 4 April 0.4 kg/ha |  |
| D3 | Potatoes 1 May | Maize 1 May | Winter wheat 16 April | Apples Appn 1: 15 April Minm 14 days between remaining 2 applns Last appln before 30 Jun |  | Winter wheat 16 April 0.4 kg/ha |  |
| D4 | Potatoes 12 May | Maize 5 May | Winter wheat 18 March | Apples Appn 1: 15 April Minm 14 days between remaining 2 applns Last appln before 30 Jun |  | Winter wheat 18 March 0.4 kg/ha |  |
| D5 |  | Maize 1 May | Winter wheat 14 March | Apples Appn 1: 15 April Minm 14 days between remaining 2 applns Last appln before 30 Jun |  | Winter wheat 14 March 0.4 kg/ha |  |
| D6 | Potatoesa 1 April 1 August | Maize 15 April | Winter wheat 16 February | Apples Appn 1: 15 April Minm 14 days between remaining 2 applns Last appln before 30 Jun | Vines Appn 1: 1 April Minm 10 days between remaining 4 applns Last appln before 30 Jun | Winter wheat 16 February 0.2 kg/ha | Vines Appn 1: 1 April Minm 14 days between remaining 3 applns Last appln before 30 Jun |
| R1 | Potatoes 21 April | Maize 21 April | Winter wheat 1 April | Apples Appn 1: 15 April Minm 14 days between remaining 2 applns Last appln before 30 Jun | Vines Appn 1: 1 April Minm 10 days between remaining 4 applns Last appln before 30 Jun | Winter wheat 1 April 0.4 kg/ha | Vines Appn 1: 1 April Minm 14 days between remaining 3 applns Last appln before 30 Jun |
| R2 | Potatoes 27 February | Maize 21 April |  | Apples Appn 1: 15 April Minm 14 days between remaining 2 applns Last appln before 30 Jun | Vines Appn 1: 1 April Minm 10 days between remaining 4 applns Last appln before 30 Jun |  | Vines Appn 1: 1 April Minm 14 days between remaining 3 applns Last appln before 30 Jun |
| R3 | Potatoes 31 March | Maize 1 May | Winter wheat 16 March | Apples Appn 1: 15 April Minm 14 days between remaining 2 applns Last appln before 30 Jun | Vines Appn 1: 1 April Minm 10 days between remaining 4 applns Last appln before 30 Jun | Winter wheat 16 March 0.2 kg/ha | Vines Appn 1: 1 April Minm 14 days between remaining 3 applns |
| R4 |  | Maize 26 March | Winter wheat | Apples Appn 1: 15 April Minm 14 days between remaining 2 applns Last appln before 30 Jun | Vines Appn 1: 1 April Minm 10 days between remaining 4 applns Last appln before 30 Jun | Winter wheat 0.2 kg/ha | Vines Appn 1: 1 April Minm 14 days between remaining 3 applns Last appln before 30 Jun |

a Two crops, therefore make two runs

Figure G.1-1	Stepwise Procedure for Calculating Exposure to Aquatic Organisms

# Part 2

### Tabular Results of MACRO Results with Compounds A to I

Table G.2-1	Maximum hourly fluxes from the field (% of applied in final year) for compounds A to I in step 3 scenarios D1 to D6 for autumn applications.  For comparison the amounts lost via runoff/drainage at step 2 are included.

| Compound | Northern European Scenarios |  |  |  |  | S. European Scenarios | Step 2 Scenarios assuming runoff/ drainage inputs of |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | D1 | D2 | D3 | D4 | D5 | D6 | 5 % | 4 % |
| Application date | 14 Sep to 23 Sep | 15 Oct to 23 Oct | 13 Nov to 19 Nov | 12 Sep to 20 Sep | 11 Oct to 19 Oct | 21 Nov to 6 Dec | N. Europe | S. Europe |
| A | 0.006 | 0.262 | < 0.001 | < 0.001 | < 0.001 | 0.011 | 1.96 | 1.57 |
| B | 0.002 | 0.149 | < 0.001 | < 0.001 | < 0.001 | 0.012 | 1.75 | 1.40 |
| C | <0.001 | <0.001 | < 0.001 | < 0.001 | < 0.001 | 0.001 | 0.85 | 0.68 |
| D | 0.036 | 0.605 | 0.002 | 0.033 | 0.059 | 0.080 | 4.50 | 3.60 |
| E | 0.058 | 0.297 | < 0.001 | 0.012 | 0.023 | 0.064 | 4.02 | 3.22 |
| F | 0.050 | 0.021 | < 0.001 | 0.002 | 0.003 | 0.050 | 1.96 | 1.56 |
| G | 0.065 | 0.646 | 0.016 | 0.086 | 0.189 | 0.180 | 4.89 | 3.91 |
| H | 0.108 | 0.450 | 0.005 | 0.050 | 0.091 | 0.168 | 4.37 | 3.50 |
| I | 0.045 | 0.126 | < 0.001 | 0.012 | 0.019 | 0.133 | 2.13 | 1.70 |

Table G.2-2	Maximum daily fluxes from the field (% of applied in final year) for compounds A to I in step 3 scenarios D1 to D6 for autumn applications.  For comparison the amounts lost via runoff/drainage at step 2 are included.

| Compound | Northern European Scenarios |  |  |  |  | S. European Scenarios | Step 2 Scenarios assuming runoff/ drainage inputs of |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | D1 | D2 | D3 | D4 | D5 | D6 | 5 % | 4 % |
| Application date | 14 Sep to 23 Sep | 15 Oct to 23 Oct | 13 Nov to 19 Nov | 12 Sep to 20 Sep | 11 Oct to 19 Oct | 21 Nov to 6 Dec | N. Europe | S. Europe |
| A | 0.10 | 1.39 | < 0.01 | < 0.01 | < 0.01 | 0.20 | 1.96 | 1.57 |
| B | 0.04 | 0.58 | < 0.01 | <0.01 | < 0.01 | 0.21 | 1.75 | 1.40 |
| C | <0.01 | <0.01 | < 0.01 | < 0.01 | < 0.01 | 0.01 | 0.85 | 0.68 |
| D | 0.67 | 3.84 | 0.04 | 0.53 | 0.74 | 1.00 | 4.50 | 3.60 |
| E | 1.10 | 1.53 | < 0.01 | 0.21 | 0.29 | 0.82 | 4.02 | 3.22 |
| F | 0.10 | 0.15 | < 0.01 | 0.02 | 0.02 | 0.43 | 1.96 | 1.56 |
| G | 1.22 | 4.11 | 0.39 | 1.45 | 2.53 | 2.92 | 4.89 | 3.91 |
| H | 2.05 | 2.79 | 0.11 | 0.87 | 1.26 | 2.19 | 4.37 | 3.50 |
| I | 0.85 | 0.92 | < 0.01 | 0.17 | 0.17 | 1.67 | 2.13 | 1.70 |

Table G.2-3	Total fluxes from the field from the time of application (% of applied in final year) for compounds A to I in step 3 scenarios D1 to D6 for autumn applications.  For comparison the amounts lost via runoff/drainage at step 2 are included.

| Compound | Northern European Scenarios |  |  |  |  | S. European Scenarios | Step 2 Scenarios assuming runoff/ drainage inputs of |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | D1 | D2 | D3 | D4 | D5 | D6 | 5 % | 4 % |
| Application date | 14 Sep to 23 Sep | 15 Oct to 23 Oct | 13 Nov to 19 Nov | 12 Sep to 20 Sep | 11 Oct to 19 Oct | 21 Nov to 6 Dec | N. Europe | S. Europe |
| A | 1.6 | 3.1 | <0.1 | < 0.1 | < 0.1 | 1.7 | 1.96 | 1.57 |
| B | 0.4 | 2.1 | < 0.1 | < 0.1 | < 0.1 | 1.4 | 1.75 | 1.40 |
| C | <0.1 | <0.1 | < 0.1 | < 0.1 | < 0.1 | < 0.1 | 0.85 | 0.68 |
| D | 8.9 | 19.3 | 8.4 | 11.6 | 9.6 | 11.3 | 4.50 | 3.60 |
| E | 11.1 | 18.1 | < 0.1 | 2.4 | 1.6 | 8.1 | 4.02 | 3.22 |
| F | 1.3 | 1.3 | < 0.1 | 0.2 | 0.1 | 0.9 | 1.96 | 1.56 |
| G | 23.2 | 35.2 | 57.2 | 40.7 | 50.1 | 38.2 | 4.89 | 3.91 |
| H | 25.6 | 34.5 | 15.1 | 19.4 | 15.5 | 23.5 | 4.37 | 3.50 |
| I | 12.8 | 11.7 | < 0.1 | 1.9 | 1.3 | 4.0 | 2.13 | 1.70 |

Table G.2-4	Maximum hourly fluxes from the field (% of applied in final year) for compounds A to I in Step 3 Scenarios D1 to D6 for spring applications.  For comparison the amounts lost via runoff/drainage at step 2 are included.

| Compound | Northern European Scenarios |  |  |  |  | S. European Scenarios | Step 2 Scenarios assuming runoff/ drainage inputs of |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | D1 | D2 | D3 | D4 | D5 | D6 | 2 % | 4 % |
| Application date | 6 May to 15 May | 31 Mar to 4 Apr | 8 Apr to 16 Apr | 11 Mar to 18 Mar | 6 Mar to 14 Mar | 8 Feb to 14 Feb | N. Europe | S. Europe |
| A | 0.002 | 0.318 | < 0.001 | < 0.001 | < 0.001 | 0.076 | 0.59 | 1.17 |
| B | < 0.001 | 0.085 | < 0.001 | < 0.001 | < 0.001 | 0.021 | 0.53 | 1.05 |
| C | < 0.001 | <0.001 | < 0.001 | < 0.001 | < 0.001 | 0.008 | 0.26 | 0.51 |
| D | 0.005 | 0.454 | < 0.001 | 0.003 | 0.002 | 0.091 | 1.35 | 2.70 |
| E | 0.007 | 0.230 | < 0.001 | 0.001 | 0.001 | 0.029 | 1.21 | 2.41 |
| F | 0.001 | 0.004 | < 0.001 | < 0.001 | < 0.001 | < 0.001 | 0.59 | 1.17 |
| G | 0.032 | 0.546 | 0.010 | 0.031 | 0.079 | 0.124 | 1.47 | 2.93 |
| H | 0.039 | 0.287 | 0.004 | 0.029 | 0.046 | 0.067 | 1.31 | 2.62 |
| I | 0.025 | 0.090 | < 0.001 | 0.009 | 0.011 | 0.070 | 0.64 | 1.28 |

Table G.2-5	Maximum daily fluxes from the field (% of applied in final year) for compounds A to I in step 3 scenarios D1 to D6 for spring applications.  For comparison the amounts lost via runoff/drainage at step 2 are included.

| Compound | Northern European Scenarios |  |  |  |  | S. European Scenarios | Step 2 Scenarios assuming runoff/ drainage inputs of |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | D1 | D2 | D3 | D4 | D5 | D6 | 2 % | 4 % |
| Application date | 6 May to 15 May | 31 Mar to 4 Apr | 8 Apr to 16 Apr | 11 Mar to 18 Mar | 6 Mar to 14 Mar | 8 Feb to 14 Feb | N. Europe | S. Europe |
| A | < 0.01 | 1.51 | < 0.01 | < 0.01 | < 0.01 | 1.44 | 0.59 | 1.17 |
| B | < 0.01 | 0.38 | < 0.01 | < 0.01 | < 0.01 | 0.90 | 0.53 | 1.05 |
| C | < 0.01 | <0.01 | < 0.01 | < 0.01 | < 0.01 | 0.05 | 0.26 | 0.51 |
| D | 0.08 | 2.17 | < 0.01 | 0.05 | 0.03 | 1.75 | 1.35 | 2.70 |
| E | 0.13 | 1.66 | < 0.01 | 0.02 | 0.01 | 0.53 | 1.21 | 2.41 |
| F | 0.02 | 0.02 | < 0.01 | < 0.01 | < 0.01 | < 0.01 | 0.59 | 1.17 |
| G | 0.65 | 2.80 | 0.24 | 0.58 | 1.15 | 2.44 | 1.47 | 2.93 |
| H | 0.73 | 2.30 | 0.09 | 0.50 | 0.60 | 1.03 | 1.31 | 2.62 |
| I | 0.46 | 0.60 | < 0.01 | 0.12 | 0.12 | 0.87 | 0.64 | 1.28 |

Table G.2-6	Total fluxes from the field from the time of application (% of applied in final year) for compounds A to I in step 3 scenarios D1 to D6 for spring applications.  For comparison the amounts lost via runoff/drainage at step 2 are included.

| Compound | Northern European Scenarios |  |  |  |  | S. European Scenarios | Step 2 Scenarios assuming runoff/ drainage inputs of |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | D1 | D2 | D3 | D4 | D5 | D6 | 2 % | 4 % |
| Application date | 6 May to 15 May | 31 Mar to 4 Apr | 8 Apr to 16 Apr | 11 Mar to 18 Mar | 6 Mar to 14 Mar | 8 Feb to 14 Feb | N. Europe | S. Europe |
| A | < 0.1 | 5.2 | < 0.1 | < 0.1 | < 0.1 | 3.3 | 0.59 | 1.17 |
| B | < 0.1 | 1.3 | < 0.1 | < 0.1 | < 0.1 | 1.1 | 0.53 | 1.05 |
| C | < 0.1 | <0.1 | < 0.1 | < 0.1 | < 0.1 | 0.1 | 0.26 | 0.51 |
| D | 1.6 | 14.0 | 0.7 | 1.3 | 0.5 | 5.1 | 1.35 | 2.70 |
| E | 3.1 | 7.0 | < 0.1 | 0.3 | 0.1 | 0.6 | 1.21 | 2.41 |
| F | 0.6 | 0.3 | < 0.1 | < 0.1 | < 0.1 | < 0.1 | 0.59 | 1.17 |
| G | 13.8 | 28.5 | 27.4 | 18.5 | 23.5 | 21.2 | 1.47 | 2.93 |
| H | 11.0 | 13.9 | 13.1 | 12.6 | 9.8 | 14.2 | 1.31 | 2.62 |
| I | 7.2 | 9.5 | < 0.1 | 1.4 | 0.9 | 2.8 | 0.64 | 1.28 |

Table G.2-7	Maximum hourly fluxes from the field (% of applied in final year) for compounds A to I in step 3 scenarios D1 to D6 for summer applications.  For comparison the amounts lost via runoff/drainage at step 2 are included.

| Compound | Northern European Scenarios |  |  |  |  |  | S. European Scenarios | Step 2 Scenarios assuming runoff/ drainage inputs of |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | D1 | D2 | D3 | D4 | D5 | D6 |  | 2 % | 3 % |
| Application date | 15 Jun to 23 Jun | 30 Jun | 24 Jul to 31 Jul | 15 Jun to 21 Jun | 26 May to 31 May | 22 Mar to 30 Mar |  | N. Europe | S. Europe |
| A | 0.001 | < 0.001 | < 0.001 | < 0.001 | < 0.001 | < 0.001 |  | 0.39 | 0.59 |
| B | <0.001 | < 0.001 | < 0.001 | < 0.001 | < 0.001 | < 0.001 |  | 0.35 | 0.53 |
| C | <0.001 | < 0.001 | < 0.001 | < 0.001 | < 0.001 | < 0.001 |  | 0.17 | 0.26 |
| D | 0.004 | 0.011 | < 0.001 | 0.001 | 0.004 | 0.001 |  | 0.90 | 1.35 |
| E | 0.002 | 0.014 | < 0.001 | 0.002 | 0.003 | < 0.001 |  | 0.80 | 1.21 |
| F | 0.002 | 0.003 | < 0.001 | 0.001 | < 0.001 | < 0.001 |  | 0.39 | 0.59 |
| G | 0.031 | 0.049 | 0.010 | 0.038 | 0.066 | 0.094 |  | 0.98 | 1.47 |
| H | 0.012 | 0.033 | 0.001 | 0.008 | 0.016 | 0.072 |  | 0.87 | 1.31 |
| I | 0.011 | 0.026 | < 0.001 | 0.003 | 0.005 | 0.031 |  | 0.43 | 0.64 |

Table G.2-8	Maximum daily fluxes from the field (% of applied in final year) for compounds A to I in step 3 scenarios D1 to D6 for summer applications.  For comparison the amounts lost via runoff/drainage at step 2 are included.

| Compound | Northern European Scenarios |  |  |  |  | S. European Scenarios | Step 2 Scenarios assuming runoff/ drainage inputs of |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | D1 | D2 | D3 | D4 | D5 | D6 | 2 % | 3 % |
| Application date | 15 Jun | 30 Jun | 24 Jul | 16 Jun | 27 May | 24 Mar | N. Europe | S. Europe |
| A | 0.01 | < 0.01 | < 0.01 | < 0.01 | < 0.01 | < 0.01 | 0.39 | 0.59 |
| B | <0.01 | < 0.01 | < 0.01 | < 0.01 | < 0.01 | < 0.01 | 0.35 | 0.53 |
| C | <0.01 | < 0.01 | < 0.01 | < 0.01 | < 0.01 | < 0.01 | 0.17 | 0.26 |
| D | 0.07 | 0.11 | 0.01 | 0.03 | 0.07 | 0.01 | 0.90 | 1.35 |
| E | 0.04 | 0.13 | < 0.01 | 0.04 | 0.04 | < 0.01 | 0.80 | 1.21 |
| F | 0.03 | 0.01 | < 0.01 | 0.01 | < 0.01 | < 0.01 | 0.39 | 0.59 |
| G | 0.61 | 0.50 | 0.23 | 0.69 | 1.10 | 1.56 | 0.98 | 1.47 |
| H | 0.23 | 0.30 | 0.02 | 0.14 | 0.24 | 1.09 | 0.87 | 1.31 |
| I | 0.21 | 0.15 | < 0.01 | 0.04 | 0.06 | 0.38 | 0.43 | 0.64 |

Table G.2-9	Total fluxes from the field from the time of application (% of applied in final year) for compounds A to I in step 3 scenarios D1 to D6 for summer applications. For comparison the amounts lost via runoff/drainage at step 2 are included.

| Compound | Northern European Scenarios |  |  |  |  | S. European Scenarios | Step 2 Scenarios assuming runoff/ drainage inputs of |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | D1 | D2 | D3 | D4 | D5 | D6 | 2 % | 3 % |
| Application date | 15 Jun | 30 Jun | 24 Jul | 16 Jun | 27 May | 24 Mar | N. Europe | S. Europe |
| A | 0.2 | < 0.1 | < 0.1 | < 0.1 | < 0.1 | < 0.1 | 0.39 | 0.59 |
| B | <0.1 | < 0.1 | < 0.1 | < 0.1 | < 0.1 | < 0.1 | 0.35 | 0.53 |
| C | <0.1 | < 0.1 | < 0.1 | < 0.1 | < 0.1 | < 0.1 | 0.17 | 0.26 |
| D | 1.2 | 0.5 | 1.2 | 0.7 | 0.3 | 0.1 | 0.90 | 1.35 |
| E | 0.6 | 0.7 | < 0.1 | 0.6 | 0.2 | < 0.1 | 0.80 | 1.21 |
| F | 0.5 | 0.2 | < 0.1 | 0.1 | < 0.1 | < 0.1 | 0.39 | 0.59 |
| G | 13.2 | 7.7 | 27.3 | 21.0 | 14.9 | 20.9 | 0.98 | 1.47 |
| H | 4.1 | 4.4 | 3.0 | 3.6 | 3.6 | 9.1 | 0.87 | 1.31 |
| I | 3.0 | 2.2 | < 0.1 | 0.5 | 0.3 | 1.5 | 0.43 | 0.64 |

# Part 3

### Tabular Results of PRZM Results with Compounds A to I

Table G.3-1	Maximum daily fluxes from the field (% of applied in final year) for Compounds A to I in step 3 scenarios R1 to R4 for autumn applications.  For comparison the amounts lost via runoff/drainage at step 2 are included.

| Compound | Northern European Scenarios |  | Southern European Scenarios |  |  |  |  |  | Step 2 Scenarios assuming runoff/drainage inputs of |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | R1 |  | R2 |  | R3 |  | R4 |  | 5% |  | 4% |  |
| Application date | 13-Nov |  | 1-May |  | 26-Nov |  | 4-Nov |  | N. Europe |  | S. Europe |  |
|  | Runoff | Erosion | Runoff | Erosion | Runoff | Erosion | Runoff | Erosion | RO | Er | RO | Er |
| A | <0.01 | <0.01 | 0.01 | <0.01 | 2.08 | <0.01 | <0.01 | <0.01 | 1.96 | 0.03 | 1.57 | 0.02 |
| B | <0.01 | <0.01 | <0.01 | <0.01 | 1.77 | 0.01 | <0.01 | <0.01 | 1.75 | 0.23 | 1.40 | 0.19 |
| C | <0.01 | <0.01 | <0.01 | <0.01 | 0.67 | 0.06 | <0.01 | <0.01 | 0.85 | 1.13 | 0.68 | 0.91 |
| D | 0.05 | <0.01 | 0.05 | <0.01 | 2.68 | <0.01 | 0.07 | <0.01 | 4.50 | 0.06 | 3.60 | 0.05 |
| E | 0.05 | <0.01 | 0.06 | <0.01 | 2.64 | 0.01 | 0.01 | <0.01 | 4.02 | 0.54 | 3.22 | 0.43 |
| F | 0.03 | <0.01 | 0.05 | <0.01 | 1.20 | 0.09 | 0.02 | <0.01 | 1.96 | 2.60 | 1.56 | 2.08 |
| G | 0.07 | <0.01 | 0.05 | <0.01 | 2.75 | <0.01 | <0.01 | <0.01 | 4.89 | 0.07 | 3.91 | 0.05 |
| H | 0.09 | <0.01 | 0.07 | <0.01 | 2.74 | 0.01 | 0.04 | <0.01 | 4.37 | 0.58 | 3.50 | 0.47 |
| I | 0.12 | <0.01 | 0.13 | <0.01 | 1.27 | 0.09 | 0.06 | <0.01 | 2.13 | 2.83 | 1.70 | 2.26 |

Table G.3-2	Total annual fluxes from the field (% of applied in final year) for compounds A to I in step 3 scenarios R1 to R4 for autumn applications.  For comparison the amounts lost via runoff/drainage at step 2 are included.

| Compound | Northern European Scenarios |  | Southern European Scenarios |  |  |  |  |  | Step 2 Scenarios assuming runoff/drainage inputs of |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | R1 |  | R2 |  | R3 |  | R4 |  | 5% |  | 4% |  |
| Application date | 13-Nov |  | 1-May |  | 26-Nov |  | 4-Nov |  | N. Europe |  | S. Europe |  |
|  | Runoff | Erosion | Runoff | Erosion | Runoff | Erosion | Runoff | Erosion | RO | Er | RO | Er |
| A | <0.1 | <0.1 | <0.1 | <0.1 | 3.2 | <0.1 | <0.1 | <0.1 | 1.96 | 0.03 | 1.57 | 0.02 |
| B | <0.1 | <0.1 | <0.1 | <0.1 | 3.1 | <0.1 | <0.1 | <0.1 | 1.75 | 0.23 | 1.40 | 0.19 |
| C | <0.1 | <0.1 | <0.1 | <0.1 | 1.0 | <0.1 | <0.1 | <0.1 | 0.85 | 1.13 | 0.68 | 0.91 |
| D | <0.1 | <0.1 | 0.1 | <0.1 | 4.2 | <0.1 | <0.1 | <0.1 | 4.50 | 0.06 | 3.60 | 0.05 |
| E | 0.1 | <0.1 | 0.1 | <0.1 | 4.5 | <0.1 | <0.1 | <0.1 | 4.02 | 0.54 | 3.22 | 0.43 |
| F | 0.1 | <0.1 | 0.1 | <0.1 | 1.7 | 0.1 | <0.1 | <0.1 | 1.96 | 2.60 | 1.56 | 2.08 |
| G | 0.1 | <0.1 | 0.1 | <0.1 | 4.3 | <0.1 | <0.1 | <0.1 | 4.89 | 0.07 | 3.91 | 0.05 |
| H | 0.1 | <0.1 | 0.2 | <0.1 | 4.7 | <0.1 | <0.1 | <0.1 | 4.37 | 0.58 | 3.50 | 0.47 |
| I | 0.2 | <0.1 | 0.7 | <0.1 | 1.9 | 0.1 | 0.2 | <0.1 | 2.13 | 2.83 | 1.70 | 2.26 |

Table G.3-3	Maximum daily fluxes from the field (% of applied in final year) for compounds A to I in step 3 scenarios R1 to R4 for spring applications.  For comparison the amounts lost via runoff/drainage at step 2 are included.

| Compound | Northern European Scenarios |  | Southern European Scenarios |  |  |  |  |  | Step 2 Scenarios assuming runoff/drainage inputs of |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | R1 |  | R2 |  | R3 |  | R4 |  | 2% |  | 4% |  |
| Application date | 7-Apr |  | 28-May |  | 23-Mar |  | 3-Mar |  | N. Europe |  | S. Europe |  |
|  | Runoff | Erosion | Runoff | Erosion | Runoff | Erosion | Runoff | Erosion | RO | Er | RO | Er |
| A | 0.02 | <0.01 | <0.01 | <0.01 | <0.01 | <0.01 | 0.04 | <0.01 | 0.59 | 0.01 | 1.17 | 0.02 |
| B | 0.02 | <0.01 | <0.01 | <0.01 | <0.01 | <0.01 | 0.03 | <0.01 | 0.53 | 0.07 | 1.05 | 0.14 |
| C | 0.01 | <0.01 | <0.01 | <0.01 | <0.01 | <0.01 | 0.01 | <0.01 | 0.26 | 0.34 | 0.51 | 0.68 |
| D | 0.07 | <0.01 | 0.15 | <0.01 | <0.01 | <0.01 | 0.06 | <0.01 | 1.35 | 0.02 | 2.70 | 0.04 |
| E | 0.12 | <0.01 | 0.40 | <0.01 | 0.03 | <0.01 | 0.04 | <0.01 | 1.21 | 0.16 | 2.41 | 0.32 |
| F | 0.04 | <0.01 | 0.11 | 0.01 | 0.09 | <0.01 | 0.02 | <0.01 | 0.59 | 0.78 | 1.17 | 1.56 |
| G | 0.08 | <0.01 | 0.22 | <0.01 | 0.01 | <0.01 | 0.06 | <0.01 | 1.47 | 0.02 | 2.93 | 0.04 |
| H | 0.15 | <0.01 | 0.58 | <0.01 | 0.04 | <0.01 | 0.04 | <0.01 | 1.31 | 0.17 | 2.62 | 0.35 |
| I | 0.05 | <0.01 | 0.16 | 0.01 | 0.22 | <0.01 | 0.14 | <0.01 | 0.64 | 0.85 | 1.28 | 1.70 |

Table G.3-4	Total annual fluxes from the field (% of applied in final year) for compounds A to I in step 3 scenarios R1 to R4 for spring applications.  For comparison the amounts lost via runoff/drainage at step 2 are included.

| Compound | Northern European Scenarios |  | Southern European Scenarios |  |  |  |  |  | Step 2 Scenarios assuming runoff/drainage inputs of |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | R1 |  | R2 |  | R3 |  | R4 |  | 2% |  | 4% |  |
| Application date | 7-Apr |  | 28-May |  | 23-Mar |  | 3-Mar |  | N. Europe |  | S. Europe |  |
|  | Runoff | Erosion | Runoff | Erosion | Runoff | Erosion | Runoff | Erosion | RO | Er | RO | Er |
| A | <0.1 | <0.1 | <0.1 | <0.1 | <0.1 | <0.1 | <0.1 | <0.1 | 0.59 | 0.01 | 1.17 | 0.02 |
| B | <0.1 | <0.1 | <0.1 | <0.1 | <0.1 | <0.1 | <0.1 | <0.1 | 0.53 | 0.07 | 1.05 | 0.14 |
| C | <0.1 | <0.1 | <0.1 | <0.1 | <0.1 | <0.1 | <0.1 | <0.1 | 0.26 | 0.34 | 0.51 | 0.68 |
| D | 0.1 | <0.1 | 0.2 | <0.1 | <0.1 | <0.1 | 0.1 | <0.1 | 1.35 | 0.02 | 2.70 | 0.04 |
| E | 0.1 | <0.1 | 0.4 | <0.1 | <0.1 | <0.1 | <0.1 | <0.1 | 1.21 | 0.16 | 2.41 | 0.32 |
| F | 0.1 | <0.1 | 0.2 | <0.1 | 0.1 | <0.1 | <0.1 | <0.1 | 0.59 | 0.78 | 1.17 | 1.56 |
| G | 0.1 | <0.1 | 0.2 | <0.1 | <0.1 | <0.1 | 0.1 | <0.1 | 1.47 | 0.02 | 2.93 | 0.04 |
| H | 0.2 | <0.1 | 0.6 | <0.1 | 0.1 | <0.1 | <0.1 | <0.1 | 1.31 | 0.17 | 2.62 | 0.35 |
| I | 0.3 | <0.1 | 1.3 | <0.1 | 0.3 | <0.1 | 0.3 | <0.1 | 0.64 | 0.85 | 1.28 | 1.70 |

Table G.3-5	Maximum daily fluxes from the field (% of applied in final year) for compounds A to I in step 3 scenarios R1 to R4 for summer applications.  For comparison the amounts lost via runoff/drainage at step 2 are included.

| Compound | Northern European Scenarios |  | Southern European Scenarios |  |  |  |  |  | Step 2 Scenarios assuming runoff/drainage inputs of |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | R1 |  | R2 |  | R3 |  | R4 |  | 2% |  | 3% |  |
| Application date | 16-Jun |  | 20-Aug |  | 17-May |  | 27-Apr |  | N. Europe |  | S. Europe |  |
|  | Runoff | Erosion | Runoff | Erosion | Runoff | Erosion | Runoff | Erosion | RO | Er | RO | Er |
| A | <0.01 | <0.01 | <0.01 | <0.01 | <0.01 | <0.01 | <0.01 | <0.01 | 0.39 | 0.01 | 0.59 | 0.01 |
| B | 0.02 | <0.01 | <0.01 | <0.01 | <0.01 | <0.01 | <0.01 | <0.01 | 0.35 | 0.05 | 0.53 | 0.07 |
| C | 0.01 | <0.01 | <0.01 | <0.01 | <0.01 | <0.01 | <0.01 | <0.01 | 0.17 | 0.23 | 0.26 | 0.34 |
| D | 0.07 | <0.01 | 0.01 | <0.01 | <0.01 | <0.01 | 0.08 | <0.01 | 0.90 | 0.01 | 1.35 | 0.02 |
| E | 0.33 | <0.01 | 0.11 | <0.01 | 0.02 | <0.01 | 0.12 | <0.01 | 0.80 | 0.11 | 1.21 | 0.16 |
| F | 0.18 | 0.01 | 0.04 | 0.01 | <0.01 | <0.01 | 0.01 | <0.01 | 0.39 | 0.52 | 0.59 | 0.78 |
| G | 0.09 | <0.01 | 0.05 | <0.01 | 0.01 | <0.01 | 0.11 | <0.01 | 0.98 | 0.01 | 1.47 | 0.02 |
| H | 0.45 | <0.01 | 0.52 | <0.01 | 0.03 | <0.01 | 0.17 | <0.01 | 0.87 | 0.12 | 1.31 | 0.17 |
| I | 0.25 | 0.02 | 0.25 | 0.09 | 0.04 | <0.01 | 0.55 | <0.01 | 0.43 | 0.57 | 0.64 | 0.85 |

Table G.3-6	Total annual fluxes from the field (% of applied in final year) for compounds A to I in step 3 scenarios R1 to R4 for summer applications.  For comparison the amounts lost via runoff/drainage at step 2 are included.

| Compound | Northern European Scenarios |  | Southern European Scenarios |  |  |  |  |  | Step 2 Scenarios assuming runoff/drainage inputs of |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | R1 |  | R2 |  | R3 |  | R4 |  | 2% |  | 3% |  |
| Application date | 16-Jun |  | 20-Aug |  | 17-May |  | 27-Apr |  | N. Europe |  | S. Europe |  |
|  | Runoff | Erosion | Runoff | Erosion | Runoff | Erosion | Runoff | Erosion | RO | Er | RO | Er |
| A | <0.1 | <0.1 | <0.1 | <0.1 | <0.1 | <0.1 | <0.1 | <0.1 | 0.39 | 0.01 | 0.59 | 0.01 |
| B | <0.1 | <0.1 | <0.1 | <0.1 | <0.1 | <0.1 | <0.1 | <0.1 | 0.35 | 0.05 | 0.53 | 0.07 |
| C | <0.1 | <0.1 | <0.1 | <0.1 | <0.1 | <0.1 | <0.1 | <0.1 | 0.17 | 0.23 | 0.26 | 0.34 |
| D | 0.1 | <0.1 | <0.1 | <0.1 | <0.1 | <0.1 | 0.1 | <0.1 | 0.90 | 0.01 | 1.35 | 0.02 |
| E | 0.3 | <0.1 | 0.2 | <0.1 | <0.1 | <0.1 | 0.3 | <0.1 | 0.80 | 0.11 | 1.21 | 0.16 |
| F | 0.3 | <0.1 | 0.2 | <0.1 | <0.1 | <0.1 | <0.1 | <0.1 | 0.39 | 0.52 | 0.59 | 0.78 |
| G | 0.1 | <0.1 | 0.1 | <0.1 | <0.1 | <0.1 | 0.2 | <0.1 | 0.98 | 0.01 | 1.47 | 0.02 |
| H | 0.5 | <0.1 | 0.9 | <0.1 | <0.1 | <0.1 | 0.4 | <0.1 | 0.87 | 0.12 | 1.31 | 0.17 |
| I | 0.5 | <0.1 | 2.0 | 0.1 | 0.3 | <0.1 | 1.3 | <0.1 | 0.43 | 0.57 | 0.64 | 0.85 |

# Part 4

### Tabular Results of TOXSWA Results with Compounds A, D, E, F, H and I

Table G.4-1	Comparison of PECmax and 28 day time weighted Average concentration at steps 1, 2 and 3 for compound A in Northern European scenarios for autumn applications.

| Step | Scenario | Water |  | Sediment |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Global maximum (μg/L) | TWAC 28d (μg/L) | Global maximum (μg/kg DW) | TWAC 28d (μg/kg DW) |
| 1 | All | 33.82 | 1.78 | 3.29 | 0.18 |
| 2 | N. Europe | 6.59 | 0.35 | 0.66 | 0.049 |
| 3 | D1-Ditch | 1.00b | 0.62 | 0.21 | 0.19 |
|  | D1-Stream | 0.72b | 0.46 | 0.16 | 0.14 |
|  | D2-Ditch | 8.61b | 1.49 | 0.76 | 0.42 |
|  | D2-Stream | 5.68b | 1.02 | 0.51 | 0.29 |
|  | D3-Ditch | 0.50a | 0.013 | 0.029 | 0.004 |
|  | D4-Pond | 0.022a | 0.002 | 0.002 | 0.001 |
|  | D4-Stream | 0.46a | 0.006 | 0.022 | 0.002 |
|  | D5-Pond | 0.022a | 0.002 | 0.002 | 0.001 |
|  | D5-Stream | 0.49a | 0.006 | 0.023 | 0.002 |
|  | R1-Pond | 0.039a | 0.027 | 0.014 | 0.013 |
|  | R1-Stream | 0.35a | 0.003 | 0.012 | 0.002 |

a Peak occurs at time of spray drift event

b peak occurs at time of drainage event

Table G.4-2	Comparison of PECmax and 28 day time weighted average concentration at steps 1, 2 and 3 for compound A in Southern European scenarios for autumn applications.

| Step | Scenario | Water |  | Sediment |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Global maximum (μg/L) | TWAC 28d (μg/L) | Global maximum (μg/kg DW) | TWAC 28d (μg/kg DW) |
| 1 | All | 33.82 | 1.78 | 3.29 | 0.18 |
| 2 | S. Europe | 5.28 | 0.28 | 0.53 | 0.040 |
| 3 | D6-Ditch | 0.52a | 0.12 | 0.046 | 0.031 |
|  | R2-Stream | 0.46a | 0.007 | 0.022 | 0.002 |
|  | R3-Stream | 7.06c | 0.14 | 0.482 | 0.050 |
|  | R4-Stream | 0.35a | 0.003 | 0.013 | 0.001 |

a Peak occurs at time of spray drift event

c peak occurs at time of runoff event

Table G.4-3	Comparison of PECmax and 28 day time weighted average concentration at steps 1, 2 and 3 for compound A in Northern European scenarios for spring applications.

| Step | Scenario | Water |  | Sediment |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Global maximum (μg/L) | TWAC 28d (μg/L) | Global maximum (μg/kg DW) | TWAC 28d (μg/kg DW) |
| 1 | All | 33.82 | 1.78 | 3.29 | 0.18 |
| 2 | N. Europe | 2.02 | 0.11 | 0.20 | 0.015 |
| 3 | D1-Ditch | 0.51a | 0.062 | 0.049 | 0.017 |
|  | D1-Stream | 0.47a | 0.013 | 0.031 | 0.004 |
|  | D2-Ditch | 11.95b | 2.68 | 1.37 | 0.80 |
|  | D2-Stream | 8.97b | 1.67 | 0.82 | 0.49 |
|  | D3-Ditch | 0.51a | 0.016 | 0.032 | 0.004 |
|  | D4-Pond | 0.022a | 0.004 | 0.003 | 0.001 |
|  | D4-Stream | 0.40a | 0.001 | 0.007 | <0.001 |
|  | D5-Pond | 0.022a | 0.003 | 0.002 | 0.001 |
|  | D5-Stream | 0.43a | 0.001 | 0.007 | <0.001 |
|  | R1-Pond | 0.023a | 0.004 | 0.003 | 0.001 |
|  | R1-Stream | 0.35a | 0.003 | 0.012 | 0.001 |

a Peak occurs at time of spray drift event

b peak occurs at time of drainage event

Table G.4-4	Comparison of PECmax and 28 day time weighted average concentration at steps 1, 2 and 3 for Compound A in Southern European scenarios for spring applications.

| Step | Scenario | Water |  | Sediment |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Global maximum (μg/L) | TWAC 28d (μg/L) | Global maximum (μg/kg DW) | TWAC 28d (μg/kg DW) |
| 1 | All | 33.82 | 1.78 | 3.29 | 0.18 |
| 2 | S. Europe | 3.97 | 0.21 | 0.40 | 0.030 |
| 3 | D6-Ditch | 6.81b | 0.92 | 0.67 | 0.24 |
|  | R2-Stream | 0.46a | 0.002 | 0.011 | 0.001 |
|  | R3-Stream | 0.48a | 0.003 | 0.016 | 0.001 |
|  | R4-Stream | 5.12c | 0.071 | 0.293 | 0.019 |

a Peak occurs at time of spray drift event

b peak occurs at time of drainage event

c peak occurs at time of runoff event

Table G.4-5	Comparison of PECmax and 28 day time weighted average concentration at steps 1, 2 and 3 for compound A in Northern European scenarios for summer applications.

| Step | Scenario | Water |  | Sediment |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Global maximum (μg/L) | TWAC 28d (μg/L) | Global maximum (μg/kg DW) | TWAC 28d (μg/kg DW) |
| 1 | All | 33.82 | 1.78 | 3.29 | 0.18 |
| 2 | N. Europe | 1.36 | 0.073 | 0.14 | 0.010 |
| 3 | D1-Ditch | 0.51a | 0.084 | 0.043 | 0.025 |
|  | D1-Stream | 0.47 a | 0.013 | 0.030 | 0.003 |
|  | D2-Ditch | 0.51 a | 0.037 | 0.039 | 0.008 |
|  | D2-Stream | 0.48 a | 0.034 | 0.036 | 0.008 |
|  | D3-Ditch | 0.51 a | 0.018 | 0.030 | 0.004 |
|  | D4-Pond | 0.022 a | 0.002 | 0.002 | <0.001 |
|  | D4-Stream | 0.46 a | 0.006 | 0.022 | 0.001 |
|  | D5-Pond | 0.022 a | 0.002 | 0.002 | <0.001 |
|  | D5-Stream | 0.49 a | 0.006 | 0.023 | 0.001 |
|  | R1-Pond | 0.022 a | 0.002 | 0.002 | 0.001 |
|  | R1-Stream | 0.35 a | 0.003 | 0.013 | 0.001 |

a Peak occurs at time of spray drift event

Table G.4-6	Comparison of PECmax and 28 day time weighted average concentration at steps 1, 2 and 3 for compound A in Southern European scenarios for summer applications.

| Step | Scenario | Water |  | Sediment |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Global maximum (μg/L) | TWAC 28d (μg/L) | Global maximum (μg/kg DW) | TWAC 28d (μg/kg DW) |
| 1 | All | 33.82 | 1.78 | 3.29 | 0.18 |
| 2 | S. Europe | 2.02 | 0.11 | 0.20 | 0.015 |
| 3 | D6-Ditch | 0.51 a | 0.047 | 0.043 | 0.011 |
|  | R2-Stream | 0.47 a | 0.002 | 0.012 | 0.001 |
|  | R3-Stream | 0.48 a | 0.004 | 0.017 | 0.001 |
|  | R4-Stream | 0.35 a | 0.004 | 0.013 | 0.001 |

a Peak occurs at time of spray drift event

Table G.4-7	Comparison of PECmax and 28 day time weighted average concentration at steps 1, 2 and 3 for compound D in Northern European scenarios for autumn applications.

| Step | Scenario | Water |  | Sediment |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Global maximum (μg/L) | TWAC 28d (μg/L) | Global maximum (μg/kg DW) | TWAC 28d (μg/kg DW) |
| 1 | All | 33.82 | 14.92 | 3.29 | 1.49 |
| 2 | N. Europe | 15.69 | 6.96 | 1.55 | 0.73 |
| 3 | D1-Ditch | 4.09b | 2.24 | 1.19 | 1.18 |
|  | D1-Stream | 2.91b | 1.43 | 0.72 | 0.71 |
|  | D2-Ditch | 22.95 b | 8.51 | 3.75 | 3.45 |
|  | D2-Stream | 14.52b | 4.55 | 2.01 | 1.85 |
|  | D3-Ditch | 1.54 a | 1.12 | 0.74 | 0.71 |
|  | D4-Pond | 2.30 b | 2.11 | 1.26 | 1.23 |
|  | D4-Stream | 2.32 b | 1.93 | 1.03 | 0.99 |
|  | D5-Pond | 2.08 b | 1.73 | 0.88 | 0.85 |
|  | D5-Stream | 1.59 b | 0.99 | 0.57 | 0.52 |
|  | R1-Pond | 0.039 c | 0.027 | 0.014 | 0.013 |
|  | R1-Stream | 2.07 c | 0.032 | 0.12 | 0.016 |

a Peak occurs at time of spray drift event

b peak occurs at time of drainage event

c peak occurs at time of runoff event

Table G.4-8	Comparison of PECmax and 28 day time weighted average concentration at steps 1, 2 and 3 for compound D in Southern European scenarios for autumn applications.

| Step | Scenario | Water |  | Sediment |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Global maximum (μg/L) | TWAC 28d (μg/L) | Global maximum (μg/kg DW) | TWAC 28d (μg/kg DW) |
| 1 | All | 33.82 | 14.92 | 3.29 | 1.49 |
| 2 | S. Europe | 12.69 | 5.63 | 1.25 | 0.59 |
| 3 | D6-Ditch | 3.69 b | 1.32 | 0.63 | 0.56 |
|  | R2-Stream | 1.21c | 0.023 | 0.080 | 0.011 |
|  | R3-Stream | 9.04c | 0.18 | 0.63 | 0.10 |
|  | R4-Stream | 0.35a | 0.003 | 0.013 | 0.001 |

a Peak occurs at time of spray drift event

b peak occurs at time of drainage event

c peak occurs at time of runoff event

Table G.4-9	Comparison of PECmax and 28 day time weighted average concentration at steps 1, 2 and 3 for compound D in Northern European scenarios for spring applications.

| Step | Scenario | Water |  | Sediment |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Global maximum (μg/L) | TWAC 28d (μg/L) | Global maximum (μg/kg DW) | TWAC 28d (μg/kg DW) |
| 1 | All | 33.82 | 14.92 | 3.29 | 1.49 |
| 2 | N. Europe | 5.19 | 2.30 | 0.51 | 0.23 |
| 3 | D1-Ditch | 0.65a | 0.33 | 0.19 | 0.18 |
|  | D1-Stream | 0.40 a | 0.01 | 0.009 | 0.005 |
|  | D2-Ditch | 16.06b | 5.04 | 2.74 | 2.19 |
|  | D2-Stream | 10.36 b | 2.79 | 1.58 | 1.22 |
|  | D3-Ditch | 0.59 a | 0.11 | 0.096 | 0.067 |
|  | D4-Pond | 0.22 b | 0.21 | 0.14 | 0.13 |
|  | D4-Stream | 0.55b | 0.19 | 0.12 | 0.12 |
|  | D5-Pond | 0.093 b | 0.079 | 0.042 | 0.041 |
|  | D5-Stream | 0.46a | 0.055 | 0.036 | 0.030 |
|  | R1-Pond | 0.052 a | 0.035 | 0.016 | 0.016 |
|  | R1-Stream | 1.68 c | 0.032 | 0.11 | 0.015 |

a Peak occurs at time of spray drift event

b peak occurs at time of drainage event

c peak occurs at time of runoff event

Table G.4-10	Comparison of PECmax and 28 day time weighted average concentration at steps 1, 2 and 3 for compound D in Southern European scenarios for spring applications.

| Step | Scenario | Water |  | Sediment |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Global maximum (μg/L) | TWAC 28d (μg/L) | Global maximum (μg/kg DW) | TWAC 28d (μg/kg DW) |
| 1 | All | 33.82 | 14.92 | 3.29 | 1.49 |
| 2 | S. Europe | 9.69 | 4.30 | 0.95 | 0.45 |
| 3 | D6-Ditch | 11.97 b | 1.98 | 0.98 | 0.75 |
|  | R2-Stream | 0.46a | 0.014 | 0.036 | 0.006 |
|  | R3-Stream | 0.48a | 0.006 | 0.021 | 0.003 |
|  | R4-Stream | 6.65c | 0.093 | 0.39 | 0.032 |

a Peak occurs at time of spray drift event

b peak occurs at time of drainage event

c peak occurs at time of runoff event

Table G.4-11	Comparison of PECmax and 28 day time weighted average concentration at steps 1, 2 and 3 for compound D in Northern European scenarios for summer applications.

| Step | Scenario | Water |  | Sediment |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Global maximum (μg/L) | TWAC 28d (μg/L) | Global maximum (μg/kg DW) | TWAC 28d (μg/kg DW) |
| 1 | All | 33.82 | 14.92 | 3.29 | 1.49 |
| 2 | N. Europe | 3.69 | 1.64 | 0.36 | 0.16 |
| 3 | D1-Ditch | 0.55 a | 0.45 | 0.25 | 0.23 |
|  | D1-Stream | 0.47 a | 0.015 | 0.037 | 0.008 |
|  | D2-Ditch | 0.52 a | 0.27 | 0.12 | 0.11 |
|  | D2-Stream | 0.48 a | 0.24 | 0.10 | 0.091 |
|  | D3-Ditch | 0.62 a | 0.16 | 0.11 | 0.11 |
|  | D4-Pond | 0.12 b | 0.12 | 0.074 | 0.073 |
|  | D4-Stream | 0.47 a | 0.13 | 0.086 | 0.086 |
|  | D5-Pond | 0.21 b | 0.20 | 0.11 | 0.11 |
|  | D5-Stream | 0.49 a | 0.18 | 0.094 | 0.091 |
|  | R1-Pond | 0.051 c | 0.026 | 0.012 | 0.010 |
|  | R1-Stream | 0.35 a | 0.010 | 0.025 | 0.004 |

a Peak occurs at time of spray drift event

b peak occurs at time of drainage event

c peak occurs at time of runoff event

Table G.4-12	Comparison of PECmax and 28 day time weighted average concentration at steps 1, 2 and 3 for compound D in Southern European scenarios for summer applications.

| Step | Scenario | Water |  | Sediment |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Global maximum (μg/L) | TWAC 28d (μg/L) | Global maximum (μg/kg DW) | TWAC 28d (μg/kg DW) |
| 1 | All | 33.82 | 14.92 | 3.29 | 1.49 |
| 2 | S. Europe | 5.19 | 2.30 | 0.51 | 0.23 |
| 3 | D6-Ditch | 0.51 a | 0.092 | 0.069 | 0.038 |
|  | R2-Stream | 0.47 a | 0.002 | 0.012 | 0.001 |
|  | R3-Stream | 0.48 a | 0.009 | 0.023 | 0.004 |
|  | R4-Stream | 1.64 c | 0.043 | 0.12 | 0.019 |

a Peak occurs at time of spray drift event

c peak occurs at time of runoff event

Table G.4-13	Comparison of PECmax and 28 day time weighted average concentration at steps 1, 2 and 3 for compound E in Northern European scenarios for autumn applications.

| Step | Scenario | Water |  | Sediment |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Global maximum (μg/L) | TWAC 28d (μg/L) | Global maximum (μg/kg DW) | TWAC 28d (μg/kg DW) |
| 1 | All | 30.34 | 14.98 | 29.41 | 14.97 |
| 2 | N. Europe | 14.06 | 6.51 | 13.86 | 6.80 |
| 3 | D1-Ditch | 5.20 b | 3.14 | 3.48 | 3.36 |
|  | D1-Stream | 3.59 b | 1.97 | 1.99 | 1.76 |
|  | D2-Ditch | 9.76 b | 4.35 | 5.40 | 4.13 |
|  | D2-Stream | 5.97 b | 2.26 | 2.79 | 2.50 |
|  | D3-Ditch | 0.64 a | 0.028 | 0.11 | 0.034 |
|  | D4-Pond | 0.55 b | 0.50 | 0.63 | 0.62 |
|  | D4-Stream | 0.84 b | 0.42 | 0.43 | 0.40 |
|  | D5-Pond | 0.39 b | 0.32 | 0.37 | 0.36 |
|  | D5-Stream | 0.62 b | 0.15 | 0.24 | 0.20 |
|  | R1-Pond | 0.042 c | 0.029 | 0.033 | 0.032 |
|  | R1-Stream | 2.40 c | 0.037 | 0.30 | 0.041 |

a Peak occurs at time of spray drift event

b peak occurs at time of drainage event

c peak occurs at time of runoff event

Table G.4-14	Comparison of PECmax and 28 day time weighted average concentration at steps 1, 2 and 3 for compound E in Southern European scenarios for autumn applications.

| Step | Scenario | Water |  | Sediment |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Global maximum (μg/L) | TWAC 28d (μg/L) | Global maximum (μg/kg DW) | TWAC 28d (μg/kg DW) |
| 1 | All | 30.34 | 14.98 | 29.41 | 14.97 |
| 2 | S. Europe | 11.37 | 5.27 | 11.18 | 5.50 |
| 3 | D6-Ditch | 2.94 b | 0.92 | 1.24 | 0.95 |
|  | R2-Stream | 1.65c | 0.044 | 0.23 | 0.046 |
|  | R3-Stream | 5.96c | 0.18 | 1.18 | 0.43 |
|  | R4-Stream | 0.35a | 0.005 | 0.036 | 0.007 |

a Peak occurs at time of spray drift event

b peak occurs at time of drainage event

c peak occurs at time of runoff event

Table G.4-15	Comparison of PECmax and 28 day time weighted average concentration at steps 1, 2 and 3 for compound E in Northern European scenarios for spring applications.

| Step | Scenario | Water |  | Sediment |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Global maximum (μg/L) | TWAC 28d (μg/L) | Global maximum (μg/kg DW) | TWAC 28d (μg/kg DW) |
| 1 | All | 30.34 | 14.98 | 29.41 | 14.97 |
| 2 | N. Europe | 4.67 | 2.16 | 4.54 | 2.12 |
| 3 | D1-Ditch | 2.30 b | 1.72 | 2.12 | 2.04 |
|  | D1-Stream | 1.42 b | 0.86 | 1.20 | 1.03 |
|  | D2-Ditch | 7.87 b | 2.04 | 3.15 | 2.69 |
|  | D2-Stream | 4.88 b | 1.14 | 1.70 | 1.43 |
|  | D3-Ditch | 0.64 a | 0.026 | 0.11 | 0.03 |
|  | D4-Pond | 0.074 b | 0.066 | 0.089 | 0.087 |
|  | D4-Stream | 0.43a | 0.058 | 0.062 | 0.056 |
|  | D5-Pond | 0.05 a | 0.037 | 0.056 | 0.054 |
|  | D5-Stream | 0.44 a | 0.028 | 0.042 | 0.038 |
|  | R1-Pond | 0.071 a | 0.055 | 0.051 | 0.05 |
|  | R1-Stream | 2.82 c | 0.053 | 0.38 | 0.059 |

a Peak occurs at time of spray drift event

b peak occurs at time of drainage event

c peak occurs at time of runoff event

Table G.4-16	Comparison of PECmax and 28 day time weighted average concentration at steps 1, 2 and 3 for compound E in Southern European scenarios for spring applications.

| Step | Scenario | Water |  | Sediment |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Global maximum (μg/L) | TWAC 28d (μg/L) | Global maximum (μg/kg DW) | TWAC 28d (μg/kg DW) |
| 1 | All | 30.34 | 14.98 | 29.41 | 14.97 |
| 2 | S. Europe | 8.69 | 4.02 | 8.50 | 4.20 |
| 3 | D6-Ditch | 1.99 b | 0.36 | 0.61 | 0.38 |
|  | R2-Stream | 0.94c | 0.037 | 0.22 | 0.043 |
|  | R3-Stream | 1.30c | 0.024 | 0.165 | 0.026 |
|  | R4-Stream | 4.82c | 0.067 | 0.59 | 0.052 |

a Peak occurs at time of spray drift event

b peak occurs at time of drainage event

c peak occurs at time of runoff event

Table G.4-17	Comparison of PECmax and 28 day Time Weighted Average concentration at steps 1, 2 and 3 for Compound E in Northern European Scenarios for summer applications.

| Step | Scenario | Water |  | Sediment |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Global maximum (μg/L) | TWAC 28d (μg/L) | Global maximum (μg/kg DW) | TWAC 28d (μg/kg DW) |
| 1 | All | 30.34 | 14.98 | 29.41 | 14.97 |
| 2 | N. Europe | 3.33 | 1.53 | 3.23 | 1.51 |
| 3 | D1-Ditch | 1.49 b | 0.80 | 1.10 | 1.10 |
|  | D1-Stream | 1.03 b | 0.51 | 0.66 | 0.66 |
|  | D2-Ditch | 1.49 b | 0.70 | 0.82 | 0.75 |
|  | D2-Stream | 0.92 b | 0.30 | 0.37 | 0.33 |
|  | D3-Ditch | 0.64 a | 0.068 | 0.16 | 0.068 |
|  | D4-Pond | 0.16 b | 0.15 | 0.19 | 0.19 |
|  | D4-Stream | 0.46 a | 0.13 | 0.13 | 0.12 |
|  | D5-Pond | 0.13 b | 0.12 | 0.17 | 0.16 |
|  | D5-Stream | 0.49 a | 0.11 | 0.15 | 0.15 |
|  | R1-Pond | 0.18 c | 0.11 | 0.089 | 0.083 |
|  | R1-Stream | 1.32 c | 0.039 | 0.24 | 0.042 |

a Peak occurs at time of spray drift event

b peak occurs at time of drainage event

c peak occurs at time of runoff event

Table G.4-18	Comparison of PECmax and 28 day time weighted average concentration at steps 1, 2 and 3 for compound E in Southern European scenarios for summer applications.

| Step | Scenario | Water |  | Sediment |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Global maximum (μg/L) | TWAC 28d (μg/L) | Global maximum (μg/kg DW) | TWAC 28d (μg/kg DW) |
| 1 | All | 30.34 | 14.98 | 29.41 | 14.97 |
| 2 | S. Europe | 4.67 | 2.16 | 4.54 | 2.12 |
| 3 | D6-Ditch | 0.64 a | 0.10 | 0.21 | 0.11 |
|  | R2-Stream | 0.47a | 0.014 | 0.056 | 0.018 |
|  | R3-Stream | 1.76c | 0.036 | 0.22 | 0.037 |
|  | R4-Stream | 2.45c | 0.077 | 0.39 | 0.079 |

a Peak occurs at time of spray drift event

c peak occurs at time of runoff event

Table G.4-19	Comparison of PECmax and 28 day time weighted average concentration at steps 1, 2 and 3 for compound F in Northern European scenarios for autumn applications.

| Step | Scenario | Water |  | Sediment |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Global maximum (μg/L) | TWAC 28d (μg/L) | Global maximum (μg/kg DW) | TWAC 28d (μg/kg DW) |
| 1 | All | 15.21 | 9.76 | 142.86 | 97.47 |
| 2 | N. Europe | 6.90 | 3.89 | 67.87 | 40.67 |
| 3 | D1-Ditch | 0.64 a | 0.34 | 1.56 | 1.28 |
|  | D1-Stream | 0.47 a | 0.21 | 0.95 | 0.87 |
|  | D2-Ditch | 0.67 a | 0.16 | 0.94 | 0.66 |
|  | D2-Stream | 0.48 a | 0.10 | 0.61 | 0.37 |
|  | D3-Ditch | 0.63 a | 0.021 | 0.25 | 0.071 |
|  | D4-Pond | 0.051 b | 0.045 | 0.18 | 0.17 |
|  | D4-Stream | 0.46 a | 0.039 | 0.13 | 0.11 |
|  | D5-Pond | 0.028 b | 0.021 | 0.073 | 0.071 |
|  | D5-Stream | 0.49 a | 0.008 | 0.062 | 0.025 |
|  | R1-Pond | 0.028 c | 0.02 | 0.085 | 0.083 |
|  | R1-Stream | 0.59 c | 0.012 | 0.17 | 0.049 |

a Peak occurs at time of spray drift event

b peak occurs at time of drainage event

c peak occurs at time of runoff event

Table G.4-20	Comparison of PECmax and 28 day time weighted average concentration at steps 1, 2 and 3 for compound F in Southern European scenarios for autumn applications.

| Step | Scenario | Water |  | Sediment |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Global maximum (μg/L) | TWAC 28d (μg/L) | Global maximum (μg/kg DW) | TWAC 28d (μg/kg DW) |
| 1 | All | 15.21 | 9.76 | 142.86 | 97.47 |
| 2 | S. Europe | 5.60 | 3.15 | 54.85 | 32.92 |
| 3 | D6-Ditch | 0.95 b | 0.11 | 0.43 | 0.33 |
|  | R2-Stream | 0.46a | 0.011 | 0.20 | 0.11 |
|  | R3-Stream | 1.37c | 0.076 | 7.05 | 4.99 |
|  | R4-Stream | 0.41c | 0.012 | 0.16 | 0.046 |

a Peak occurs at time of spray drift event

b peak occurs at time of drainage event

c peak occurs at time of runoff event

Table G.4-21	Comparison of PECmax and 28 day time weighted average concentration at steps 1, 2 and 3 for compound F in Northern European scenarios for spring applications.

| Step | Scenario | Water |  | Sediment |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Global maximum (μg/L) | TWAC 28d (μg/L) | Global maximum (μg/kg DW) | TWAC 28d (μg/kg DW) |
| 1 | All | 15.21 | 9.76 | 142.86 | 97.47 |
| 2 | N. Europe | 2.34 | 1.30 | 22.29 | 13.55 |
| 3 | D1-Ditch | 0.72 a | 0.11 | 0.59 | 0.44 |
|  | D1-Stream | 0.44 a | 0.051 | 0.23 | 0.22 |
|  | D2-Ditch | 0.66 a | 0.065 | 0.48 | 0.26 |
|  | D2-Stream | 0.43 a | 0.020 | 0.093 | 0.087 |
|  | D3-Ditch | 0.63 a | 0.023 | 0.26 | 0.074 |
|  | D4-Pond | 0.022 a | 0.015 | 0.046 | 0.045 |
|  | D4-Stream | 0.42 a | 0.009 | 0.027 | 0.024 |
|  | D5-Pond | 0.023 a | 0.014 | 0.044 | 0.042 |
|  | D5-Stream | 0.42 a | 0.001 | 0.009 | 0.004 |
|  | R1-Pond | 0.037 a | 0.025 | 0.072 | 0.07 |
|  | R1-Stream | 0.87 c | 0.019 | 0.29 | 0.097 |

a Peak occurs at time of spray drift event

c peak occurs at time of runoff event

Table G.4-22	Comparison of PECmax and 28 day time weighted average concentration at steps 1, 2 and 3 for compound F in Southern European scenarios for spring applications.

| Step | Scenario | Water |  | Sediment |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Global maximum (μg/L) | TWAC 28d (μg/L) | Global maximum (μg/kg DW) | TWAC 28d (μg/kg DW) |
| 1 | All | 15.21 | 9.76 | 142.86 | 97.47 |
| 2 | S. Europe | 4.30 | 2.41 | 41.82 | 25.17 |
| 3 | D6-Ditch | 0.61 a | 0.004 | 0.054 | 0.012 |
|  | R2-Stream | 0.46a | 0.013 | 0.81 | 0.47 |
|  | R3-Stream | 0.76c | 0.015 | 0.21 | 0.058 |
|  | R4-Stream | 1.21c | 0.017 | 0.29 | 0.041 |

a Peak occurs at time of spray drift event

c peak occurs at time of runoff event

Table G.4-23	Comparison of PECmax and 28 day time weighted average concentration at steps 1, 2 and 3 for compound F in Northern European scenarios for summer applications.

| Step | Scenario | Water |  | Sediment |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Global maximum (μg/L) | TWAC 28d (μg/L) | Global maximum (μg/kg DW) | TWAC 28d (μg/kg DW) |
| 1 | All | 15.21 | 9.76 | 142.86 | 97.47 |
| 2 | N. Europe | 1.69 | 0.93 | 15.90 | 9.28 |
| 3 | D1-Ditch | 0.65 a | 0.30 | 1.08 | 0.96 |
|  | D1-Stream | 0.47 a | 0.068 | 0.33 | 0.33 |
|  | D2-Ditch | 0.65 a | 0.28 | 0.82 | 0.74 |
|  | D2-Stream | 0.48 a | 0.18 | 0.57 | 0.49 |
|  | D3-Ditch | 0.64 a | 0.064 | 0.44 | 0.19 |
|  | D4-Pond | 0.024 b | 0.021 | 0.08 | 0.079 |
|  | D4-Stream | 0.46 a | 0.019 | 0.06 | 0.054 |
|  | D5-Pond | 0.022 a | 0.011 | 0.035 | 0.032 |
|  | D5-Stream | 0.49 a | 0.004 | 0.064 | 0.014 |
|  | R1-Pond | 0.11 c | 0.073 | 0.23 | 0.22 |
|  | R1-Stream | 0.72 c | 0.037 | 0.81 | 0.44 |

a Peak occurs at time of spray drift event

b peak occurs at time of drainage event

c peak occurs at time of runoff event

Table G.4-24	Comparison of PECmax and 28 day time weighted average concentration at steps 1, 2 and 3 for compound F in Southern European scenarios for summer applications.

| Step | Scenario | Water |  | Sediment |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Global maximum (μg/L) | TWAC 28d (μg/L) | Global maximum (μg/kg DW) | TWAC 28d (μg/kg DW) |
| 1 | All | 15.21 | 9.76 | 142.86 | 97.47 |
| 2 | S. Europe | 2.34 | 1.30 | 22.29 | 13.55 |
| 3 | D6-Ditch | 0.64 a | 0.099 | 0.57 | 0.30 |
|  | R2-Stream | 0.47a | 0.008 | 1.14 | 0.76 |
|  | R3-Stream | 0.48a | 0.006 | 0.081 | 0.033 |
|  | R4-Stream | 1.06c | 0.077 | 0.67 | 0.30 |

a Peak occurs at time of spray drift event

c peak occurs at time of runoff event

Table G.4-25	Comparison of PECmax and 28 day Time Weighted Average concentration at steps 1, 2 and 3 for Compound H in Northern European Scenarios for autumn applications.

| Step | Scenario | Water |  | Sediment |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Global maximum (μg/L) | TWAC 28d (μg/L) | Global maximum (μg/kg DW) | TWAC 28d (μg/kg DW) |
| 1 | All | 30.34 | 28.02 | 29.41 | 28.00 |
| 2 | N. Europe | 15.39 | 14.06 | 15.33 | 14.03 |
| 3 | D1-Ditch | 8.90 b | 6.54 | 11.36 | 11.24 |
|  | D1-Stream | 5.99b | 4.11 | 6.52 | 6.11 |
|  | D2-Ditch | 11.72b | 5.91 | 10.20 | 9.97 |
|  | D2-Stream | 7.36b | 3.38 | 6.06 | 5.91 |
|  | D3-Ditch | 2.59a | 2.05 | 6.70 | 6.70 |
|  | D4-Pond | 5.75 b | 5.64 | 13.47 | 13.43 |
|  | D4-Stream | 3.49b | 3.05 | 5.18 | 5.09 |
|  | D5-Pond | 3.98 b | 3.86 | 9.13 | 8.80 |
|  | D5-Stream | 2.45b | 1.54 | 3.12 | 3.08 |
|  | R1-Pond | 0.062 c | 0.053 | 0.076 | 0.075 |
|  | R1-Stream | 3.88 c | 0.059 | 0.48 | 0.070 |

a Peak occurs at time of spray drift event

b peak occurs at time of drainage event

c peak occurs at time of runoff event

Table G.4-26	Comparison of PECmax and 28 day time weighted average concentration at steps 1, 2 and 3 for compound H in Southern European scenarios for autumn applications.

| Step | Scenario | Water |  | Sediment |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Global maximum (μg/L) | TWAC 28d (μg/L) | Global maximum (μg/kg DW) | TWAC 28d (μg/kg DW) |
| 1 | All | 30.34 | 28.02 | 29.41 | 28.00 |
| 2 | S. Europe | 12.48 | 11.39 | 12.42 | 11.37 |
| 3 | D6-Ditch | 5.40 b | 2.47 | 4.28 | 4.20 |
|  | R2-Stream | 1.92c | 0.05 | 0.27 | 0.062 |
|  | R3-Stream | 6.15c | 0.19 | 1.22 | 0.48 |
|  | R4-Stream | 0.70c | 0.015 | 0.099 | 0.019 |

a Peak occurs at time of spray drift event

b peak occurs at time of drainage event

c peak occurs at time of runoff event

Table G.4-27	Comparison of PECmax and 28 day time weighted average concentration at steps 1, 2 and 3 for compound H in Northern European scenarios for spring applications.

| Step | Scenario | Water |  | Sediment |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Global maximum (μg/L) | TWAC 28d (μg/L) | Global maximum (μg/kg DW) | TWAC 28d (μg/kg DW) |
| 1 | All | 30.34 | 28.02 | 29.41 | 28.00 |
| 2 | N. Europe | 5.19 | 4.72 | 5.15 | 4.72 |
| 3 | D1-Ditch | 3.30b | 2.60 | 5.67 | 5.59 |
|  | D1-Stream | 2.21b | 1.63 | 3.25 | 3.18 |
|  | D2-Ditch | 9.58b | 3.92 | 7.50 | 7.43 |
|  | D2-Stream | 6.02b | 2.21 | 4.19 | 4.14 |
|  | D3-Ditch | 2.21a | 1.76 | 5.76 | 5.75 |
|  | D4-Pond | 3.76b | 3.69 | 8.82 | 8.79 |
|  | D4-Stream | 2.29b | 1.98 | 3.46 | 3.39 |
|  | D5-Pond | 2.49b | 2.43 | 5.86 | 5.64 |
|  | D5-Stream | 1.25b | 0.99 | 2.06 | 2.04 |
|  | R1-Pond | 0.090a | 0.078 | 0.11 | 0.11 |
|  | R1-Stream | 3.34c | 0.062 | 0.45 | 0.074 |

a Peak occurs at time of spray drift event

b peak occurs at time of drainage event

c peak occurs at time of runoff event

Table G.4-28	Comparison of PECmax and 28 day time weighted average concentration at steps 1, 2 and 3 for compound H in Southern European scenarios for spring applications.

| Step | Scenario | Water |  | Sediment |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Global maximum (μg/L) | TWAC 28d (μg/L) | Global maximum (μg/kg DW) | TWAC 28d (μg/kg DW) |
| 1 | All | 30.34 | 28.02 | 29.41 | 28.00 |
| 2 | S. Europe | 9.56 | 8.73 | 9.51 | 8.71 |
| 3 | D6-Ditch | 10.01b | 2.02 | 2.70 | 2.35 |
|  | R2-Stream | 1.37c | 0.058 | 0.32 | 0.072 |
|  | R3-Stream | 1.68c | 0.030 | 0.21 | 0.036 |
|  | R4-Stream | 4.97c | 0.070 | 0.61 | 0.055 |

a Peak occurs at time of spray drift event

b peak occurs at time of drainage event

c peak occurs at time of runoff event

Table G.4-29	Comparison of PECmax and 28 day time weighted average concentration at steps 1, 2 and 3 for compound H in Northern European scenarios for summer applications.

| Step | Scenario | Water |  | Sediment |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Global maximum (μg/L) | TWAC 28d (μg/L) | Global maximum (μg/kg DW) | TWAC 28d (μg/kg DW) |
| 1 | All | 30.34 | 28.02 | 29.41 | 28.00 |
| 2 | N. Europe | 3.74 | 3.39 | 3.70 | 3.38 |
| 3 | D1-Ditch | 1.03b | 0.91 | 2.44 | 2.41 |
|  | D1-Stream | 0.67a | 0.57 | 1.37 | 1.34 |
|  | D2-Ditch | 1.42b | 0.93 | 2.14 | 2.12 |
|  | D2-Stream | 0.90 b | 0.48 | 1.19 | 1.18 |
|  | D3-Ditch | 0.98a | 0.41 | 1.34 | 1.33 |
|  | D4-Pond | 1.09b | 1.07 | 2.56 | 2.55 |
|  | D4-Stream | 0.65b | 0.57 | 1.00 | 0.98 |
|  | D5-Pond | 0.89b | 0.87 | 2.19 | 2.12 |
|  | D5-Stream | 0.49a | 0.38 | 0.79 | 0.78 |
|  | R1-Pond | 0.24c | 0.20 | 0.25 | 0.24 |
|  | R1-Stream | 1.79c | 0.053 | 0.33 | 0.063 |

a Peak occurs at time of spray drift event

b peak occurs at time of drainage event

c peak occurs at time of runoff event

Table G.4-30	Comparison of PECmax and 28 day time weighted average concentration at steps 1, 2 and 3 for Compound H in Southern European scenarios for summer applications.

| Step | Scenario | Water |  | Sediment |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Global maximum (μg/L) | TWAC 28d (μg/L) | Global maximum (μg/kg DW) | TWAC 28d (μg/kg DW) |
| 1 | All | 30.34 | 28.02 | 29.41 | 28.00 |
| 2 | S. Europe | 5.19 | 4.72 | 5.15 | 4.72 |
| 3 | D6-Ditch | 0.80a | 0.41 | 0.87 | 0.85 |
|  | R2-Streamd | 0.5a | 0.1 | 0.2 | 0.1 |
|  | R3-Stream | 2.15c | 0.044 | 0.26 | 0.051 |
|  | R4-Stream | 3.57c | 0.11 | 0.56 | 0.13 |

a Peak occurs at time of spray drift event

c peak occurs at time of runoff event

d error in mass balance, therefore PEC values are estimated by comparison with other simulations

Table G.4-31	Comparison of PECmax and 28 day time weighted average concentration at steps 1, 2 and 3 for compound I in Northern European scenarios for autumn applications.

| Step | Scenario | Water |  | Sediment |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Global maximum (μg/L) | TWAC 28d (μg/L) | Global maximum (μg/kg DW) | TWAC 28d (μg/kg DW) |
| 1 | All | 15.21 | 14.06 | 142.86 | 140.43 |
| 2 | N. Europe | 7.55 | 7.02 | 74.46 | 70.17 |
| 3 | D1-Ditch | 3.42b | 3.07 | 20.40 | 20.30 |
|  | D1-Stream | 2.14b | 1.92 | 11.18 | 11.03 |
|  | D2-Ditch | 2.94b | 1.48 | 13.56 | 13.07 |
|  | D2-Stream | 0.48a | 0.10 | 0.74 | 0.44 |
|  | D3-Ditch | 0.50a | 0.018 | 0.20 | 0.063 |
|  | D4-Pond | 0.56b | 0.54 | 3.26 | 3.26 |
|  | D4-Stream | 0.84b | 0.37 | 1.28 | 1.17 |
|  | D5-Pond | 0.38b | 0.36 | 2.25 | 2.21 |
|  | D5-Stream | 0.50b | 0.12 | 0.64 | 0.54 |
|  | R1-Pond | 0.093a | 0.078 | 0.41 | 0.41 |
|  | R1-Stream | 0.98c | 0.022 | 0.30 | 0.12 |

a Peak occurs at time of spray drift event

b peak occurs at time of drainage event

c peak occurs at time of runoff event

Table G.4-32	Comparison of PECmax and 28 day time weighted average concentration at steps 1, 2 and 3 for compound I in Southern European scenarios for autumn applications.

| Step | Scenario | Water |  | Sediment |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Global maximum (μg/L) | TWAC 28d (μg/L) | Global maximum (μg/kg DW) | TWAC 28d (μg/kg DW) |
| 1 | All | 15.21 | 14.06 | 142.86 | 140.43 |
| 2 | S. Europe | 6.14 | 5.69 | 60.34 | 56.86 |
| 3 | D6-Ditch | 2.80b | 0.65 | 2.16 | 1.86 |
|  | R2-Stream | 0.46a | 0.017 | 0.45 | 0.28 |
|  | R3-Stream | 1.42c | 0.085 | 7.37 | 5.65 |
|  | R4-Stream | 1.20c | 0.036 | 0.48 | 0.14 |

a Peak occurs at time of spray drift event

b peak occurs at time of drainage event

c peak occurs at time of runoff event

Table G.4-33	Comparison of PECmax and 28 day time weighted average concentration at steps 1, 2 and 3 for compound I in Northern European scenarios for spring applications.

| Step | Scenario | Water |  | Sediment |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Global maximum (μg/L) [Date] | TWAC 28d (μg/L) | Global maximum (μg/kg DW) [Date] | TWAC 28d (μg/kg DW) |
| 1 | All | 15.21 | 14.06 | 142.86 | 140.43 |
| 2 | N. Europe | 2.60 | 2.36 | 25.04 | 23.59 |
| 3 | D1-Ditch | 1.87b | 1.69 | 11.70 | 11.60 |
|  | D1-Stream | 1.18b | 1.06 | 6.51 | 6.41 |
|  | D2-Ditch | 2.12 b | 1.09 | 10.08 | 9.77 |
|  | D2-Stream | 0.44 a | 0.03 | 0.26 | 0.25 |
|  | D3-Ditch | 0.51 a | 0.018 | 0.21 | 0.065 |
|  | D4-Pond | 0.42 b | 0.40 | 2.44 | 2.43 |
|  | D4-Stream | 0.60b | 0.27 | 0.96 | 0.88 |
|  | D5-Pond | 0.27 b | 0.25 | 1.70 | 1.68 |
|  | D5-Stream | 0.45a | 0.090 | 0.47 | 0.41 |
|  | R1-Pond | 0.08 c | 0.07 | 0.38 | 0.38 |
|  | R1-Stream | 1.05 c | 0.032 | 0.35 | 0.16 |

a Peak occurs at time of spray drift event

b peak occurs at time of drainage event

c peak occurs at time of runoff event

Table G.4-34	Comparison of PECmax and 28 day Time Weighted Average concentration at steps 1, 2 and 3 for Compound I in Southern European Scenarios for spring applications.

| Step | Scenario | Water |  | Sediment |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Global maximum (μg/L) | TWAC 28d (μg/L) | Global maximum (μg/kg DW) | TWAC 28d (μg/kg DW) |
| 1 | All | 15.21 | 14.06 | 142.86 | 140.43 |
| 2 | S. Europe | 4.72 | 4.36 | 46.22 | 43.55 |
| 3 | D6-Ditch | 1.48 b | 0.37 | 1.37 | 1.12 |
|  | R2-Stream | 0.46a | 0.022 | 1.20 | 0.83 |
|  | R3-Stream | 1.01c | 0.032 | 0.36 | 0.14 |
|  | R4-Stream | 1.26c | 0.031 | 0.32 | 0.11 |

a Peak occurs at time of spray drift event

b peak occurs at time of drainage event

c peak occurs at time of runoff event

Table G.4-35	Comparison of PECmax and 28 day time weighted average concentration at steps 1, 2 and 3 for compound I in Northern European scenarios for summer applications.

| Step | Scenario | Water |  | Sediment |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Global maximum (μg/L) | TWAC 28d (μg/L) | Global maximum (μg/kg DW) | TWAC 28d (μg/kg DW) |
| 1 | All | 15.21 | 14.06 | 142.86 | 140.43 |
| 2 | N. Europe | 1.89 | 1.70 | 17.98 | 16.94 |
| 3 | D1-Ditch | 0.85 b | 0.77 | 5.71 | 5.66 |
|  | D1-Stream | 0.54b | 0.48 | 3.02 | 2.97 |
|  | D2-Ditch | 0.63 a | 0.46 | 3.26 | 3.21 |
|  | D2-Stream | 0.48 a | 0.32 | 1.06 | 1.05 |
|  | D3-Ditch | 0.51 a | 0.035 | 0.30 | 0.12 |
|  | D4-Pond | 0.15 b | 0.15 | 0.92 | 0.91 |
|  | D4-Stream | 0.46a | 0.098 | 0.35 | 0.32 |
|  | D5-Pond | 0.11 b | 0.099 | 0.69 | 0.68 |
|  | D5-Stream | 0.49a | 0.035 | 0.18 | 0.16 |
|  | R1-Pond | 0.19 c | 0.17 | 0.75 | 0.75 |
|  | R1-Stream | 0.99 c | 0.064 | 1.14 | 0.75 |

a Peak occurs at time of spray drift event

b peak occurs at time of drainage event

c peak occurs at time of runoff event

Table G.4-36	Comparison of PECmax and 28 day time weighted average concentration at steps 1, 2 and 3 for compound I in Southern European scenarios for summer applications.

| Step | Scenario | Water |  | Sediment |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Global maximum (μg/L) | TWAC 28d (μg/L) | Global maximum (μg/kg DW) | TWAC 28d (μg/kg DW) |
| 1 | All | 15.21 | 14.06 | 142.86 | 140.43 |
| 2 | S. Europe | 2.60 | 2.36 | 25.04 | 23.59 |
| 3 | D6-Ditch | 0.61b | 0.18 | 0.71 | 0.57 |
|  | R2-Stream | 0.47a | 0.049 | 5.78 | 4.67 |
|  | R3-Stream | 0.99c | 0.050 | 0.33 | 0.18 |
|  | R4-Stream | 1.57c | 0.12 | 1.06 | 0.53 |

a Peak occurs at time of spray drift event

b peak occurs at time of drainage event

c peak occurs at time of runoff event

# Part 5

### Tabular Results of MACRO Simulations with Compounds 1 to 7.

|  |  |  | Loadings |  |  | Maximum daily values |  | Percent Lost to drains |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Scenario | Season | Test Compound | Application rate (g/ha) | Crop | First application date | flux (mg/m2) | concentration (microg/L) | 16 months | From day of appln | Max in 1 Day | Max in 1 Hour |
| D3 | Spring | 1 | 3000 | Potatoes | 21-Apr-92 | 1.29E-05 | 0.009384 | 0.00 | 0.000862 | 4.31E-06 | 1.85E-07 |
| D4 | Spring | 1 | 3000 | Potatoes | 16-May-85 | 0.003517 | 2.089809 | 0.03 | 0.027569 | 0.001172 | 4.97E-05 |
| D6 | Spring | 1 | 3000 | Potatoes | 24-Mar-86 | 0.002937 | 0.904706 | 0.006667 | 0.005815 | 0.00049 | 2.15E-05 |
| D3 | Spring | 2 | 1000 | Maize | 22-Apr-92 | 0.000245 | 0.14692 | 0.07 | 0.045714 | 0.000245 | 1.03E-05 |
| D4 | Spring | 2 | 1000 | Maize | 27-Apr-85 | 0.05036 | 9.945772 | 1.01 | 0.791199 | 0.05036 | 0.0033 |
| D5 | Spring | 2 | 1000 | Maize | 23-Apr-78 | 0.035781 | 3.269854 | 0.89 | 0.414838 | 0.035781 | 0.00291 |
| D6 | Spring | 2 | 1000 | Maize | 08-Apr-86 | 0.0576 | 3.126263 | 1.46 | 0.837597 | 0.0576 | 0.00325 |
| D1 | Spring | 3 | 1000 | Winter wheat | 24-Mar-82 | 1.1671 | 157.4299 | 3.92 | 3.819745 | 1.1671 | 0.057 |
| D2 | Spring | 3 | 1000 | Winter wheat | 03-Apr-86 | 1.61225 | 445.3869 | 7.61 | 7.57798 | 1.61225 | 0.395 |
| D3 | Spring | 3 | 1000 | Winter wheat | 16-Apr-92 | 1.01E-06 | 0.000949 | 0.00 | 0.000192 | 1.01E-06 | 4.47E-08 |
| D4 | Spring | 3 | 1000 | Winter wheat | 17-Mar-85 | 0.002912 | 2.555831 | 0.03 | 0.026639 | 0.002912 | 0.000128 |
| D5 | Spring | 3 | 1000 | Winter wheat | 16-Mar-78 | 1.29E-06 | 0.001556 | 0.00 | 1.67E-05 | 1.29E-06 | 5.41E-08 |
| D6 | Spring | 3 | 1000 | Winter wheat | 18-Feb-86 | 0.03955 | 10.90734 | 0.21 | 0.176959 | 0.03955 | 0.00179 |
| D3 | Spring | 4 | 3 x 12.5 | Pome | 19-Apr-92 | 0.00E+00 | 0.00E+00 | 0.00E+00 | 0.00E+00 | 0.00E+00 | 0.00E+00 |
| D4 | Spring | 4 | 3 x 12.5 | Pome | 15-Apr-85 | 6.59E-39 | 2.31E-36 | 0.00 | 1.76E-37 | 1.76E-37 | 1.69E-38 |
| D5 | Spring | 4 | 3 x 12.5 | Pome | 21-Apr-78 | 1.36E-14 | 1.56E-12 | 0.00 | 4.93E-13 | 3.63E-13 | 1.29E-13 |
| D6 | Spring | 5 | 5 x 75 | Vines | 31-Mar-86 | 0.050553 | 1.985691 | 0.906667 | 0.529173 | 0.134808 | 0.0104 |
| D1 | Spring | 6 | 400 | Winter wheat | 03-Apr-82 | 0.3691 | 64.24976 | 5.75 | 5.527279 | 0.92275 | 0.044 |
| D2 | Spring | 6 | 400 | Winter wheat | 03-Apr-86 | 0.71568 | 114.2475 | 9.525 | 9.413817 | 1.7892 | 0.3175 |
| D3 | Spring | 6 | 400 | Winter wheat | 16-Apr-92 | 0.000152 | 0.079866 | 0.1 | 0.062243 | 0.000379 | 1.58E-05 |
| D4 | Spring | 6 | 400 | Winter wheat | 17-Mar-85 | 0.007314 | 1.491867 | 0.475 | 0.396826 | 0.018285 | 0.001178 |
| D5 | Spring | 6 | 400 | Winter wheat | 16-Mar-78 | 0.003744 | 0.435786 | 0.225 | 0.104738 | 0.009359 | 0.000623 |
| D6 | Spring | 6 | 200 | Winter wheat | 18-Feb-86 | 0.008539 | 2.414579 | 0.75 | 0.559729 | 0.042695 | 0.00186 |
| D1 | Spring | 6 metab | 400 | Winter wheat | 03-Apr-82 | 0.06291 | 8.254269 | 3.95 | 3.747081 | 0.157275 | 0.00825 |
| D2 | Spring | 6 metab | 400 | Winter wheat | 03-Apr-86 | 0.045938 | 6.036677 | 2.7 | 2.293097 | 0.114846 | 0.017 |
| D3 | Spring | 6 metab | 400 | Winter wheat | 16-Apr-92 | 8.09E-05 | 0.042165 | 0.05 | 0.033129 | 0.000202 | 8.43E-06 |
| D4 | Spring | 6 metab | 400 | Winter wheat | 17-Mar-85 | 0.008632 | 1.607544 | 0.325 | 0.296481 | 0.02158 | 0.001435 |
| D5 | Spring | 6 metab | 400 | Winter wheat | 16-Mar-78 | 0.006713 | 0.655823 | 0.325 | 0.190632 | 0.016783 | 0.00157 |
| D6 | Spring | 6 metab | 200 | Winter wheat | 18-Feb-86 | 0.00754 | 0.305474 | 0.35 | 0.210328 | 0.0377 | 0.00274 |
| D6 | Spring | 7 | 4 x 750 | Vines | 01-Apr-86 | 0.031058 | 2.100712 | 0.100 | 0.070977 | 0.010353 | 0.00094 |

# Part 6

### Tabular Results of PRZM Simulations with Compounds 1 to 7.

|  |  |  |  |  | Loadings |  | Maximum daily values |  |  |  | Percent Lost in Runoff |  |  | Percent Lost in Erosion |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Scenario | Season | Test Compound | Application rate (g/ha) | Crop | First application date | Application rate (g/ha) | Runoff flux (mg/m2) | Runoff Concentration (microg/L) | Erosion flux (mg/m2) | Erosion concentration (microg/kg) | 12 months | From day of appln | Max in 1 Day | 12 months | From day of appln | Max in 1 Day |
| R1 | Spring | 1 | 3000 | pots | 30-Apr-84 | 3000 | 7.70E-01 | 1.81E+03 | 2.51E-04 | 4.55E-01 | 2.59E-01 | 2.04E-03 | 2.57E-01 | 8.38E-05 | 7.53E-10 | 8.38E-05 |
| R2 | Spring | 1 | 3000 | pots | 22-Feb-77 | 3000 | 2.39E-01 | 8.44E+02 | 9.69E-05 | 3.27E-01 | 1.20E-01 | 1.20E-01 | 7.98E-02 | 3.32E-05 | 3.32E-05 | 3.23E-05 |
| R3 | Spring | 1 | 3000 | pots | 25-Mar-80 | 3000 | 2.34E-01 | 2.09E+03 | 6.22E-06 | 1.18E+00 | 8.57E-02 | 8.57E-02 | 7.79E-02 | 2.41E-06 | 2.41E-06 | 2.07E-06 |
| R1 | Spring | 2 | 1000 | maize | 30-Apr-84 | 1000 | 1.69E-01 | 3.98E+02 | 4.32E-04 | 7.82E-01 | 1.96E-01 | 2.69E-02 | 1.69E-01 | 4.34E-04 | 1.77E-06 | 4.32E-04 |
| R3 | Spring | 2 | 1000 | maize | 11-May-77 | 1000 | 5.28E-02 | 7.25E+01 | 4.97E-05 | 9.58E-02 | 1.08E-01 | 5.50E-02 | 5.28E-02 | 5.80E-05 | 8.29E-06 | 4.97E-05 |
| R4 | Spring | 2 | 1000 | maize | 29-Apr-80 | 1000 | 1.36E-03 | 3.57E+01 | 1.25E-08 | 1.07E-03 | 2.21E-03 | 2.21E-03 | 1.36E-03 | 1.91E-08 | 1.91E-08 | 1.25E-08 |
| R4 | Spring | 2 | 1000 | maize | 18-Apr-84 | 1000 | 6.50E-01 | 2.25E+02 | 3.61E-04 | 7.92E-02 | 7.76E-01 | 4.29E-01 | 6.50E-01 | 3.79E-04 | 1.87E-04 | 3.61E-04 |
| R1 | Spring | 3 | 1000 | WW | 22-May-84 | 1000 | 4.42E-02 | 4.92E+01 | 3.61E-04 | #DIV/0! | 4.42E-02 | 2.00E-15 | 4.42E-02 | 3.79E-04 | 4.59E-11 | 3.61E-04 |
| R3 | Spring | 3 | 1000 | WW | 07-May-80 | 1000 | 1.23E-03 | 2.83E+00 | 5.41E-11 | 1.57E-07 | 1.23E-03 | 3.38E-16 | 1.23E-03 | 5.41E-11 | 1.32E-29 | 5.41E-11 |
| R4 | Spring | 3 | 1000 | WW | 22-Apr-84 | 1000 | 1.05E-11 | 1.53E-06 | 2.62E-22 | 1.82E-16 | 1.06E-11 | 3.99E-14 | 1.05E-11 | 2.62E-22 | 1.04E-25 | 2.62E-22 |
| R1 | Spring | 4 | 3 x 12.5 | pome | 11-Apr-84 | 37.5 | 7.69E-08 | 7.93E-05 | 2.71E-04 | 2.38E-01 | 7.64E-06 | 7.30E-06 | 2.05E-06 | 2.32E-02 | 2.27E-02 | 7.22E-03 |
| R2 | Spring | 4 | 3 x 12.5 | pome | 10-Apr-77 | 37.5 | 1.28E-07 | 2.31E-05 | 7.43E-04 | 3.82E-01 | 5.74E-06 | 5.69E-06 | 3.43E-06 | 3.44E-02 | 3.43E-02 | 1.98E-02 |
| R3 | Spring | 4 | 3 x 12.5 | pome | 29-Apr-80 | 37.5 | 8.58E-08 | 5.34E-05 | 1.41E-04 | 2.83E-01 | 2.92E-06 | 6.28E-07 | 2.29E-06 | 5.20E-03 | 1.45E-03 | 3.75E-03 |
| R4 | Spring | 4 | 3 x 12.5 | pome | 07-Apr-84 | 37.5 | 1.13E-06 | 1.48E-04 | 1.49E-03 | 2.28E-01 | 6.77E-05 | 6.77E-05 | 3.00E-05 | 8.35E-02 | 8.35E-02 | 3.97E-02 |
| R1 | Spring | 5 | 5 x 75 | vines | 29-Mar-84 | 375 | 3.78E-02 | 1.90E+01 | 6.32E-04 | 1.04E+00 | 4.70E-01 | 4.70E-01 | 1.01E-01 | 4.19E-03 | 4.19E-03 | 1.69E-03 |
| R2 | Spring | 5 | 5 x 75 | vines | 24-Mar-77 | 375 | 6.43E-02 | 7.27E+00 | 3.48E-03 | 1.44E+00 | 8.00E-01 | 7.99E-01 | 1.72E-01 | 1.81E-02 | 1.81E-02 | 9.28E-03 |
| R3 | Spring | 5 | 5 x 75 | vines | 24-Mar-80 | 375 | 4.67E-02 | 1.53E+01 | 9.98E-04 | 2.59E-01 | 3.07E-01 | 3.07E-01 | 1.25E-01 | 3.69E-03 | 3.69E-03 | 2.66E-03 |
| R4 | Spring | 5 | 5 x 75 | vines | 24-Mar-84 | 375 | 2.20E-01 | 3.08E+01 | 4.55E-04 | 2.48E-01 | 1.26E+00 | 1.26E+00 | 5.87E-01 | 3.14E-03 | 3.14E-03 | 1.21E-03 |
| R1 | Spring | 6 | 400 | WW | 22-May-84 | 400 | 5.63E-02 | 6.27E+01 | 0.00E+00 | 0.00E+00 | 1.42E-01 | 1.43E-06 | 1.41E-01 | 0.00E+00 | 0.00E+00 | 0.00E+00 |
| R3 | Spring | 6 | 200 | WW | 07-May-80 | 200 | 5.14E-03 | 1.18E+01 | 0.00E+00 | 0.00E+00 | 2.74E-02 | 3.47E-07 | 2.57E-02 | 0.00E+00 | 0.00E+00 | 0.00E+00 |
| R4 | Spring | 6 | 200 | WW | 22-Apr-84 | 200 | 1.25E-07 | 9.74E-03 | 0.00E+00 | 0.00E+00 | 1.21E-06 | 8.71E-07 | 6.23E-07 | 0.00E+00 | 0.00E+00 | 0.00E+00 |
| R1 | Spring | 6-metab | 400 | WW | 22-May-84 | 293.333 | 4.62E-06 | 5.13E-03 | 0.00E+00 | 0.00E+00 | 1.57E-05 | 3.29E-16 | 1.57E-05 | 0.00E+00 | 0.00E+00 | 0.00E+00 |
| R3 | Spring | 6-metab | 200 | WW | 07-May-80 | 146.667 | 7.52E-08 | 1.72E-04 | 0.00E+00 | 0.00E+00 | 5.13E-07 | 6.98E-17 | 5.13E-07 | 0.00E+00 | 0.00E+00 | 0.00E+00 |
| R4 | Spring | 6-metab | 200 | WW | 22-Apr-84 | 146.667 | 4.30E-16 | 6.25E-11 | 0.00E+00 | 0.00E+00 | 3.55E-15 | 6.17E-16 | 2.93E-15 | 0.00E+00 | 0.00E+00 | 0.00E+00 |
| R1 | Spring | 7 | 4 x 750 | vines | 29-Mar-84 | 3000 | 0.00E+00 | 0.00E+00 | 0.00E+00 | 0.00E+00 | 0.00E+00 | 0.00E+00 | 0.00E+00 | 0.00E+00 | 0.00E+00 | 0.00E+00 |
| R2 | Spring | 7 | 4 x 750 | vines | 24-Mar-77 | 3000 | 1.53E-17 | 1.74E-15 | 0.00E+00 | 0.00E+00 | 7.46E-18 | 2.15E-19 | 5.10E-18 | 0.00E+00 | 0.00E+00 | 0.00E+00 |
| R3 | Spring | 7 | 4 x 750 | vines | 24-Mar-80 | 3000 | 0.00E+00 | 0.00E+00 | 0.00E+00 | 0.00E+00 | 0.00E+00 | 0.00E+00 | 0.00E+00 | 0.00E+00 | 0.00E+00 | 0.00E+00 |
| R4 | Spring | 7 | 4 x 750 | vines | 01-Apr-84 | 3000 | 0.00E+00 | 0.00E+00 | 0.00E+00 | 0.00E+00 | 0.00E+00 | 0.00E+00 | 0.00E+00 | 0.00E+00 | 0.00E+00 | 0.00E+00 |

# Part 7

### Tabular Results of TOXSWA Simulations with Test Compounds 1 to 7 for run-off simulations.

Table G.7-1:  Summary of TOXSWA loadings and fluxes for run-off simulations.

|  |  |  |  |  |  | Water body |  |  |  |  | Loading |  |  | Daily Max Fluxes from run-off |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Scenario | Season | Test Compound | Crop | TOXSWA water body |  | width (m) | length (m) | Sediment BD (kg/m^3) | Upstream catchment (ha) | Ratio treated/non-treated | Applic (g/ha) | First applic date | Mean areic drift (%) | water flux (mm/d) | Date | Cmpd flux (mg/m^2/d) | Date | Cmpd concentration (ug/L) | Date | Cmpd eroded (mg/m^2/d)) | Date |
| R1 | Spring | 1 | Pot | stream |  | 1 | 100 | 800 | 100 | 0.2 | 3000 | 30-Apr-84 | 0 | 10.31 | 23-Nov-84 | 0.77 | 17-Apr-84 | 1786.1 | 17-Apr-84 | 0.00 | 17-Apr-84 |
|  |  | 1 | Pot | pond |  | 30 | 30 | 800 | 0 | 0 | 3000 | 30-Apr-84 | 0 | 10.39 | 23-Nov-84 | 0.77 | 17-Apr-84 | 1723.5 | 17-Apr-84 | 0.00 | 17-Apr-84 |
| R2 | Spring | 1 | Pot | stream |  | 1 | 100 | 800 | 100 | 0.2 | 3000 | 22-Feb-77 | 0 | 29.17 | 17-Feb-78 | 0.08 | 4-Apr-77 | 338.5 | 27-Mar-77 | 0.00 | 27-Mar-77 |
| R3 | Spring | 1 | Pot | stream |  | 1 | 100 | 800 | 100 | 0.2 | 3000 | 22-Feb-77 | 0 | 24.57 | 28-Nov-80 | 0.23 | 21-Apr-80 | 1348.4 | 09-Apr-80 | 0.00 | 9-Apr-80 |
| R1 | Spring | 2 | maize | stream |  | 1 | 100 | 800 | 100 | 0.2 | 1000 | 30-Apr-84 | 1.24 | 10.28 | 23-Nov-84 | 0.17 | 17-Apr-84 | 391.6 | 17-Apr-84 | 0.00 | 17-Apr-84 |
|  |  | 2 | maize | pond |  | 30 | 30 | 800 | 0 | 0 | 1000 | 30-Apr-84 | 0.212 | 10.28 | 23-Nov-84 | 0.17 | 17-Apr-84 | 377.8 | 17-Apr-84 | 0.00 | 17-Apr-84 |
| R2 | Spring | 2 | maize | stream |  | 1 | 100 | 800 | 100 | 0.2 | 1000 | 11-May-77 | 1.24 | 29.17 | 17-Feb-78 | 0.05 | 6-May-77 | 71.2 | 6-May-77 | 0.00 | 6-May-77 |
| R3 | Spring | 2 | maize | stream | 1 |  | 100 | 800 | 100 | 0.2 | 1000 | 18-Apr-84 | 1.24 | 41.68 | 02-Dec-84 | 0.65 | 19-Apr-84 | 224.4 | 19-Apr-84 | 0.00 | 19-Apr-84 |
| R4 | Spring | 2 | maize | stream |  | 1 | 100 | 800 | 100 | 0.2 | 1000 | 18-Apr-84 | 1.24 | 41.68 | 2-Dec-84 | 0.65 | 19-Apr-84 | 224.4 | 19-Apr-84 | 0.00 | 19-Apr-84 |
| R1 | Spring | 3 | WW | stream |  | 1 | 100 | 800 | 100 | 0.2 | 1000 | 22-May-84 | 1.43 | 5.06 | 10-Sep-84 | 0.04 | 2-Apr-84 | 48.8 | 02-Apr-84 | 0.00 | 2-Apr-84 |
|  |  | 3 | WW | pond |  | 30 | 30 | 800 | 0 | 0 | 1000 | 22-May-84 | 0.22 | 5.06 | 10-Sep-84 | 0.04 | 2-Apr-84 | 47.7 | 02-Apr-84 | 0.00 | 2-Apr-84 |
| R3 | Spring | 3 | WW | stream |  | 1 | 100 | 800 | 100 | 0.2 | 1000 | 7-May-80 | 1.43 | 24.56 | 28-Nov-80 | 0.00 | 23-Mar-80 | 2.66 | 23-Mar-80 | 0.00 | 23-Mar-80 |
| R4 | Spring | 3 | WW | stream |  | 1 | 100 | 800 | 100 | 0.2 | 1000 | 22-Apr-84 | 1.43 | 39.5 | 6-Nov-84 | 0.00 | 1-Apr-84 | 0.00 | 01-Apr-84 | 0.00 | 1-Apr-84 |
| R1 | Spring | 4 | pome | stream |  | 1 | 100 | 800 | 100 | 0.2 | 12.5 | 11-Apr-84 | 20.71 | 6.63 | 23-Nov-84 | 0.00 | 31-May-84 | 0.00 | 21-May-84 | 0.00 | 31-May-84 |
|  |  | 4 | pome | pond |  | 30 | 30 | 800 | 0.45 | - | 12.5 | 11-Apr-84 | 3.864 | 6.63 | 23-Nov-84 | 0.00 | 31-May-84 | 0.00 | 21-May-84 | 0.00 | 31-May-84 |
| R2 | Spring | 4 | pome | stream |  | 1 | 100 | 800 | 100 | 0.2 | 12.5 | 10-Apr-77 | 20.71 | 22.33 | 17-Feb-78 | 0.00 | 10-Jun-77 | 0.00 | 10-Jun-77 | 0.00 | 10-Jun-77 |
| R3 | Spring | 4 | pome | stream |  | 1 | 100 | 800 | 100 | 0.2 | 12.5 | 29-Apr-80 | 20.71 | 18.05 | 28-Nov-80 | 0.00 | 21-Apr-80 | 0.00 | 24-May-80 | 0.00 | 21-Apr-80 |
| R4 | Spring | 4 | pome | stream |  | 1 | 100 | 800 | 100 | 0.2 | 12.5 | 7-Apr-84 | 20.71 | 33.49 | 2-Dec-84 | 0.00 | 20-May-84 | 0.00 | 09-May-84 | 0.00 | 20-May-84 |
| R1 | Spring | 5 | Vines | stream |  | 1 | 100 | 800 | 0 | 0 | 75 | 29-Mar-84 | 1.213 | 10.32 | 23-Nov-84 | 0.04 | 21-May-84 | 15.4 | 21-May-94 | 0.00 | 21-May-84 |
|  |  | 5 | Vines | pond |  | 30 | 30 | 800 | 0 | 0 | 75 | 29-Mar-84 | 0.149 | 10.4 | 23-Nov-84 | 0.04 | 21-May-84 | 14.8 | 21-May-84 | 0.00 | 21-May-84 |
| R2 | Spring | 5 | Vines | stream |  | 1 | 100 | 800 | 0 | 0 | 75 | 24-Mar-77 | 1.213 | 29.17 | 17-Feb-78 | 0.06 | 10-Jun-77 | 7.16 | 14-May-77 | 0.00 | 6-Apr-77 |
| R3 | Spring | 5 | Vines | stream |  | 1 | 100 | 800 | 0 | 0 | 75 | 24-Mar-80 | 1.213 | 24.53 | 28-Nov-80 | 0.05 | 21-Apr-80 | 14.4 | 24-May-80 | 0.00 | 21-Apr-80 |
| R4 | Spring | 5 | Vines | stream |  | 1 | 100 | 800 | 0 | 0 | 75 | 24-Mar-84 | 1.213 | 41.69 | 2-Dec-84 | 0.22 | 20-May-84 | 30.2 | 16-May-84 | 0.00 | 20-May-84 |
| R1 | Spring | 6 | WW | stream |  | 1 | 100 | 800 | 100 | 0.2 | 400 | 22-May-84 | 1.43 | 5.06 | 10-Sep-84 | 0.06 | 2-Apr-84 | 62.1 | 02-Apr-84 | 0.00 | 1-Mar-84 |
|  |  | 6 | WW | pond |  | 30 | 30 | 800 | 0 | 0 | 400 | 22-May-84 | 0.219 | 5.06 | 10-Sep-84 | 0.06 | 2-Apr-84 | 60.8 | 02-Apr-84 | 0.00 | 1-Mar-84 |
| R3 | Spring | 6 | WW | stream |  | 1 | 100 | 800 | 100 | 0.2 | 200 | 7-May-80 | 1.43 | 24.56 | 28-Nov-80 | 0.01 | 23-Mar-80 | 11.12 | 23-Mar-80 | 0.00 | 1-Mar-80 |
| R4 | Spring | 6 | WW | stream |  | 1 | 100 | 800 | 100 | 0.2 | 200 | 22-Apr-84 | 1.43 | 39.5 | 6-Nov-84 | 0.00 | 16-May-84 | 0.00 | 01-Apr-84 | 0.00 | 1-Mar-84 |
| R1 | Spring | 6 metab | WW | stream |  | 1 | 100 | 800 | 100 | 0.2 | 0 | 22-May-84 | 0.00 | 5.06 | 10-Sep-84 | 0.00 | 2-Apr-84 | 0.01 | 01-Apr-84 | 0.00 | 1-Mar-84 |
|  |  | 6 metab | WW | pond |  | 30 | 30 | 800 | 0 | 0 | 0 | 22-May-84 | 0.00 | 5.06 | 10-Sep-84 | 0.00 | 2-Apr-84 | 0.00 | 01-Apr-84 | 0.00 | 1-Mar-84 |
| R3 | Spring | 6 metab | WW | stream |  | 1 | 100 | 800 | 100 | 0.2 | 0 | 7-May-80 | 0.00 | 24.56 | 28-Nov-80 | 0.00 | 23-Mar-80 | 0.00 | 23-Mar-80 | 0.00 | 1-Mar-80 |
| R4 | Spring | 6 metab | WW | stream |  | 1 | 100 | 800 | 100 | 0.2 | 0 | 22-Apr-84 | 0.00 | 39.5 | 6-Nov-84 | 0.00 | 1-Apr-84 | 0.00 | 01-Apr-84 | 0.00 | 1-Mar-84 |
| R1 | Spring | 7 | Vines | stream |  | 1 | 100 | 800 | 0 | 0 | 750 | 29-Mar-84 | 1.257 | 10.32 | 23-Nov-84 | 0.00 | 1-Mar-84 | 0.00 | 01-Mar-84 | 0.00 | 1-Mar-84 |
|  |  | 7 | Vines | pond |  | 30 | 30 | 800 | 0 | 0 | 750 | 29-Mar-84 | 0.153 | 10.4 | 23-Nov-84 | 0.00 | 1-Mar-84 | 0.00 | 01-Mar-84 | 0.00 | 1-Mar-84 |
| R2 | Spring | 7 | Vines | stream | 1 |  | 100 | 800 | 0 | 0 | 750 | 24-Mar-77 | 1.257 | 29.2 | 17-Feb-78 | 0.00 | 12-Mar-77 | 0.00 | 12-Mar-77 | 0.00 | 01-Mar-77 |
| R3 | Spring | 7 | Vines | stream |  | 1 | 100 | 800 | 0 | 0 | 750 | 24-Mar-80 | 1.257 | 24.53 | 28-Nov-80 | 0.00 | 1-Mar-80 | 0.00 | 01-Mar-80 | 0.00 | 1-Mar-80 |
| R4 | Spring | 7 | Vines | stream |  | 1 | 100 | 800 | 0 | 0 | 750 | 1-Apr-84 | 1.257 | 41.69 | 2-Dec-84 | 0.00 | 1-Mar-84 | 0.00 | 01-Mar-84 | 0.00 | 1-Mar-84 |

Table G.7-2:  Summary of TOXSWA maximum and time weighted average concentrations for run-off simulations.

|  |  |  |  |  | Concentrations in water (g/L) |  |  |  |  |  |  | Concentrations in sediment (g/kg) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Scenario | Season | Test Compound | Crop | TOXSWA water body | Global max | Date | TWAECsw14 d | Date | TWAECsw21 d | Date | TWAECsw28 d | Global max | Date |
| R1 | Spring | 1 | Potatoes | stream | 374.045 | 16/Apr/84 | 8.13 | 29/Apr/84 | 5.42 | 6/May/84 | 4.163 | 18.754 | 16/Apr/84 |
|  |  | 1 | Potatoes | pond | 3.811 | 16/Apr/84 | 2.692 | 30/Apr/84 | 2.287 | 7/May/84 | 1.954 | 0.858 | 25/Apr/85 |
| R2 | Spring | 1 | Potatoes | stream | 53.63 | 26/Mar/77 | 1.756 | 8/Apr/77 | 1.171 | 15/Apr/77 | 0.878 | 3.417 | 26/Mar/77 |
| R3 | Spring | 1 | Potatoes | stream | 152.655 | 8/Apr/80 | 4.601 | 22/Apr/80 | 3.068 | 29/Apr/80 | 2.301 | 8.199 | 8/Apr/80 |
| R1 | Spring | 2 | maize | stream | 82.02 | 16/Apr/84 | 1.79 | 30/Apr/84 | 1.223 | 7/May/84 | 1.129 | 9.069 | 16/Apr/84 |
|  |  | 2 | maize | pond | 0.84 | 30/Apr/84 | 0.75 | 14/May/84 | 0.741 | 7/May/84 | 0.737 | 1.199 | 24/May/84 |
| R2 | Spring | 2 | maize | stream | 13.775 | 5/May/77 | 0.761 | 18/May/77 | 0.507 | 25/May/77 | 0.38 | 2.322 | 5/May/77 |
| R3 | Spring | 2 | maize | stream | * | * | * | * | * | * | * | * | * |
| R4 | Spring | 2 | maize | stream | 48.355 | 18/Apr/84 | 2.91 | 2/May/84 | 1.94 | 8/May/84 | 1.482 | 9.522 | 18/Apr/84 |
| R1 | Spring | 3 | Winter wheat | stream | 10.048 | 1/Apr/84 | 0.357 | 14/Apr/84 | 0.238 | 14/Apr/84 | 0.179 | 0.488 | 1/Apr/84 |
|  |  | 3 | Winter wheat | pond | 0.22 | 22/May/84 | 0.077 | 15/Apr/84 | 0.054 | 22/Apr/84 | 0.041 | 0.019 | 3/Apr/84 |
| R3 | Spring | 3 | Winter wheat | stream | 4.893 | 7/May/80 | 0.098 | 15/May/80 | 0.065 | 15/May/80 | 0.049 | 0.16 | 7/May/80 |
| R4 | Spring | 3 | Winter wheat | stream | 3.494 | 22/Apr/84 | 0.051 | 30/Apr/84 | 0.034 | 30/Apr/84 | 0.026 | 0.103 | 22/Apr/84 |
| R1 | Spring | 4 | pome | stream | 0.256 | 25/Apr/84 | 0.003 | 25/Apr/84 | 0.004 | 02/May/84 | 0.003 | 1.075 | 25/Apr/84 |
|  |  | 4 | pome | pond | 0.018 | 25/Apr/84 | 0.002 | 03/May/84 | 0.003 | 02/May/84 | 0.002 | 0.561 | 15/May/84 |
| R2 | Spring | 4 | pome | stream | 0.349 | 25/May/77 | 0.003 | 25/May/77 | 0.003 | 01/Jun/77 | 0.002 | 0.956 | 25/May/77 |
| R3 | Spring | 4 | pome | stream | 0.369 | 04/Jun/80 | 0.008 | 18/Jun/80 | 0.009 | 09/Jun/80 | 0.007 | 2.524 | 04/Jun/80 |
| R4 | Spring | 4 | pome | stream | 0.257 | 21/Apr/84 | 0.003 | 21/Apr/84 | 0.004 | 28/Apr/84 | 0.003 | 1.452 | 24/May/84 |
| R1 | Spring | 5 | Vines | stream | 3.364 | 20/May/84 | 0.301 | 3/Jun/84 | 0.238 | 4/Jun/84 | 0.187 | 1.554 | 30/May/84 |
|  |  | 5 | Vines | pond | 0.292 | 30/May/84 | 0.197 | 3/Jun/84 | 0.185 | 10/Jun/84 | 0.161 | 0.651 | 5/Jun/84 |
| R2 | Spring | 5 | Vines | stream | 1.406 | 13/May/77 | 0.099 | 17/May/77 | 0.071 | 15/Oct/77 | 0.071 | 2.423 | 10/Jun/77 |
| R3 | Spring | 5 | Vines | stream | 3.214 | 23/May/80 | 0.295 | 27/May/80 | 0.249 | 29/May/80 | 0.187 | 1.628 | 23/May/80 |
| R4 | Spring | 5 | Vines | stream | 6.276 | 15/May/84 | 0.885 | 29/May/84 | 0.59 | 5/Jun/84 | 0.444 | 3.614 | 19/May/84 |
| R1 | Spring | 6 | Winter wheat | stream | 12.848 | 1/Apr/84 | 0.468 | 14/Apr/84 | 0.312 | 21/Apr/84 | 0.234 | 1.465 | 1/Apr/84 |
|  |  | 6 | Winter wheat | pond | 0.278 | 1/Apr/84 | 0.243 | 15/Apr/84 | 0.228 | 22/Apr/84 | 0.215 | 0.184 | 30/May/84 |
| R3 | Spring | 6 | Winter wheat | stream | 2.433 | 22/Mar/80 | 0.091 | 5/Apr/80 | 0.061 | 12/Apr/80 | 0.045 | 0.271 | 22/Mar/80 |
| R4 | Spring | 6 | Winter wheat | stream | 0.699 | 22/Apr/84 | 0.011 | 6/May/84 | 0.007 | 13/May/84 | 0.005 | 0.047 | 22/Apr/84 |
| R1 | Spring | 6 metab | Winter wheat | stream | 0.001 | 1/Apr/84 | 0.00 | 15/Apr/84 | 0.00 | 21/Apr/84 | 0.00 | 0.00 | 1/Apr/84 |
|  |  | 6 metab | Winter wheat | pond | 0.00 | 1/Apr/84 | 0.00 | 15/Apr/84 | 0.00 | 22/Apr/84 | 0.00 | 0.00 | 2/May/84 |
| R3 | Spring | 6 metab | Winter wheat | stream | 0.00 | 22/Mar/80 | 0.00 | 5/Apr/80 | 0.00 | 12/Apr/80 | 0.00 | 0.00 | 22/Mar/80 |
| R4 | Spring | 6 metab | Winter wheat | stream | 0.00 | 31/Mar/84 | 0.00 | 14/Apr/84 | 0.00 | 20/Apr/84 | 0.00 | 0.00 | 31/Mar/84 |
| R1 | Spring | 7 | Vines | stream | 2.303 | 29/Mar/84 | 0.034 | 12/Apr/84 | 0.045 | 19/Apr/84 | 0.033 | 0.297 | 26/Apr/84 |
|  |  | 7 | Vines | pond | 0.142 | 26/Apr/84 | 0.068 | 1/May/84 | 0.076 | 3/May/84 | 0.066 | 0.19 | 30/Apr/84 |
| R2 | Spring | 7 | Vines | stream |  |  |  |  |  |  |  |  |  |
| R3 | Spring | 7 | Vines | stream | 3.233 | 21/Apr/80 | 0.073 | 21/Apr/80 | 0.096 | 28/Apr/80 | 0.072 | 0.584 | 21/Apr/80 |
| R4 | Spring | 7 | Vines | stream | 2.302 | 29/Apr/84 | 0.034 | 29/Apr/84 | 0.045 | 6/May/84 | 0.034 | 0.302 | 29/Apr/84 |

*  This simulation gave a faulty model run.

# Part 8

### Tabular Results of TOXSWA Simulations with Test Compounds 1 to 7 for drainage simulations.

Table G.8-1:  Summary of TOXSWA loadings and fluxes for drainage simulations.

|  |  |  |  |  | Water body |  |  |  |  | Loadings |  |  | Daily Max Fluxes from drains |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Scenario | Season | Compound | Crop | TOXSWA water body | width (m) | length (m) | Sediment BD (kg-m^3) | Upstream catchment (ha) | Ratio treated-non-treated | Applic (g-ha) | First applic date | Mean areic drift (%) | Water flux (mm-d) | Date | Cmpd flux (mg-m^2-d) | Date | Cmpd concentration (g-L) | Date |
| D3 | Spring | 1 | Potatoes | ditch | 1 | 100 | 800 | 2 | 0 | 3000 | 1992-Apr-21 | 0.00 | 2.36 | 4-Jan-92 | 0.00 | 23-Nov-92 | 0.44 | 29-Nov-92 |
| D4 | Spring | 1 | Potatoes | pond | 30 | 30 | 800 | 0 | 0 | 3000 | 1985-May-16 | 0.00 | 5.47 | 6-Dec-85 | 0.01 | 24-Jul-85 | 3.08 | 17-Jun-85 |
|  |  | 1 |  | stream | 1 | 100 | 800 | 100 | 0.2 | 3000 | 1985-May-16 | 0.00 | 5.47 | 6-Dec-85 | 0.01 | 24-Jul-85 | 3.08 | 17-Jun-85 |
| D6 | Spring | 1 | Potatoes | ditch | 1 | 100 | 800 | 2 | 0 | 3000 | 1986-Mar-24 | 0.00 | 22.4 | 20-Jan-87 | 0.00 | 30-Mar-86 | 0.90 | 30-Mar-86 |
|  |  | 1 |  | pond | 30 | 30 | 800 | 0 | 0 | 3000 | 1986-Mar-24 | 0.00 | 22.4 | 20-Jan-87 | 0.00 | 30-Mar-86 | 0.90 | 30-Mar-86 |
| D3 | Spring | 2 | maize | ditch | 1 | 100 | 800 | 2 | 0 | 1000 | 1992-Apr-22 | 1.59 | 2.08 | 8-Jan-92 | 0.00 | 8-Jan-92 | 0.15 | 28-Mar-93 |
| D4 | Spring | 2 | maize | pond | 30 | 30 | 800 | 0 | 0 | 1000 | 1985-Apr-27 | 0.212 | 5.47 | 6-Dec-85 | 0.05 | 8-Dec-85 | 9.85 | 10-Dec-93 |
|  |  | 2 | maize | stream | 1 | 100 | 800 | 100 | 0.2 | 1000 | 1985-Apr-27 | 1.24 | 5.47 | 6-Dec-85 | 0.05 | 8-Dec-85 | 9.85 | 10-Dec-93 |
| D5 | Spring | 2 | maize | pond | 30 | 30 | 800 | 0 | 0 | 1000 | 1978-Apr-23 | 0.212 | 10.95 | 25-Jan-78 | 0.03 | 25-Jan-78 | 3.20 | 12-Jan-78 |
|  |  | 2 | maize | stream | 1 | 100 | 800 | 100 | 0.2 | 1000 | 1978-Apr-23 | 1.24 | 10.95 | 25-Jan-78 | 0.03 | 25-Jan-78 | 3.20 | 12-Jan-78 |
| D6 | Spring | 2 | maize | ditch | 1 | 100 | 800 | 2 | 0 | 1000 | 1986-Apr-08 | 1.59 | 22.41 | 20-Jan-87 | 0.05 | 31-Oct-86 | 2.99 | 09-May-86 |
|  |  | 2 | maize | pond | 30 | 30 | 800 | 0 | 0 | 1000 | 1986-Apr-08 | 0.212 | 22.41 | 20-Jan-87 | 0.05 | 31-Oct-86 | 2.99 | 09-May-86 |
| D1 | Spring | 3 | Winter wheat | ditch | 1 | 100 | 800 | 2 | 0 | 1000 | 1982-Mar-24 | 1.93 | 8.94 | 22-Dec-82 | 1.05 | 10-Apr-82 | 156.31 | 10-Apr-82 |
| D1 | Spring | 3 | Winter wheat | stream | 1 | 100 | 800 | 100 | 0.2 | 1000 | 1982-Mar-24 | 1.43 | 8.94 | 22-Dec-82 | 1.05 | 10-Apr-82 | 156.31 | 10-Apr-82 |
| D2 | Spring | 3 | Winter wheat | ditch | 1 | 100 | 800 | 2 | 0 | 1000 | 1986-Apr-03 | 1.93 | 11.81 | 5-Apr-87 | 1.57 | 15-Apr-86 | 441.29 | 08-Apr-86 |
|  |  |  |  | stream | 1 | 100 | 800 | 100 | 0.2 | 1000 | 1986-Apr-03 | 1.43 | 11.81 | 5-Apr-87 | 1.57 | 15-Apr-86 | 441.29 | 08-Apr-86 |
| D3 | Spring | 3 | Winter wheat | ditch | 1 | 100 | 800 | 2 | 0 | 1000 | 1992-Apr-16 | 1.93 | 2.19 | 8-Jan-92 | 0.00 | 6-Nov-92 | 0.00 | 11-Sep-92 |
| D4 | Spring | 3 | Winter wheat | pond | 30 | 30 | 800 | 0 | 0 | 1000 | 1985-Mar-17 | 0.22 | 5.48 | 8-Dec-85 | 0.00 | 4-Apr-85 | 2.53 | 05-Apr-85 |
|  |  | 3 |  | stream | 1 | 100 | 800 | 100 | 0.2 | 1000 | 1985-Mar-17 | 1.43 | 5.48 | 8-Dec-85 | 0.00 | 4-Apr-85 | 2.53 | 05-Apr-85 |
| D5 | Spring | 3 | Winter wheat | pond | 30 | 30 | 800 | 0 | 0 | 1000 | 1978-Mar-16 | 0.22 | 10.94 | 25-Jan-78 | 0.00 | 7-Apr-78 | 0.00 | 08-Apr-78 |
|  |  | 3 |  | stream | 1 | 100 | 800 | 100 | 0.2 | 1000 | 1978-Mar-16 | 1.43 | 10.94 | 25-Jan-78 | 0.00 | 7-Apr-78 | 0.00 | 08-Apr-78 |
| D6 | Spring | 3 | Winter wheat | ditch | 1 | 100 | 800 | 2 | 0 | 1000 | 1986-Feb-18 | 1.93 | 21.65 | 20-Jan-87 | 0.04 | 25-Feb-86 | 10.86 | 25-Feb-85 |
|  |  | 3 |  | pond | 30 | 30 | 800 | 0 | 0 | 1000 | 1986-Feb-18 | 0.22 | 21.65 | 20-Jan-87 | 0.04 | 25-Feb-86 | 10.86 | 25-Feb-85 |
| D3 | Spring | 4 | apples | ditch | 1 | 100 | 800 | 2.0 | 0 | 12.5 | 1992-Apr-19 | 19.02 | 1.03 | 16-Jan-92 | 0.00 | 01-Jan-92 | 0.00 | 01-Jan-92 |
| D4 | Spring | 4 | apples | pond | 30 | 30 | 800 | 0.45 | 0 | 12.5 | 1985-Apr-15 | 3.864 | 2.81 | 23-Jan-86 | 0.00 | 01-Jan-85 | 0.00 | 01-Jan-85 |
|  |  | 4 | apples | stream | 1 | 100 | 800 | 100 | 0.2 | 12.5 | 15-Apr-85 | 20.71 | 2.81 | 23-Jan-86 | 0.00 | 01-Jan-85 | 0.00 | 01-Jan-85 |
| D5 | Spring | 4 | apples | pond | 30 | 30 | 800 | 0.45 | 0 | 12.5 | 21-Apr-85 | 3.864 | 10.91 | 25-Jan-78 | 0.00 | 25-Jan-78 | 0.00 | 25-Jan-78 |
|  |  | 4 | apples | stream | 1 | 100 | 800 | 100 | 0.2 | 12.5 | 21-Apr-78 | 20.71 | 10.91 | 25-Jan-78 | 0.00 | 25-Jan-78 | 0.00 | 25-Jan-78 |
| D6 | Spring | 5 | Vines | ditch | 1 | 100 | 800 | 0 | 0 | 75 | 1986-Mar-31 | 1.485 | 22.41 | 20-Jan-87 | 0.04 | 20-Jan-87 | 1.76 | 11-Feb-87 |
|  |  | 5 |  | pond | 30 | 30 | 800 | 0 | 0 | 75 | 1986-Mar-31 | 0.149 | 22.41 | 20-Jan-87 | 0.04 | 20-Jan-87 | 1.76 | 11-Feb-87 |
| D1 | Spring | 6 | Winter wheat | ditch | 1 | 100 | 800 | 2 | 0 | 400 | 1982-Apr-03 | 1.928 | 8.94 | 22-Dec-82 | 0.36 | 10-Apr-82 | 63.97 | 11-Apr-82 |
|  |  | 6 |  | stream | 1 | 100 | 800 | 100 | 0.2 | 400 | 1982-Apr-03 | 1.43 | 8.94 | 22-Dec-82 | 0.36 | 10-Apr-82 | 63.97 | 11-Apr-82 |
| D2 | Spring | 6 | Winter wheat | ditch | 1 | 100 | 800 | 2 | 0 | 400 | 1986-Apr-03 | 1.928 | 11.8 | 5-Apr-87 | 0.71 | 15-May-86 | 107.49 | 20-Apr-86 |
|  |  | 6 |  | stream | 1 | 100 | 800 | 100 | 0.2 | 400 | 1986-Apr-03 | 1.43 | 11.8 | 5-Apr-87 | 0.71 | 15-May-86 | 107.49 | 20-Apr-86 |
| D3 | Spring | 6 | Winter wheat | ditch | 1 | 100 | 800 | 2 | 0 | 400 | 1992-Apr-16 | 1.928 | 2.19 | 8-Jan-92 | 0.00 | 8-Jan-92 | 0.08 | 12-May-92 |
| D4 | Spring | 6 | Winter wheat | pond | 30 | 30 | 800 | 0 | 0 | 400 | 1985-Mar-17 | 0.219 | 5.48 | 8-Dec-85 | 0.01 | 10-Dec-85 | 1.48 | 21-Dec-85 |
|  |  | 6 |  | stream | 1 | 100 | 800 | 100 | 0.2 | 400 | 1985-Mar-17 | 1.43 | 5.48 | 8-Dec-85 | 0.01 | 10-Dec-85 | 1.48 | 21-Dec-85 |
| D5 | Spring | 6 | Winter wheat | pond | 30 | 30 | 800 | 0 | 0 | 400 | 1978-Mar-16 | 0.219 | 10.94 | 25-Jan-78 | 0.00 | 25-Jan-78 | 0.44 | 04-Jan-78 |
|  |  | 6 |  | stream | 1 | 100 | 800 | 100 | 0.2 | 400 | 1978-Mar-16 | 1.43 | 10.94 | 25-Jan-78 | 0.00 | 25-Jan-78 | 0.44 | 04-Jan-78 |
| D6 | Spring | 6 | Winter wheat | ditch | 1 | 100 | 800 | 2 | 0 | 200 | 1986-Feb-18 | 1.93 | 21.65 | 20-Jan-87 | 0.01 | 25-Feb-86 | 2.26 | 26-Feb-86 |
|  |  | 6 |  | pond | 30 | 30 | 800 | 0 | 0 | 200 | 1986-Feb-18 | 0.219 | 21.65 | 02-Jan-87 | 0.01 | 25-Feb-86 | 2.26 | 26-Feb-86 |
| D1 | Spring | 6 metab | Winter wheat | ditch | 1 | 100 | 800 | 2 | 0 | 400 | 1982-Apr-03 | 0.00 | 8.94 | 22-Dec-82 | 0.05 | 25-Nov-82 | 8.25 | 27-Oct-82 |
|  |  | 6 metab |  | stream | 1 | 100 | 800 | 100 | 0.2 | 400 | 1982-Apr-03 | 0.00 | 8.94 | 22-Dec-82 | 0.05 | 25-Nov-82 | 8.25 | 27-Oct-82 |
| D2 | Spring | 6 metab | Winter wheat | ditch | 1 | 100 | 800 | 2 | 0 | 400 | 1986-Apr-03 | 0.00 | 11.81 | 5-Apr-87 | 0.04 | 26-Aug-86 | 5.89 | 26-Aug-86 |
|  |  | 6 metab |  | stream | 1 | 100 | 800 | 100 | 0.2 | 400 | 1986-Apr-03 | 0.00 | 11.81 | 5-Apr-87 | 0.04 | 26-Aug-86 | 5.89 | 26-Aug-86 |
| D3 | Spring | 6 metab | Winter wheat | ditch | 1 | 100 | 800 | 2 | 0 | 400 | 1992-Apr-16 | 0.00 | 2.19 | 8-Jan-92 | 0.00 | 8-Jan-92 | 0.04 | 08-Feb-93 |
| D4 | Spring | 6 metab | Winter wheat | pond | 30 | 30 | 800 | 0 | 0 | 400 | 1985-Mar-17 | 0.00 | 5.48 | 8-Dec-85 | 0.01 | 8-Dec-85 | 0.04 | 08-Feb-93 |
|  |  | 6 metab |  | stream | 1 | 100 | 800 | 100 | 0.2 | 400 | 1985-Mar-17 | 0.00 | 5.48 | 8-Dec-85 | 0.01 | 8-Dec-85 | 1.59 | 10-Dec-85 |
| D5 | Spring | 6 metab | Winter wheat | pond | 30 | 30 | 800 | 0 | 0 | 400 | 1978-Mar-16 | 0.00 | 10.94 | 25-Jan-78 | 0.01 | 12-Feb-79 | 0.65 | 12-Feb-79 |
|  |  | 6 metab |  | stream | 1 | 100 | 800 | 100 | 0.2 | 400 | 1978-Mar-16 | 0.00 | 10.94 | 25-Jan-78 | 0.01 | 12-Feb-79 | 0.65 | 12-Feb-79 |
| D6 | Spring | 6 metab | Winter wheat | ditch | 1 | 100 | 800 | 2 | 0 | 200 | 1986-Feb-18 | 0.00 | 21.66 | 20-Jan-87 | 0.01 | 11-Feb-86 | 0.28 | 11-Feb-86 |
|  |  | 6 metab |  | pond | 30 | 30 | 800 | 0 | 0 | 200 | 1986-Feb-18 | 0.00 | 21.66 | 20-Jan-87 | 0.01 | 11-Feb-86 | 0.28 | 11-Feb-86 |
| D6 | Spring | 7 | Vines | ditch | 1 | 100 | 800 | 0 | 0 | 750 | 1986-Apr-01 | 1.544 | 22.39 | 20-Jan-87 | 0.03 | 20-Jan-87 | 2.09 | 24-Dec-86 |
|  |  | 7 | Vines | pond | 30 | 30 | 800 | 0 | 0 | 750 | 1986-Apr-01 | 0.153 | 22.39 | 20-Jan-87 | 0.03 | 20-Jan-87 | 2.09 | 24-Dec-86 |

Table G.8-2:  Summary of TOXSWA maximum and time weighted average concentrations for drainage simulations.

|  |  |  |  |  | Concentrations in water (g-L) |  |  |  |  |  |  | Concentrations in sediment (g-kg) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Scenario | Season | Compound | Crop | TOXSWA water body | Global max | Date | TWAECsw 14 day | Date | TWAECsw 21 day | Date | TWAECsw 28 day | Global max | Date |
| D3 | Spring | 1 | Potatoes | ditch | 0.144 | 2-Dec-92 | 0.144 | 16-Dec-92 | 0.144 | 23-Dec-92 | 0.144 | 0.071 | 1-Jan-93 |
| D4 | Spring | 1 | Potatoes | pond | 0.168 | 3-Aug-85 | 0.146 | 9-Aug-85 | 0.136 | 13-Aug-85 | 0.124 | 0.05 | 5-Aug-85 |
|  |  | 1 | Potatoes | stream | 0.621 | 16-Jun-85 | 0.44 | 21-Jun-85 | 0.422 | 5-Aug-85 | 0.379 | 0.155 | 18-Jun-85 |
| D6 | Spring | 1 | Potatoes | ditch | 0.294 | 29-Mar-86 | 0.082 | 24-Aug-86 | 0.069 | 31-Aug-86 | 0.061 | 0.038 | 31-Mar-86 |
|  |  | 1 | Potatoes | pond | 0.029 | 1-Apr-86 | 0.022 | 13-Apr-86 | 0.02 | 12-May-86 | 0.018 | 0.007 | 4-May-86 |
| D3 | Spring | 2 | maize | ditch | 5.27 | 22-Apr-92 | 0.43 | 6-May-92 | 0.3 | 13-May-92 | 0.24 | 1.24 | 23-Apr-92 |
| D4 | Spring | 2 | maize | pond | 1.99 | 28-Dec-85 | 1.95 | 5-Jan-86 | 1.91 | 10-Jan-86 | 1.87 | 3.84 | 17-Feb-86 |
|  |  | 2 | maize | stream | 3.92 | 27-Apr-85 | 1.54 | 18-Dec-85 | 1.46 | 25-Dec-85 | 1.31 | 2.1 | 23-Dec-85 |
| D5 | Spring | 2 | maize | pond | 1.01 | 15-Feb-79 | 0.97 | 26-Feb-79 | 0.94 | 4-Mar-79 | 0.91 | 1.78 | 1-May-78 |
|  |  | 2 | maize | stream | 4.034 | 23-Apr-78 | 0.494 | 5-Feb-78 | 0.476 | 1-Feb-78 | 0.464 | 1.044 | 23-Apr-78 |
| D6 | Spring | 2 | maize | ditch | 5.69 | 8-Apr-86 | 0.86 | 12-Apr-86 | 0.72 | 12-Apr-86 | 0.66 | 2.18 | 9-Apr-86 |
|  |  | 2 | maize | pond | 0.8 | 20-Feb-86 | 0.79 | 2-Mar-86 | 0.77 | 7-Mar-86 | 0.75 | 1.51 | 1-Apr-87 |
| D1 | Spring | 3 | Winter wheat | ditch | 52.34 | 9-Apr-82 | 33.83 | 22-Apr-82 | 27.94 | 29-Apr-82 | 35.74 | 6.24 | 13-Apr-82 |
|  |  | 3 | Winter wheat | stream | 33.1 | 9-Apr-82 | 22.15 | 22-Apr-82 | 18.61 | 29-Apr-82 | 14.12 | 3.99 | 13-Apr-82 |
| D2 | Spring | 3 | Winter wheat | ditch | 176.53 | 7-Apr-86 | 57.85 | 21-Apr-86 | 45.31 | 28-Apr-86 | 35.74 | 11.83 | 15-Apr-86 |
|  |  |  | Winter wheat | stream | 132.49 | 7-Apr-86 | 32.95 | 21-Apr-86 | 26.24 | 27-Apr-86 | 21.09 | 6.95 | 15-Apr-86 |
| D3 | Spring | 3 | Winter wheat | ditch | 6.35 | 16-Apr-92 | 0.42 | 30-Apr-92 | 0.28 | 1-May-92 | 0.21 | 0.33 | 17-Apr-92 |
| D4 | Spring | 3 | Winter wheat | pond | 0.19 | 18-Mar-85 | 0.1 | 31-Mar-85 | 0.08 | 7-Apr-85 | 0.08 | 0.02 | 22-Mar-85 |
|  |  | 3 | Winter wheat | stream | 4.04 | 17-Mar-85 | 0.31 | 13-Apr-85 | 0.24 | 19-Apr-85 | 0.19 | 0.07 | 6-Apr-85 |
| D5 | Spring | 3 | Winter wheat | pond | 0.18 | 17-Mar-78 | 0.08 | 30-Mar-78 | 0.05 | 6-Apr-78 | 0.04 | 0.02 | 20-Mar-78 |
|  |  | 3 | Winter wheat | stream | 4.453 | 16-Mar-78 | 0.018 | 30-Mar-78 | 0.012 | 6-Apr-78 | 0.009 | 0.068 | 16-Mar-78 |
| D6 | Spring | 3 | Winter wheat | ditch | 6.19 | 18-Feb-86 | 0.73 | 4-Mar-86 | 0.56 | 11-Mar-86 | 0.45 | 0.29 | 25-Feb-86 |
|  |  | 3 |  | pond | 0.28 | 26-Feb-86 | 0.18 | 4-Mar-86 | 0.14 | 10-Mar-86 | 0.11 | 0.04 | 28-Feb-86 |
| D3 | Spring | 4 | apples | ditch | 0.324 | 3-May-92 | 0.018 | 3-May-92 | 0.022 | 10-May-92 | 0.017 | 4.186 | 23-May-92 |
| D4 | Spring | 4 | apples | pond | 0.018 | 18-May-85 | 0.003 | 29-Apr-85 | 0.003 | 06-May-85 | 0.003 | 0.605 | 20-May-85 |
|  |  | 4 | apples | stream | 0.328 | 18-May-85 | 0.003 | 01-Jun-85 | 0.003 | 25-May-85 | 0.002 | 0.939 | 18-May-85 |
| D5 | Spring | 4 | apples | pond | 0.018 | 26-May-78 | 0.002 | 05-May-78 | 0.002 | 12-May-78 | 0.002 | 0.481 | 28-May-78 |
|  |  | 4 | apples | stream | 0.369 | 26-May-78 | 0.008 | 09-Jun-78 | 0.008 | 31-May-78 | 0.006 | 2.263 | 26-May-78 |
| D6 | Spring | 5 | Vines | ditch | 0.76 | 9-Feb-86 | 0.27 | 4-Jan-87 | 0.21 | 28-Feb-86 | 0.2 | 0.87 | 14-May-86 |
|  |  | 5 | Vines | pond | 0.32 | 14-Feb-86 | 0.29 | 24-Feb-86 | 0.25 | 3-Mar-86 | 0.22 | 0.74 | 28-Jan-87 |
| D1 | Spring | 6 | Winter wheat | ditch | 21.09 | 11-Apr-82 | 17.88 | 23-Apr-82 | 16.45 | 30-Apr-82 | 15.02 | 13.53 | 11-May-82 |
|  |  | 6 |  | stream | 13.28 | 10-Apr-82 | 10.92 | 23-Apr-82 | 9.45 | 29-Apr-82 | 7.29 | 7.57 | 24-Apr-82 |
| D2 | Spring | 6 | Winter wheat | ditch | 43.47 | 19-Apr-86 | 18.41 | 28-Apr-86 | 15.53 | 5-May-86 | 13.5 | 13.24 | 19-May-86 |
|  |  | 6 | Winter wheat | stream | 27.65 | 19-Apr-86 | 10.65 | 28-Apr-86 | 8.86 | 5-May-86 | 7.67 | 7.45 | 19-May-86 |
| D3 | Spring | 6 | Winter wheat | ditch | 2.56 | 16-Apr-92 | 0.21 | 30-Apr-92 | 0.15 | 7-May-92 | 0.12 | 0.38 | 17-Apr-92 |
| D4 | Spring | 6 | Winter wheat | pond | 0.09 | 17-Mar-85 | 0.08 | 31-Mar-85 | 0.07 | 7-Apr-85 | 0.07 | 0.06 | 27-Apr-85 |
|  |  | 6 | Winter wheat | stream | 1.7 | 17-Mar-85 | 0.28 | 23-Dec-85 | 0.26 | 28-Dec-85 | 0.24 | 0.22 | 28-Jan-86 |
| D5 | Spring | 6 | Winter wheat | pond | 0.09 | 16-Mar-78 | 0.08 | 30-Mar-78 | 0.07 | 6-Apr-78 | 0.07 | 0.06 | 17-Apr-78 |
|  |  | 6 | Winter wheat | stream | 1.84 | 16-Mar-78 | 0.08 | 15-Jan-78 | 0.08 | 22-Jan-78 | 0.08 | 0.13 | 16-Mar-78 |
| D6 | Spring | 6 | Winter wheat | ditch | 1.31 | 18-Feb-86 | 0.26 | 9-Mar-86 | 0.22 | 11-Mar-86 | 0.19 | 0.27 | 26-Feb-86 |
|  |  | 6 | Winter wheat | pond | 0.04 | 18-Feb-86 | 0.03 | 4-Mar-86 | 0.03 | 11-Mar-86 | 0.03 | 0.02 | 12-Mar-86 |
| D1 | Spring | 6 metab | Winter wheat | ditch | 2.65 | 24-Nov-82 | 2.4 | 4-Dec-82 | 2.3 | 10-Dec-82 | 2.2 | 7.12 | 20-Apr-82 |
|  |  | 6 metab | Winter wheat | stream | 1.61 | 22-Nov-82 | 1.51 | 3-Dec-82 | 1.45 | 10-Dec-82 | 1.39 | 4.27 | 18-Apr-82 |
| D2 | Spring | 6 metab | Winter wheat | ditch | 2.07 | 25-Aug-86 | 1.15 | 23-Nov-86 | 1.1 | 30-Nov-86 | 1.09 | 4.21 | 22-Nov-86 |
|  |  | 6 metab | Winter wheat | stream | 1.32 | 25-Aug-86 | 0.69 | 27-Nov-86 | 0.66 | 30-Nov-86 | 0.63 | 2.34 | 15-Dec-86 |
| D3 | Spring | 6 metab | Winter wheat | ditch | 0.01 | 6-Feb-93 | 0.01 | 13-Feb-93 | 0.01 | 15-Feb-93 | 0.01 | 0.07 | 30-Mar-93 |
| D4 | Spring | 6 metab | Winter wheat | pond | 0.00 | 1-Jan-85 | 0.00 | 15-Jan-85 | 0.00 | 22-Jan-85 | 0.00 | 0.00 | 1-Jan-85 |
|  |  | 6 metab | Winter wheat | stream | 0.4 | 9-Dec-85 | 0.25 | 20-Dec-85 | 0.24 | 27-Dec-85 | 0.22 | 0.53 | 27-Dec-85 |
| D5 | Spring | 6 metab | Winter wheat | pond | 0.00 | 1-Jan-78 | 0.00 | 15-Jan-78 | 0.00 | 22-Jan-78 | 0.00 | 0.00 | 1-Jan-78 |
|  |  | 6 metab | Winter wheat | stream | 0.17 | 11-Feb-79 | 0.09 | 18-Feb-79 | 0.08 | 25-Feb-79 | 0.07 | 0.25 | 11-Apr-78 |
| D6 | Spring | 6 metab | Winter wheat | ditch | 0.12 | 9-Feb-86 | 0.06 | 31-Dec-86 | 0.05 | 7-Jan-87 | 0.04 | 0.12 | 26-Dec-86 |
|  |  | 6 metab | Winter wheat | pond | 0.00 | 1-Jan-86 | 0.00 | 15-Jan-86 | 0.00 | 22-Jan-86 | 0.00 | 0.00 | 1-Jan-86 |
| D6 | Spring | 7 | Vines | ditch | 3.85 | 22-Apr-86 | 1.04 | 6-May-86 | 0.7 | 13-May-86 | 0.66 | 2.28 | 26-Apr-86 |
|  |  | 7 | Vines | pond | 0.28 | 26-Dec-86 | 0.19 | 6-Jan-87 | 0.15 | 13-Jan-87 | 0.13 | 0.32 | 1-Jan-87 |