Study Report

Predicted Environmental Concentrations in Surface Water 
of Urea based on FOCUS STEP3

Simulations potatoes, oil seed rape, and vegetables (granular application)

Sponsor

AlzChem Trostberg GmbH

Dr.-Albert-Frank-Str. 32

83308 Trostberg

Germany

Institute

Fraunhofer-Institute for Molecular
	Biology and Applied Ecology (IME)

Auf dem Aberg 1

57392 Schmallenberg
	Germany

Director Applied Ecology
	Prof. Dr. Christoph Schäfers

Author
Dr. Michael Klein

Dr. Judith Klein

October 28, 2019

This page was intentionally left blank for statements of the sponsor or submitter.

Statement of compliance

This study “Predicted Environmental Concentrations in Surface Water of Urea based on FOCUS STEP3; Simulations potatoes, oil seed rape, and vegetables (granular application)” was conducted according to the procedures described herein. This report is a true and accurate record of the results obtained. There were no circumstances that may have adversely impacted the quality or integrity of the study.

The GLP-regulation is not applicable. However, the study was performed in accordance to the “Codex of Good Modelling Practices” (Görlitz 1993 und Travis 1995).

______________________________________	__28 October 2019______

Dr. Judith Klein		Date

Statistics and Modelling

Fraunhofer Institute

Auf dem Aberg 1

57392 Schmallenberg

Germany

Tel +49 2972 302 256

Fax +49 2972 302 319

judith.klein@ime.fraunhofer.de

Contents	page

Statement of compliance	3

1.	Simulation models	5

2.	Input parameters	5

2.1	Agricultural use pattern	5

2.2	Substance properties of urea	8

3.	Results	10

4.	Conclusions	13

5.	References	14

6.	Appendix: SWASH Report Files	15

6.1	Oilseed rape (winter), end of February/ begin of April, 260kg, begin/mid of April, 130kg	15

6.2	Potatoes, begin of April, 260kg, mid of May, 90kg	17

6.3	Vegetables (leafy), default, 430kg,	19

# Simulation models

For the STEP3 calculations the computer tool SWASH was used which mainly creates the necessary input data for MACRO 5.5.4, PRZM 4.3.1 and TOXSWA 4.4 which were used for the simulations. All models are described in FOCUS (2001). The standard buffer zone was 1 m.

They represent start-of-the-art PEC-calculations for all type active compounds (pesticides, biocides and veterinary compounds). They are also the most recent versions. The history of versions is summarised at the FOCUS homepage (https://esdac.jrc.ec.europa.eu/projects/focus-dg-sante).

# Input parameters

## Agricultural use pattern

Urea is applied in various crops with different application patterns as summarised in the following table.

The term “Granular linear incoperated at 4 cm” means a linear incorporation into the upper 4 cm of the soil.

Table 1: Application pattern of urea in various crops considered for the simulations

| Scen. id | Crop | Nr. of app. | App. method | Incorp. Depth | App. Rate Urea | Start of the application window |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | (cm) | (kg/ha) |  |
| 1 | Oil seed rape | 1st | Granular, linear | 4 (default) | 260 | end of February/ beginning of April |
|  |  | 2nd | Granular, linear | 4 (default) | 130 | mid of May |
| 2 | Potatoes | 1st | Granular, linear | 4 (default) | 260 | beginning of April |
|  |  | 2nd | Granular, linear | 4 (default) | 90 | mid of May |
| 3 | Vegetables (leafy) | 1st | Granular, linear | 4 (default) | 430 | default |

Based on the information given in Table 1 the following application dates were automatically calculated for the different crops at the different scenarios.

Table 2: Application date calculated by FOCUS PAT considered for the simulations

| Scen. ID | Crop | Scenario | application date calculated by FOCUS PAT (1st) | application date calculated by FOCUS PAT (2nd) |
| --- | --- | --- | --- | --- |
| 1 | Oil seed rape | D2_Ditch | 22. Feb 86 | 09. Apr 86 |
|  |  | D2_Stream | 22. Feb 86 | 09. Apr 86 |
|  |  | D3_Ditch | 29. Feb 92 | 20. Apr 92 |
|  |  | D4_Pond | 24. Feb 85 | 18. Apr 85 |
|  |  | D4_Stream | 24. Feb 85 | 18. Apr 85 |
|  |  | D5_Pond | 21. Feb 78 | 08. Apr 78 |
|  |  | D5_Stream | 21. Feb 78 | 08. Apr 78 |
|  |  | R1_Pond | 24. Feb 79 | 12. Apr 79 |
|  |  | R1_Stream | 24. Feb 79 | 12. Apr 79 |
|  |  | R3_Stream | 20. Feb 81 | 13. Apr 81 |
| 2 | Potatoes | D3_Ditch | 04. Apr 92 | 24. May 92 |
|  |  | D4_Pond | 18. Apr 85 | 02. Jun 85 |
|  |  | D4_Stream | 18. Apr 85 | 02. Jun 85 |
|  |  | D6_Ditch | 02. Apr 86 | 17. May 86 |
|  |  | D6_Ditch2 | 02. Apr 86 | 17. May 86 |
|  |  | R1_Pond | 26. Apr 84 | 13. Jun 84 |
|  |  | R1_Stream | 26. Apr 84 | 13. Jun 84 |
|  |  | R2_Stream | 22. Apr 77 | 14. Jun 77 |
|  |  | R3_Stream | 04. Apr 80 | 01. Jun 80 |
| 3 | Vegetables (leafy) | D3_Ditch | 10. Apr 92 |  |
|  |  | D3_Ditch2 | 25. Jul 92 |  |
|  |  | D4_Pond | 16. May 85 |  |
|  |  | D4_Stream | 16. May 85 |  |
|  |  | D6_Ditch | 04. Aug 86 |  |
|  |  | R1_Pond | 26. Apr 84 |  |
|  |  | R1_Pond2 | 28. Jul 78 |  |
|  |  | R1_Stream | 26. Apr 84 |  |
|  |  | R1_Stream2 | 28. Jul 78 |  |
|  |  | R2_Stream | 06. Mar 78 |  |
|  |  | R2_Stream2 | 05. Aug 89 |  |
|  |  | R3_Stream | 19. Feb 81 |  |
|  |  | R3_Stream2 | 02. Jun 75 |  |
|  |  | R4_Stream | 01. Mar 80 |  |
|  |  | R4_Stream2 | 01. Jun 85 |  |

## Substance properties of urea

For the half-life of urea in surface water 4.8 days at 20°C was taken. This is the geometric mean given in EFSA (2010). A default value of 1000 days was considered for the sediment phase. For soil, the default for a readily biodegradable substance is used, namely 30 days at 12°C. The computer automatically transfers the half lives at standard temperatures into the actual conditions of the scenarios.

All DegT50 values in water, sediment-system and in soil considered in the simulations are presented in Table 3.

Table 3: DegT50-values (d) of urea

| Parameter | Urea |
| --- | --- |
| Water | 4.8 (at 20 °C) |
| Sediment | 1000 (at 20 °C) |
| Soil | 30 (at 12 °C) |

The sorption constant in soil KOC was set to 7.2 L/kg which was calculated from Hongprayoon (1991). The taken KOC value for urea corresponds to the mean of KOC values ranging from 5.3 to 9.1.

Plant uptake via roots was not considered since urea is usually applied before emergence of the crop.

All other input parameters used for the simulations are summarised in Table 4.

Table 4: Other input parameters used for the simulations of urea

| Parameter | Urea | Remark |
| --- | --- | --- |
| Sorption constant KOC in soil (L/kg) | 7.2 | EFSA (2010) |
| Sorption constant KOC in water body (L/kg) | 7.2 | EFSA (2010) |
| Freundlich exponent (-) | 1 | EFSA (2010) |
| Vapour pressure (25°C, Pa) | 0.0016 |  |
| Molar mass (g/mol) | 60.06 |  |
| Water solubility (20°C, mg/L) | 624000 | EFSA (2010) |
| Molar enthalpy of vaporisation | 95000 | default |
| Molar enthalpy of dissolution | 27000 | default |
| Diffusion coefficient in water | 4.3 10-5 | default |
| Diffusion coefficient in air | 0.43 | default |
| Plant uptake factor | 0 | default |

# Results

The maximum concentrations for all scenarios and crops are summarised in the following Table 6.

Table 5: Maximum concentrations in water (PECmax) of urea at FOCUS Step 3

| Scen. ID | Scenaro/Crop | PECmax (µg/L) |
| --- | --- | --- |
| 1 | Oilseed rape (winter), end of February/ beginning of April, 260kg, beginning/mid of April, 130kg | 36220.4 |
| 2 | Potatoes, beginning of April, 260kg, mid of May, 90kg | 15900.8 |
| 3 | (Leafy) vegetables, default, 430kg, | 16882 |

Table 6: Global maximum concentrations of urea at FOCUS Step 3

| Scen. ID | Crop/Scenario | Scenario | PECsw (µg/L) | PECsed (µg/kg) |
| --- | --- | --- | --- | --- |
| 1 | Oilseed rape (winter), end of February/ begin of April, 260kg, begin/mid of April, 130kg | D2_Ditch | 36220.4 | 8016.9 |
|  |  | D2_Stream | 24354.3 | 4649.7 |
|  |  | D3_Ditch | 413 | 378.6 |
|  |  | D4_Pond | 323.6 | 187.5 |
|  |  | D4_Stream | 432.7 | 204.2 |
|  |  | D5_Pond | 42.98 | 22.18 |
|  |  | D5_Stream | 56.57 | 24.19 |
|  |  | R1_Pond | 64.73 | 17.24 |
|  |  | R1_Stream | 2708.9 | 142.5 |
|  |  | R3_Stream | 4478.8 | 262.6 |
| 2 | Potatoes, begin of April, 260kg, mid of May, 90kg | D3_Ditch | 510.3 | 459.2 |
|  |  | D4_Pond | 302 | 201.8 |
|  |  | D4_Stream | 1061.6 | 470.3 |
|  |  | D6_Ditch | 1752.8 | 179.4 |
|  |  | D6_Ditch2 | 1752.8 | 179.4 |
|  |  | R1_Pond | 233.5 | 44.06 |
|  |  | R1_Stream | 3665.9 | 323 |
|  |  | R2_Stream | 2850.4 | 190.5 |
|  |  | R3_Stream | 15900.8 | 766.7 |
| 3 | (Leafy) vegetables, default, 430kg, | D3_Ditch | 498.2 | 459.6 |
|  |  | D3_Ditch2 | 3292.2 | 2872.8 |
|  |  | D4_Pond | 403.8 | 263.5 |
|  |  | D4_Stream | 1318.6 | 542.3 |
|  |  | D6_Ditch | 5614.7 | 602 |
|  |  | R1_Pond | 4.263 | 1.182 |
|  |  | R1_Pond2 | 0.001203 | 0.000233 |
|  |  | R1_Stream | 1086.5 | 62.8 |
|  |  | R1_Stream2 | 0.01917 | 0.001594 |
|  |  | R2_Stream | 4124.1 | 262 |
|  |  | R2_Stream2 | 113.6 | 8.354 |
|  |  | R3_Stream | 6961.7 | 419.8 |
|  |  | R3_Stream2 | 7104.6 | 666 |
|  |  | R4_Stream | 14276.7 | 1187.8 |
|  |  | R4_Stream2 | 16882 | 1415.4 |

# Conclusions

The following maximum concentrations were calculated for urea at step 3 simulations (no additional buffer strip to the surface water body):

Table 7: Maximum concentrations in water (PECmax) of urea at FOCUS Step 3

| Scen. ID | Scenaro/Crop | PECmax (µg/L) |
| --- | --- | --- |
| 1 | Oilseed rape (winter), end of February/ begin of April, 260kg, begin/mid of April, 130kg | 36220.4 |
| 2 | Potatoes, begin of April, 260kg, mid of May, 90kg | 15900.8 |
| 3 | (Leafy) vegetables, default, 430kg, | 16882 |

# References

- EFSA (2010): “Conclusion on the peer review of the pesticide risk assessment of the active substance cyanamide”. EFSA Journal 2010;8(11):1873.

- FOCUS (2001). “FOCUS Surface Water Scenarios in the EU Evaluation Process under 91/414/EEC”. Report of the FOCUS Working Group on Surface Water Scenarios, EC

- Görlitz. G. (1993): „Verfahrensregeln zur korrekten Durchführung und Auswertung von Modellrechnungen zur Simulation des Umweltverhaltens von Pflanzenschutzmitteln.“

- Travis. K.Z. (1995): “Recommendations for the correct use of models and reporting of modelling results.”: ‘Leaching Models and EU registration’. Final report of the FOCUS Group. Doc. 4952/VI/95.

# Appendix: SWASH Report Files

## Oilseed rape (winter), end of February/ begin of April, 260kg, begin/mid of April, 130kg

* SWASH report file

* made by FOCUS-SWASH UI v. 5 (internal version 5.1.0, 02 April 2015)

*

* File Name   : E:\SwashProjects\HS_20190925\HS_OSR\HS_OSR_report.txt

* Description : Oilseed rape, end of February/ begin of April, 260kg, begin/mid of April, 130kg

* Substance   : HS

*

* Creation    : 02-Oct-2019, 12:50

*

* Remarks : SWASH report helps you to set up the needed runs to calculate the PECsw and PECsed, occuring in the EU

*           for the selected substance, used on the selected crop. The scenario code informs you which models you need to

*           run for this scenario.

*           D1-D6: drainage entries calculated by the MACRO model, fate in surface water calculated by the TOXSWA model

*

*           R1-R4: runoff and erosion entries calculated by the PRZM model, fate in surface water calculated by the TOXSWA model

*

*

*           For STREAMS the Mean Deposition and Mass Loading, as calculated by the FOCUS Drift Calculator, have been multiplied by a

*           factor 1.2 to account for pesticide mass incoming from the upstream catchment as decided by the FOCUS Surface Water

*           Scenarios Working Group.

*

*

**********************************************************************************************************************************

*  CREATED RUNS

**********************************************************************************************************************************

*

*                                                     |--------------- APPLICATION -----------------|----- on Water Surface -----|

* -ID------Crop(1st/2nd)-------Scenario-WaterbodyType-|-Method--------First/Last/Interval--#---Rate-|-Mean Deposition-Mass Loading

(d)          (kg/ha) (% of Appl. Rate)    (mg/m2)

*

* 41    Oil seed rape, winte(1st) D2_Ditch              granular appl. 20-Feb/6-May /45     1  260.0000     0.000           0.000

*                                                                                          2  130.0000     0.000           0.000

*

* 42    Oil seed rape, winte(1st) D2_Stream             granular appl. 20-Feb/6-May /45     1  260.0000     0.000           0.000

*                                                                                          2  130.0000     0.000           0.000

*

* 43    Oil seed rape, winte(1st) D3_Ditch              granular appl. 20-Feb/6-May /45     1  260.0000     0.000           0.000

*                                                                                          2  130.0000     0.000           0.000

*

* 44    Oil seed rape, winte(1st) D4_Pond               granular appl. 20-Feb/6-May /45     1  260.0000     0.000           0.000

*                                                                                          2  130.0000     0.000           0.000

*

* 45    Oil seed rape, winte(1st) D4_Stream             granular appl. 20-Feb/6-May /45     1  260.0000     0.000           0.000

*                                                                                          2  130.0000     0.000           0.000

*

* 46    Oil seed rape, winte(1st) D5_Pond               granular appl. 20-Feb/6-May /45     1  260.0000     0.000           0.000

*                                                                                          2  130.0000     0.000           0.000

*

* 47    Oil seed rape, winte(1st) D5_Stream             granular appl. 20-Feb/6-May /45     1  260.0000     0.000           0.000

*                                                                                          2  130.0000     0.000           0.000

*

* 48    Oil seed rape, winte(1st) R1_Pond               granular appl. 20-Feb/6-May /45     1  260.0000     0.000           0.000

*                                                                                          2  130.0000     0.000           0.000

*

* 49    Oil seed rape, winte(1st) R1_Stream             granular appl. 20-Feb/6-May /45     1  260.0000     0.000           0.000

*                                                                                          2  130.0000     0.000           0.000

*

* 50    Oil seed rape, winte(1st) R3_Stream             granular appl. 20-Feb/6-May /45     1  260.0000     0.000           0.000

*                                                                                          2  130.0000     0.000           0.000

*

************************************************** Surface WAter Scenarios Help **************************************************

## Potatoes, begin of April, 260kg, mid of May, 90kg

* SWASH report file

* made by FOCUS-SWASH UI v. 5 (internal version 5.1.0, 02 April 2015)

*

* File Name   : E:\SwashProjects\HS_20190925\HS_PO\HS_PO_report.txt

* Description : Potatoes, begin of April, 260kg, mid of May, 90kg

* Substance   : HS

*

* Creation    : 02-Oct-2019, 12:44

*

* Remarks : SWASH report helps you to set up the needed runs to calculate the PECsw and PECsed, occuring in the EU

*           for the selected substance, used on the selected crop. The scenario code informs you which models you need to

*           run for this scenario.

*           D1-D6: drainage entries calculated by the MACRO model, fate in surface water calculated by the TOXSWA model

*

*           R1-R4: runoff and erosion entries calculated by the PRZM model, fate in surface water calculated by the TOXSWA model

*

*

*           For STREAMS the Mean Deposition and Mass Loading, as calculated by the FOCUS Drift Calculator, have been multiplied by a

*           factor 1.2 to account for pesticide mass incoming from the upstream catchment as decided by the FOCUS Surface Water

*           Scenarios Working Group.

*

*

**********************************************************************************************************************************

*  CREATED RUNS

**********************************************************************************************************************************

*

*                                                     |--------------- APPLICATION -----------------|----- on Water Surface -----|

* -ID------Crop(1st/2nd)-------Scenario-WaterbodyType-|-Method--------First/Last/Interval--#---Rate-|-Mean Deposition-Mass Loading

(d)          (kg/ha) (% of Appl. Rate)    (mg/m2)

*

* 9     Potatoes(1st)            D3_Ditch              granular appl. 1-Apr /15-Jun/45     1  260.0000     0.000           0.000

*                                                                                          2  90.0000      0.000           0.000

*

* 10    Potatoes(1st)            D4_Pond               granular appl. 1-Apr /15-Jun/45     1  260.0000     0.000           0.000

*                                                                                          2  90.0000      0.000           0.000

*

* 11    Potatoes(1st)            D4_Stream             granular appl. 1-Apr /15-Jun/45     1  260.0000     0.000           0.000

*                                                                                          2  90.0000      0.000           0.000

*

* 12    Potatoes(1st)            D6_Ditch              granular appl. 1-Apr /15-Jun/45     1  260.0000     0.000           0.000

*                                                                                          2  90.0000      0.000           0.000

*

* 13    Potatoes(2nd)            D6_Ditch              granular appl. 1-Apr /15-Jun/45     1  260.0000     0.000           0.000

*                                                                                          2  90.0000      0.000           0.000

*

* 14    Potatoes(1st)            R1_Pond               granular appl. 1-Apr /15-Jun/45     1  260.0000     0.000           0.000

*                                                                                          2  90.0000      0.000           0.000

*

* 15    Potatoes(1st)            R1_Stream             granular appl. 1-Apr /15-Jun/45     1  260.0000     0.000           0.000

*                                                                                          2  90.0000      0.000           0.000

*

* 16    Potatoes(1st)            R2_Stream             granular appl. 1-Apr /15-Jun/45     1  260.0000     0.000           0.000

*                                                                                          2  90.0000      0.000           0.000

*

* 17    Potatoes(1st)            R3_Stream             granular appl. 1-Apr /15-Jun/45     1  260.0000     0.000           0.000

*                                                                                          2  90.0000      0.000           0.000

*

************************************************** Surface WAter Scenarios Help **************************************************

## Vegetables (leafy), default, 430kg,

* SWASH report file

* made by FOCUS-SWASH UI v. 5 (internal version 5.1.0, 02 April 2015)

*

* File Name   : E:\SwashProjects\HS_20190925\HS_VEG\HS_VEG_report.txt

* Description : (Leafy) vegetables, default, 430kg

* Substance   : HS

*

* Creation    : 02-Oct-2019, 12:50

*

* Remarks : SWASH report helps you to set up the needed runs to calculate the PECsw and PECsed, occuring in the EU

*           for the selected substance, used on the selected crop. The scenario code informs you which models you need to

*           run for this scenario.

*           D1-D6: drainage entries calculated by the MACRO model, fate in surface water calculated by the TOXSWA model

*

*           R1-R4: runoff and erosion entries calculated by the PRZM model, fate in surface water calculated by the TOXSWA model

*

*

*           For STREAMS the Mean Deposition and Mass Loading, as calculated by the FOCUS Drift Calculator, have been multiplied by a

*           factor 1.2 to account for pesticide mass incoming from the upstream catchment as decided by the FOCUS Surface Water

*           Scenarios Working Group.

*

*

**********************************************************************************************************************************

*  CREATED RUNS

**********************************************************************************************************************************

*

*                                                     |--------------- APPLICATION -----------------|----- on Water Surface -----|

* -ID------Crop(1st/2nd)-------Scenario-WaterbodyType-|-Method--------First/Last/Interval--#---Rate-|-Mean Deposition-Mass Loading

(d)          (kg/ha) (% of Appl. Rate)    (mg/m2)

*

* 26    Vegetables, leafy(1st)   D3_Ditch              granular appl. 11-Apr/11-May/1      1  430.0000     0.000           0.000

*

* 27    Vegetables, leafy(2nd)   D3_Ditch              granular appl. 22-Jul/21-Aug/1      1  430.0000     0.000           0.000

*

* 28    Vegetables, leafy(1st)   D4_Pond               granular appl. 26-Apr/26-May/1      1  430.0000     0.000           0.000

*

* 29    Vegetables, leafy(1st)   D4_Stream             granular appl. 26-Apr/26-May/1      1  430.0000     0.000           0.000

*

* 30    Vegetables, leafy(1st)   D6_Ditch              granular appl. 1-Aug /31-Aug/1      1  430.0000     0.000           0.000

*

* 31    Vegetables, leafy(1st)   R1_Pond               granular appl. 6-Apr /6-May /1      1  430.0000     0.000           0.000

*

* 32    Vegetables, leafy(2nd)   R1_Pond               granular appl. 17-Jul/16-Aug/1      1  430.0000     0.000           0.000

*

* 33    Vegetables, leafy(1st)   R1_Stream             granular appl. 6-Apr /6-May /1      1  430.0000     0.000           0.000

*

* 34    Vegetables, leafy(2nd)   R1_Stream             granular appl. 17-Jul/16-Aug/1      1  430.0000     0.000           0.000

*

* 35    Vegetables, leafy(1st)   R2_Stream             granular appl. 14-Feb/16-Mar/1      1  430.0000     0.000           0.000

*

* 36    Vegetables, leafy(2nd)   R2_Stream             granular appl. 17-Jul/16-Aug/1      1  430.0000     0.000           0.000

*

* 37    Vegetables, leafy(1st)   R3_Stream             granular appl. 15-Feb/17-Mar/1      1  430.0000     0.000           0.000

*

* 38    Vegetables, leafy(2nd)   R3_Stream             granular appl. 1-Jun /1-Jul /1      1  430.0000     0.000           0.000

*

* 39    Vegetables, leafy(1st)   R4_Stream             granular appl. 15-Feb/17-Mar/1      1  430.0000     0.000           0.000

*

* 40    Vegetables, leafy(2nd)   R4_Stream             granular appl. 1-Jun /1-Jul /1      1  430.0000     0.000           0.000

*

************************************************** Surface WAter Scenarios Help **************************************************