- Appendix 1

# Information on moisture correction

Laboratory degradation are undertaken at various moisture contents often between 40-50% MWHC. Additional data provided in study reports may include the actual moisture content of the soil during the study as volumetric (% volume/volume), or as gravimetric (% mass/mass). Other studies may define the reference soil moisture in terms of; % field capacity (FC), or as matric potential values such as pF, kPa or Bar. According to FOCUS (2000) a special procedure called “normalisation” has to be performed before an average value can be calculated.

For the normalisation following equation is used:

- DT50pf2:	DT50 value at moisture content pF2 (normalised condition)

- DT50exp:	DT50 value at experimental conditions

- exp:		experimental soil moisture

- pF2:		normalised soil moisture (pF 2)

# Information on temperature correction

Laboratory degradation are undertaken at various temperatures 10 °C and 25 °. However, the FOCUS models require the degradation rates (or half lives) at 20 °C (normalised conditions). Therefore, according to FOCUS (2000) a special procedure called “normalisation” has to be performed before an average value can be calculated.

For the normalisation following equation is used:

- DT5020°C:		DT50 value at 20 °C (normalised condition)

- DT50exp:		DT50 value at experimental conditions

- Texp:		Temperature during study (in °C)

In the following table the resulting normalised values are presented for the transformation of Ca CN2 to cyanamide before and after normalising the studies to a temperature of 20°C:

- Table 1: Temperature normalisation of DT50 values of Ca CN2 to reference conditions (20 °C)

| Soil | DT50 at study conditions (days) | Temperature (°C) | Normalsation factor | DT50 after normalisation to 20 °C (days) |
| --- | --- | --- | --- | --- |
| Loamy sand | 1.1 | 12 | 0.532 | 0.585 |
| Loamy sand | 1.8 | 12 | 0.532 | 0.958 |
| Loamy sand | 0.6 | 20 | 1.000 | 0.600 |
| Loamy sand | 1.21 | 20 | 1.000 | 1.210 |
| Silt loam | 0.87 | 12 | 0.532 | 0.463 |
| Silt loam | 1.63 | 12 | 0.532 | 0.867 |
| silty clay | 2.51 | 20 | 1.000 | 2.510 |
| silty clay | 2.47 | 20 | 1.000 | 2.470 |
| Silt loam | 1.61 | 20 | 1.000 | 1.610 |
| Silt loam | 1.63 | 20 | 1.000 | 1.630 |
| Geometric mean | 1.42 |  |  | 1.10 |

After normalisation to 20 °C the geometric mean of all studies changed from 1.42 d to 1.10 d.

In the following table the DT50 values at 20 °C from the table above were used to do the additional soil moisture normalisation.

- Table 2: Soil moisture normalisation of DT50 values (20 °C) of Ca CN2 to reference conditions (pF 2)

| Soil | DT50 at 20 °C (days) | exp. Soil moistur (%) | reference soil moisture (%) | normalisation factor | DT50 after normalisation to 20 °C and pF 2(days)^ |
| --- | --- | --- | --- | --- | --- |
| Loamy sand | 0.585 | 10 | 12 | 0.880 | 0.515 |
| Loamy sand | 0.958 | 5 | 12 | 0.542 | 0.519 |
| Loamy sand | 0.600 | 10 | 12 | 0.880 | 0.528 |
| Loamy sand | 1.210 | 5 | 12 | 0.542 | 0.656 |
| Silt loam | 0.463 | 21 | 26 | 0.861 | 0.399 |
| Silt loam | 0.867 | 10.4 | 26 | 0.527 | 0.457 |
| silty clay | 2.510 | 16 | 46 | 0.477 | 1.198 |
| silty clay | 2.470 | 32 | 46 | 0.776 | 1.916 |
| Silt loam | 1.610 | 9.1 | 26 | 0.480 | 0.772 |
| Silt loam | 1.630 | 18.2 | 26 | 0.779 | 1.270 |
| Geometric mean | 1.10 |  |  |  | 0.72 |

- * These are default values taken from FOCUS (2000)	
^ The optimised soil moisture is pF 2 (Field capacity) according to FOCUS (2000)

For the modelling a half-life of 0.72 days should be used. The value represents the geometric mean of all experimental results after normalisation to 20 °C including soil moisture normalisation.

For cyanamide no soil moisture normalisation was done since according to the experimental results the degradation of cyanamide does not always increase with soil moisture (see the following table). Consequently, the soil moisture correction in the models MACRO and PRZM are not suitable and the moisture correction was switched off in the simulation. The geometric mean of all studies without considering temperature normalisation was found to be 0.82 d. However, for the modelling a half-life of 0.78 days should be used. The value represents the geometric mean of all experimental data after normalisation to 20 °C but without soil moisture normalisation. The details can be found in the following table

| Soil | exp. Temperature (°C) | exp. soil moisture* | DT50 at study conditions (days) | Normalisation factor | DT50 after normalisation to 20 °C (days) |
| --- | --- | --- | --- | --- | --- |
| Refesol 01-A | 12 | 10% | 2.2 | 0.532 | 1.171 |
| Refesol 01-A | 12 | 5% | 1.3 | 0.532 | 0.692 |
| Refesol 01-A | 20 | 10% | 0.95 | 1.000 | 0.820 |
| Refesol 01-A | 20 | 5% | 0.82 | 1.000 | 0.770 |
| Refesol 02-A | 12 | 21% | 1.06 | 0.532 | 0.506 |
| Refesol 02-A, | 12 | 10.4% | 1.15 | 0.532 | 0.420 |
| Refesol 06-A | 20 | 10% | 0.42 | 1.000 | 0.820 |
| Refesol 06-A | 20 | 5% | 0.55 | 1.000 | 0.770 |
| Dugliolo | 20 | 9.1% | 1.21 | 1.000 | 0.820 |
| Dugliolo | 20 | 18% | 0.79 | 1.000 | 0.770 |
| Sandy Loam (Ashland, USA) | 20 | - | 0.7 | 1.000 | 0.700 |
| Loamy sand (SP 257) | 20 | - | 0.96 | 1.000 | 0.960 |
| Loamy sand (SP 357) | 20 | - | 1.24 | 1.000 | 1.240 |
| Geometric mean |  |  | 0.82 |  | 0.78 |

- * not considered for the normalisation